3GPP TSG-RAN WG4 Meeting # 114	R4-2500809

# Athens, Greece, 17th - 21th, Feb 2025

Source: 	Samsung

Title: 	Link budget calculation for DL performance with shared Rx RF chain

Agenda Item:	7.6.3

Document for:	Discussion

- Introduction

During the RAN4#113 meeting, the impact of shared Rx RF chain on DL performance was discussed, and some major issues were identified in WF [1]. In this contribution, we will focus on discussing issue 3-3, which pertains to the impact on DL performance for UE enabling fragmented carrier mode, particularly in relation to reference sensitivity requirement.

- 2. Discussion

As an example for link budget calculation of DL performance with shared Rx RF chain, the following NR CA configuration has been chosen. This configuration represents a wide Wgap between the PCC and SCC and also has a short duplex distance between the uplink and downlink.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 10 | 5 | FDD |

Table 1 The CA configuration of n25(2A)

For the link budget calculation, the table format has been modified for simplification.

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor |
| --- | --- | --- | --- | --- |
| PA out [dBm] | 27 |  |  |  |  |
| Duplexer isolation [dB] | 50 |  |  |  |  |
| Rx Antenna input [dBm] | - | -96.5 | -91.5 | -95.9 |  |
| FE Loss [dB] | - | 4 | 4 | 4 |  |
| Duplexer output [dBm] | -23 | -100.5 | -95.5 | -99.9 |  |
| eLNA output [dBm] | -17 | -85.5 | -80.5 | -84.9 |  |
| Post RFFE Loss [dBm] (Between RFFE and RFIC) | -18 | -86.5 | -81.5 | -85.9 |  |
| RFIC output [dBm] (Shared Rx RF chain) | 19.4 | -35.1 | -30.1 | -29.5 |  |
| RFIC output [dBm] (Partially shared Rx RF chain) | -60.6 |  |  |  |  |

Table 2 Simple link budget table for the CA configuration of n25(2A)

After conducting a simple link budget calculation, it is evident that the Tx leakage level of the shared Rx RF chain at the RFIC output is unacceptably high. While the AGC can be utilized to reduce the Tx leakage level in the RFIC block, it is important to note that the Tx leakage level is already quite strong in the analog block of the RFIC, which has the potential to reach the saturation limit. When the analog block saturates, it loses its ability to effectively amplify the received signal, resulting in distortion and degradation of the signal-to-noise ratio (SNR).

Observation 1: Despite the implementation of AGC to mitigate the Tx leakage level, it remains significantly high at the analog block of the RFIC. The strength of this leakage reaches the saturation limit and contributes to both distortion and degradation of the signal-to-noise ratio (SNR).

The primary reason for the high Tx leakage is the wider cut-off frequency of the analog filter. When using a partially shared Rx RF chain, separate analog filters can be employed for the PCC and SCC. Since each channel has a bandwidth of 5 MHz, a narrow filter response can be applied to both carrier components. However, with the implementation of a shared Rx RF chain, the filter response becomes wider, resulting in inadequate filter rejection at the Tx frequency. For instance, the total filter rejection of the shared Rx RF chain can be assumed to be 15 dB, considering the total channel bandwidth of CA_n25(2A) as 65 MHz, including the Wgap. This is in contrast to the approximately 80 dB difference in total filter rejection of the partially shared Rx RF chain. Therefore, it is essential to design an analog filter that provides greater rejection at the Tx frequency.

Observation 2: The strong Tx leakage in the shared Rx RF chain can be attributed to the wider cut-off frequency of the analog filter, which results in insufficient filter rejection at the Tx frequency.

Additionally, we have taken into account other impairments associated with the strong Tx leakage to determine the extent of extra relaxation needed to support the shared Rx RF chain architecture through AGC operation. Based on our findings, we recommend a minimum of 5-10 dB of additional relaxation to achieve the same signal-to-noise ratio (SNR) performance as that of the partially shared Rx RF chain.

Proposal 1: For CA_n25(2A) with Wgap = 55 MHz, we recommend a minimum of 5-10 dB of additional relaxation to achieve the SNR performance as that of the partially shared Rx RF chain.

- 3. Conclusion

In this contribution, we have conducted an analysis on the impact of the shared Rx RF chain architecture on DL performance, specifically with regards to the reference sensitivity requirement. Based on our analysis, the following observations have been made:

Observation 1: Despite the implementation of AGC to mitigate the Tx leakage level, it remains significantly high at the analog block of the RFIC. The strength of this leakage reaches the saturation limit and contributes to both distortion and degradation of the signal-to-noise ratio (SNR).

Observation 2: The strong Tx leakage in the shared Rx RF chain can be attributed to the wider cut-off frequency of the analog filter, which results in insufficient filter rejection at the Tx frequency.

Proposal 1: For CA_n25(2A) with Wgap = 55 MHz, we recommend a minimum of 5-10 dB of additional relaxation to achieve the SNR performance as that of the partially shared Rx RF chain.

- 4. Reference

| [1] | R4-2420337, WF on NR FR1 DL fragmented carriers study, MTK, RAN4#113 |
| --- | --- |