3GPP TSG RAN WG4 Meeting#114                                    R4- 2500825

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.20.2

Source:	vivo

Title:	Discussion on testing framework and procedure for one-sided models

Document for:	Discussion

- Introduction

In last RAN4 meeting, there are some further discussion on testability aspects for one-sided models, which are mainly for beam management and positioning accuracy enhancement [1].

Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs.

In this contribution, we further provide our views on the testability aspects for beam management and positioning accuracy enhancement.

- Testability issues for beam management

- 2.1 Channel model

In the RAN4#112 meeting, the following agreements about the channel models are captured

Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

As we discussed in the previous meeting, a flat channel (e.g., TDL channel model) could not reflect the real performance of AI model and channel models with spatial characteristic need to be emulated in the test system. Thus, RAN4 should at least focus on the CDL channel as the test environment. However, CDL channel with multi-clusters are too complicated in the whole sphere space to emulate. Fig. 1. Shows the power angular spectrum (PAS) of AoA and ZoA of CDL-A, CDL-B, CDL-C channel model with NLOS clusters in a sphere space.

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Fig. 1 Illustration of PAS of CDL-A, CDL-B and CDL-C channels

Based on the theory of prefaded signal (PFS) method to emulate the channel in OTA chamber, a great number of probes and high-number ports channel emulator will be implemented, which is not feasible for vendors. Thus, RAN4 needs to find a way to simplified the test system. For option 5, it is a feasible way to simplified the CDL channel model with a small number of clusters, but the test dependability will degrade. Based on the AI BM characteristic, we could have other feasible way and here is the analysis.

Observation 1: The simplified CDL channel with small number clusters will degrade the test dependability.

2.1.1 Feasible way to simplify the test system

UE always recognize the AoA direction of the strongest power path as the reference path for beamforming. When some certain different Tx beams are sweeping and transmitted in the channel environment, the AoA of the strongest power path will not change while the power is still changing. Fig. 2 shows an example of the barlett power spectrum for AoA estimation with different Tx beams in fading channel environment, where the UE is assumed as a 4×4 antenna array. It can be observed that at least for beam sets 2-3, 5-10, 13-14-… the AoAs of the strongest path in UE side are the same.

Fig. 2. Bartlett power spectrum for AoA estimation with different Tx beams

Based on the above understanding, some Tx beams can be grouped together (e.g., beam 5-10) if the strongest receive power AoA are the same. Besides, there are different ways to group the Tx beams and needs to be studied. For example, setting a range for AoA of the strongest path and using this range to group Tx beams. Strongest path AoA based grouping is just an example for introducing this theory. An illustration of the Tx beam grouping is shown in Fig. 3.

Fig. 3. Tx beams grouping

Since only AoA of the signal needs to be emulated in the anechoic chamber while different Tx beams in one group are corresponding to only one AoA, which means that only one probe with one AoA to emulate multiple Tx beams in one group is feasible, and the number of groups equals to the number of probes. TE only needs to change the power of the specific probe in the specific time to emulate the power of the different Tx beams transmitted through fading channel, which makes the test system much easier. Fig. 4. shows an illustration of multi-probe anechoic chamber test system with the Tx beam grouping method. In this MPAC system, only power and direction need to be emulated which is applicable for AI/ML based BM case.

In Fig. 4., the probe 1, 2, …, L is for emulating the Tx beams in group 1, 2, …, L, respectively. Only one probe is allowed to be active at the same time for transmitting the beams in the corresponding group. The Tx beams are realized in BS emulator and the fading channel is realized in channel emulator (CE). When the probe 1 is active and probe 2 to L are deactive, probe 1 could emulate the same AoA and different power of different beams in group 1, which are the basic elements for AI/ML based BM testing (L1-RSRPs and spatial characteristics).

Fig. 4. Illustration of MPAC test system with Tx beam grouping for emulating AoA and power

Besides, if other channel characteristics needs to be reconstructed in quiet zoom, the test system in Fig. 5 can be referred to reconstruct the channel environment, e.g., spatial correlation, PAS, etc. However, more probes will be needed in this MPAC system.

In Fig. 5, the probe set [1, 2, …,K] is for emulating the Tx beams in group 1, and another set […, i, j,…] is for emulating the Tx beams in group 2, and so on. Only one set of probes is allowed to be active at the same time for transmitting the beams in the corresponding group. When the probes in probe set [1, 2, …,K] are active and other probe sets are deactive, probe set [1, 2, …,K] could emulate the spatial correlation, PAS. This test system could emulate a more accurate environment that the test system in Fig. 5., but more probes are needed.

Fig. 5. Illustration of MPAC test system with Tx beam grouping for reconstructing channel environment

Therefore, following procedures can be used for simplification:

Step 1. Generate the BS beamforming power pattern of every Tx beam

Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

Step 5. Using one or small number of probes to emulate one beam group.

Step 6. Delete the unnecessary probes and merge some adjacent probes.

Observation 2: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 3: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 1: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

Proposal 2: RAN4 to consider the following procedures to simplify the test system:

- Step 1. Generate the BS beamforming power pattern of every Tx beam

- Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

- Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

- Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

- Step 5. Using one or small number of probes to emulate one beam group.

- Step 6. Delete the unnecessary probes and merge some adjacent probes.

Proposal 3: RAN4 to consider the following two approaches for design the test system

approach a

approach b

- 2.2 Test setup

In RAN4#112bis meeting, the following agreements have been achieved.

- Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

For this issue, we provide some simulation results for the OTA probe setup under the NLOS channel CDL-A, B, C based on the method proposed in section 2.1.1, where the 128 Tx beams set A beamforming characteristics are given as:

Azimuth angle in degree: [-84.375 -73.125 -61.875 -50.625 -39.375 -28.125 -16.875 -5.625 5.625 16.875 28.125 39.375 50.625 61.875 73.125 84.375]

Zenith angle in degree: [81 84 87 90 93 96 99 102]

The power patterns of the 128 Tx beams in azimuth cut and elevation cut (elevation angle in degree = zenith angle in degree – 90degree) are shown in Fig. 6.

(a)

(b)

Fig. 6 BS beamforming power pattern of 128 Tx beams

CDL-A

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 7 Probe allocation for CDL-A

CDL-B

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 8 Probe allocation for CDL-B

CDL-C

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Fig. 9 Probe allocation for CDL-C

Based on the probe allocation in Fig 7-9, it can be seen that some allocated probes are really close in angle-domain and can be merged and replaced by only one probe. Figure 10 shows an example for how to merging the probes in angle domain.

(a) original probe allocation for CDL-A, B, C (red: CDL-A, blue: CDL-B, green: CDL-C)

(b) new probe allocation after UE rotation in zenith angle domain

(c) 5 probe allocation after merge considering UE Rx resolution

Fig. 10 probe allocation after merge and rotation

In Fig. 10 (a), the allocated probes for CDL-A, B, C are illustrated and the total number is 31, which is not feasible. However, test should be performed under different channel models and no need to consider channel model switch (e.g., CDL-A to CDL-B) in one test. Thus, we can rotate the channel (rotate DUT in 3D-turnable) to narrow the channel model range in angular domain, which is shown in Fig. 10 (b). In Fig. 10 (b), all the channels are rotated in zenith angular domain and probes are symmetry about zenith angle = 90°. Besides, UE Rx beamforming has its resolution in angular domain, e.g., 30 deg. Fig. 10 (c) shows an example of probe allocation when considering UE Rx 30 degrees resolution, which eventually provided 5 candidates probe locations and greatly decreases the cost of the test system.

Proposal 4: Companies to provide candidate probe locations for CDL-A/B/C for merging and rotating. E.g.,

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Proposal 5: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations.

- Testability issues for positioning accuracy enhancement

Conformance and pre-deployment testing

For the conformance or the pre-deployment test, the issue of how to obtain the ground truth value needs to be first raised to considered. UE cannot derive its ground truth value of position and intermediate feature (except LOS/NLOS indicator) during the conformance testing by any methodology, if the legacy test method is used. The only way for DUT to derive the ground truth value is to deploy some PRUs in the test system.

For direct AI/ML positioning, it is not possible for UE to obtain its ground truth value for position, no matter which methodologies are used, e.g., GNSS, motion sensor, OTDOA, etc. All these methods would bring errors and cannot be treated as the ground truth value of target UE. However, the PRU (positioning reference unit), which is a type of use equipment with known location information, can perform positioning measurements, is introduced in Rel-18 positioning. With the known location and measurement information of nearby PRU, UE/NW can perform label-based performance validation by comparing the model output position with the known PRU location, while the model input is the channel measurement of PRU.

For AI/ML assisted positioning, PRU measurements together with the location information are still needed. Also, some other information, e.g. locations of TRPs stored in AMF, is also needed. Based on the assisted information, the ground truth value of intermediate feature can be calculated by geometry. Similar label-based validation procedure can be performed as the above contents for direct AI/ML positioning.

Based on the analysis, the test can also be feasible by performing label-based model output performance validation, and the test metrics should be KPIs defined in RAN4 accuracy requirement.

However, the label-based model input validation may be hard to conduct since the ground truth value of channel estimation is difficult to obtain. One potential feasible way is to transmit a package of additional reference dataset from LMF or server to the AI-deployed entity, and the dataset can be used as the ground truth for model input. In that case, the post-deployment and pre-deployment test can be feasible by comparing the provided ground truth value and the estimated channel measurements, and the test metrics should be KPIs defined in RAN4 accuracy requirement, if the model input KPI is identified by RAN4. Besides, if the reference dataset contains the ground truth value of model input and output, the test can be easily performed. The dataset can be generated by entity vendors or defined in the specifications.

Observation 4: PRUs can be deployed in the test scenario to provide measurements together with the location information.

Proposal 6: RAN4 to consider deploy PRUs in conformance or the pre-deployment test so as to derive the ground truth value of UE position and intermediate features.

Observation 5: At least LoS/NLoS indicator hard value can be tested since the LoS/NLoS environment is known in test system.

Generalization

AI/ML based positioning is a very area specific use case, especially for direct AI/ML positioning. According to RAN1 evaluation, assisted positioning with multi-TRPs is the methodology with better generalization performance than single-TRP. RAN1 also discussed other ways to improve the generalization performance, e.g., fine-tuning, which increases the performance in different drops. From RAN4 perspective, the testability for direct AI/ML positioning and assisted AI/ML positioning needs to be considered respectively. For the direct AI/ML positioning, RAN4 first needs to discuss whether to introduce the reference scenario/model/dataset. The test under reference scenario/model/dataset can provide a reference performance in certain conditions to guarantee the positioning performance would not degrade to an unexpected level in the field. For the assisted AI/ML positioning, since the generalization performance is better than AI/ML direct positioning, there may be no need to introduce the reference scenario/model/dataset, and the legacy test method can be reused. Further discussion is needed for the generalization aspects, and RAN4 needs to find out what kind of key factors would impact the performance of positioning.

Proposal 7: RAN4 to discuss the key factors that affect the performance and consider introduce reference scenario/model/dataset for AI/ML based positioning test for generalization.

- Summary

In this contribution, we further provided our views on testability related issues on one-sided models. Based on above analysis, following proposals are present.

Observation 1: The simplified CDL channel with small number clusters will degrade the test dependability.

Observation 2: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 3: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 1: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

Proposal 2: RAN4 to consider the following procedures to simplify the test system:

- Step 1. Generate the BS beamforming power pattern of every Tx beam

- Step 2. Add the BS beamforming power pattern into the power of different clusters in specific ZoD and AoD, and derive the new power of every cluster

- Step 3. Use the new power of clusters to generate the power angular spectrum (PAS) of ZoA and AoA

- Step 4. Some Tx beams with the corresponding ZoA/AoA are to be grouped together based on a certain rule (e.g., the ZoA/AoA of the strongest power derived from the PAS in step 3 are the same angle or in a pre-defined range)

- Step 5. Using one or small number of probes to emulate one beam group.

- Step 6. Delete the unnecessary probes and merge some adjacent probes.

Proposal 3: RAN4 to consider the following two approaches for design the test system

approach a

approach b

Proposal 4: Companies to provide candidate probe locations for CDL-A/B/C for merging and rotating. E.g.,

| (a) CDL-A | (b) CDL-B | (c) CDL-C |
| --- | --- | --- |

Proposal 5: RAN4 to consider the UE Rx beamforming resolution in angular domain to perform OTA probe allocations.

Observation 4: PRUs can be deployed in the test scenario to provide measurements together with the location information.

Proposal 6: RAN4 to consider deploy PRUs in conformance or the pre-deployment test so as to derive the ground truth value of UE position and intermediate features.

Observation 5: At least LoS/NLoS indicator hard value can be tested since the LoS/NLoS environment is known in test system.

Proposal 7: RAN4 to discuss the key factors that affect the performance and consider introduce reference scenario/model/dataset for AI/ML based positioning test for generalization.

- References

- R4-2420513 WF on AIML air interface part 1, Qualcomm.

- Z. Jiang, Z. Wang, C. Guo, Z. -C. Hao and W. Hong, Time-Domain Nonstationary Channel Emulation in Multiprobe Anechoic Chamber Setups for Over-the-Air Testing[J]. IEEE Antennas and Wireless Propagation Letters, 2021, 20(12):2511-2515.

- Appendix

- 6.1 Introduction of MIMO channel modelling

A MIMO communication system can be denoted as

Y(f,t)=H(f,t)X(f,t)+N(f,t)

Where Y∈CNu×1 is the received signal vector, Nu is the number the Rx antenna elements, X∈CMs×1 is the transmit signal vector, Ms is the number of Tx antenna number, N(f,t)∈CNu×1 denotes the noise, H(f,t)∈CNu×Ms is the channel matrix. Obviously, the multi-beam signals transmitted from the BS is Xf,t, the purpose of the test system is to reconstruct the spatial and time domain characteristics of the channel, which is H(f,t)∈CNu×Ms.

According to the 3GPP TR 38.901, the channel coefficient of s-th Tx element to u-th Rx element of the n-th cluster for the NLOS channel can be written as

Where Al(f,t)∈C2×2 is the polarization matrix

Al(f,t)=exp(jΦn,lθθ)κn,l-1exp(jΦn,lθϕ)κn,l-1exp(jΦn,lϕθ)exp(jΦn,lϕϕ)

Fig. 6. illustrates an example of 3D MIMO channel modelling

Fig. 11. The illustration of the 3D MIMO channel modelling

For LOS channel, we can have the similar analysis and details are not explained here.

For a conducted test, it can be observed that if the ZOA, ZOD of DL Tx beams are different, the channel parameters (e.g., the field pattern of Tx, the power of the clusters, etc) will be different. Besides, if UE movements are considered, it can be also observed that the AOA/ZOA of clusters will be changed. Also, the number and direction of clusters will be different during the UE movement. Note that velocity v can also be a time-varying factor that impact the Doppler.

However, for MIMO OTA tests, there are many different aspects needs be considered.

- 6.2 MIMO OTA channel modelling

One of the mainstream methods for MIMO OTA channel modelling is the prefaded signal synthesis (PFS) method. The PFS method allocates different power weight (real number, not complex number) at different probes in different locations, so as to reconstruct the target channel in the center of the test zone. The power weight and the time-domain characteristics are both realized in the channel emulator, every single cluster are synthesized by probes and all clusters are independent identically distributed. For an OTA MPAC system with K probes, the channel H'(f,t) can be denoted as product of the matrix F(f,t) which denotes the characteristics from the probes to the DUT Rx elements in the air and the matrix HOTA(f,t) realized in channel emulator

H'(f,t)=F(f,t)HOTA(f,t)

Assume there are U Rx elements in DUT side, then Ff,t={γu,k}∈CU×K, and γu,k can be written as

γu,k=Frx,u,θ(-ru,k)Frx,u,ϕ(-ru,k)TFo,k,θ(ru,k)Fo,k,ϕ(ru,k)L(du,k)exp(j2πf(ru,kT⋅du,k)c0)

Where Fo,k,θ and Fo,k,ϕ are the field pattern of the k-th OTA probe in the polarization direction,  and  respectively. ru,k is the sphere unit vector from k-th OTA probe to the u-th DUT Rx element, du,k is the distance between k-th OTA probe to the u-th DUT Rx element, L(du,k) is the path loss between them.

From the analysis, it can be observed that the matrix F(f,t) is decided by the DUT, OTA and the transmission space between them, different DUT, OTA probe and the space lead to different F(f,t). For HOTA(f,t) needs to be realized in the channel emulator, it can be written as the following for the Tx side with S antennal elements.

Where gn,k is the power weight of k-th probe for n-th cluster, Gid is the polarization matrix for OTA probe.

The purpose of the test system is to reconstruct the H'(f,t) similar as H(f,t) as much as possible. However, F(f,t) is always unknown or time varying and we need to use some method to derive the number of probes, location of probes and the power weight gn,k to emulate the spatial environment. Note that the power weights of probes are obtain by some optimization algorithms e.g., convex optimization, by setting some good target function.

- 6.3 AI/ML based BM scenarios analysis

For the BM case 1, if a certain number of beams of set B, e.g., NB, are to be emulated in the anechoic chamber, different AOD/ZOD in Tx side needs to be considered in HOTA(f,t) and same AOA/ZOA of clusters can be considered for the power weight of probes can be written as g∈RN×K×NB. The channel coefficient can be written as HOTA(f,t)∈CK×S×N×NB. For k-th probe, it can be denoted as

gk=g1,1…g1,NB⋮⋱⋮gN,1⋯gN,NB

Therefore, for BM case 1, channel emulator to realize the coefficients HOTA(f,t)∈CK×S×N×NB for NB beams of set B during the total testing time T as

Where K is the number of probes, S is the number of the Tx antenna elements, N is the number of clusters, NB is the number of the DL Tx beams. gn,k,nB is the power weight of k-th probe for the n-th cluster when nB-th DL Tx beam is transmitted in Tx side. Al(f,t)∈C2×2 is the polarization matrix.

For the BM case 2, the UE movement and the time domain non-stationary characteristics are considered for test. The straight forward thinking is that almost every parameter in the above channel coefficients are time-varing (add the independent variable t in the corresponding parameters). Besides, as we mentioned before, the AOA/ZOA and UE velocity needs to be considered. However, it is not easy to consider a continuous time domain procedure. Here we given a simple way to deal with the time-domain issue [2].

When UE is moving, the channel environment is changing all the time and the related parameters includes the AOA/ZOA, AOD/ZOD, LOS/NLOS, number of clusters, path loss, etc. Fig. 7. illustrates an example of the channel parameters changing during the UE movement.

Fig. 12. An illustration of the channel parameters changing during the UE movement

Thus, some non-static factors need to be considered in the BM case 2 test. We can consider to reconstruct the non-static channel in different moments or time-domain segments. Fig. 8. shows an example of the mapping of the non-static channel and the FR2 MIMO OTA test system. Note that the LOS in Fig. 8 is only for the LOS direction, it does not mean that the channel model under test is a LOS channel.

Fig. 13. An illustration of the mapping between of MIMO channel model and the FR2 MIMO OTA test system

RAN4 could define a UE trajectory with its channel environments, then separate the duration of UE movement into multiple NB segments. In every segment, the Tx side needs to emulate a fixed DL Tx beam and the total number of segments equals to the number of set B. For example, if the total testing time is T, it needs to be separated into NB segments as T1, T2, …, TNB, where T=T1+T2+…+TNB. Fig. 9. shows the illustration of test framework of BM case 2 when UE is moving.

Fig. 14. An illustration of test framework of BM case 2

Note that different or same probe deployment (number and position of probes) for different DL Tx beams could be considered for this test system. It depends on different implementation and cost of the test system. Fig .9. is a different probe deployment implementation for different DL Tx beam.

Therefore, for BM case 2, the total testing time T could be divided into NB segments as [T1, T2, …, TNB], where NB is the number of beams of set B. The power weight of probes, channel coefficients and the UE direction need to be considered in every single segment.

For BM case 2, channel emulator to realize the coefficients HOTA(f,t)∈CK×S×N×NB for NB beams of set B remain the same as proposal 4 for BM case 1 except velocity becomes vnB. and testing time is TnB

Where K is the number of probes, S is the number of the Tx antenna elements, N is the number of clusters, NB is the number of the DL Tx beams. gn,k,nB is the power weight of k-th probe for the n-th cluster when nB-th DL Tx beam is transmitted in Tx side. Al(f,t)∈C2×2 is the polarization matrix.