3GPP TSG RAN WG4 Meeting #114	R4-2500827

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.28.2

Source:	vivo

Title:	Further discussion on RRM requirements for XR

Document for:	Discussion

# Introduction

In the RAN4#113 meeting, there were further discussions on RRM requirements impact for XR. Following agreements were made and captured in the WF [1]. Open issues were also captured in the WF.

## Issue 1-1: Deployment scenarios < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID <Way forward>: - Further discussion needed for - FR2-2 - NR-DC ## Issue 1-2: Types of measurements to be considered <Agreement>: - The following measurement types are considered for XR measurement skipping - Intra-frequency - Inter-frequency - LTE-inter- RAT (NR-EUTRAN FDD/TDD) - The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS. ## Issue 2-1: General approach for defining requirements for measurements with gaps <Agreement>: - For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI. ## Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay ## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [2] was sent to RAN4 to trigger further discussions in RAN4.

Later on, RAN1 concluded solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions as working assumption and an LS [3] was sent to RAN4 for confirmation. RAN4 replied the LS in [4] in the last RAN4 meeting mentioning that RAN4 is discussing other solutions.

In [5, 6], we provided our views on the RRM requirements impact and issues raised in the RAN1 LSs. In this contribution, we further provide our views on RRM requirements impact for XR in Rel-19 and questions raised in the RAN1 LSs.

# RRM requirements impact

## 2.1 gaps/restrictions

RRM measurements can be categorized into L3 measurements and L1 measurements. L3 measurements can be further grouped as measurements within gaps and outside gaps. The scheduling restrictions are different for the three types of measurements.

L3 measurements within gaps

The L3 measurements within gaps are performed when measurement gaps are configured. During the measurement gaps, the UE is not required to conduct reception/transmission from/to the corresponding serving cells. The entire measurement gap is used for measurement only by receiving reference signals such as SSB, PRS.

L3 measurements outside gaps

The L3 measurements outside gaps are performed on SMTC occasions outside of measurement gaps if they are not fully overlapped. The scheduling restrictions are summarized as below.

- For UE which do not support simultaneousRxDataSSB-DiffNumerology for SSB with different subcarrier spacing than PDSCH/PDCCH on FR1 - The UE is not expected for transmission/reception on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - intra-band carrier aggregation: apply to all other serving cells in the same band - intra-band non-contiguous carrier aggregation configured for a UE indicating intraBandNR-CA-non-collocated-r18: scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided. - TDD bands on FR1 - The UE is not expected to transmit on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells - When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18, apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided - When TDD inter-band carrier aggregation is performed, apply to another serving cell in a different band if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair. - FR2 - The UE is not expected for transmission/reception on restricted symbols, on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured - intra-band carrier aggregation: apply to all other serving cells in the same band - inter-band carrier aggregation: for UE supporting IBM, similar restrictions as TDD bands on FR1

L1 measurements

For L1 measurements, it has similar restrictions as L3 measurement outside gaps but the restrictions are only on reference signal symbols for L1 measurements. It is summarized as below.

- FR1 - For UE which do not support simultaneousRxDataSSB-DiffNumerology SSB with different subcarrier spacing than PDSCH/PDCCH on FR1, - The UE is not expected for transmission/reception on symbols corresponding to the SSB indexes configured for L1-RSRP measurement - intra-band carrier aggregation: apply to all other serving cells in the same band - intra-band non-contiguous carrier aggregation configured for a UE indicating intraBandNR-CA-non-collocated-r18: scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided. - FR2 - The UE is not expected for transmission/reception symbols corresponding to the SSB indexes and/or periodic/semi-persistent/aperiodic CSI-RS resources which is NOT QCLed with active TCI state for PDCCH/PDSCH configured for L1 measurement - intra-band carrier aggregation: apply to all serving cells in the band on the symbols that fully or partially overlap with restricted symbols.

Moreover, L1 measurements including RLM/BFD/CDB/L1-RSRP/L1-SINR measurements. The reference signals for different L1 measurements may or may not be the same, or may or may not be overlapped.

It is observed that the UE cannot be scheduled during the measurement gaps. For L3 measurements outside gaps when certain conditions are met, the UE cannot be scheduled on some or all of the symbols within SMTC. For L1 measurements when certain conditions are met, the UE cannot be scheduled only on the reference signal symbols.

Observation 1: Scheduling restrictions are different in terms of conditions and restricted symbols for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements.

## 2.2 Scope

In the last RAN4 meeting, it was agreed that EN-DC and NE-DC for DCI-based solution is not supported for measurement gap skipping. It is FFS for NR-DC.

If a gap/restriction occasion is cancelled by MN via DCI, it may not be able to inform SN timely. At least, There are no RAN2 discussions on this so far. It means the SN cannot schedule the UE during the gap occasion being cancelled. Thus, benefit to support the feature is compromised in DC scenarios.

From RAN4 perspective, we may not define requirements for NR-DC scenarios, or we send an LS to RAN2/RAN1 to confirm whether it is supported or not.

Proposal 1: Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective.

Regarding the gaps/restrictions to be considered in the WI, following are for FFS.

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Skipping of measurements without gaps with scheduling restrictions are supported - Option 1a: Support skipping of measurements without gaps - Option 1b: Support skipping of measurements without gaps for 100MHz CBW band - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported - Option 2a: Measurement skipping does not apply for measurements without gaps - Option 2b: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction. ## Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures. - Option 1a: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 1b: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. - Option 1c: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures. - Option 2a: Measurement skipping does not apply for L1 measurements and procedures - Option 2b: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact. ## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Way forward>: - Option 1: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM - Option 3: Only L1 measurements for LTM within gaps are supported for gap skipping. - Option 4: For L1 measurement for mobility, apply same decision as normal L1 measurement.

L3 measurements outside gaps

L3 measurements outside gaps could be intra-frequency measurements, inter-frequency measurements and inter-RAT measurements. As analyzed in section 3.2, there could be some SMTC occasions for a MO within a time period are not used for measurements depending on measurement configuration. These SMTC occasions can be used for data transmission/reception. Similar to gaps, the SMTC occasions not used for measurements in the first place may not be enough for XR traffic. Additional SMTC occasions may be indicated by NW for data transmission/reception.

On the other hand, in FR1 the scheduling restrictions for L3 measurements outside gaps is only for UE which do not support simultaneousRxDataSSB-DiffNumerology. In general, mixed numerology of data and SSB is not a typical deployment. Moreover, it should be no extra complexity for UE to support this UE capability. Thus, it is better choice for XR device to support the UE capability simultaneousRxDataSSB-DiffNumerology so that it can be scheduled without restriction under mixed numerology deployment. In FR2 the scheduling restrictions for L3 measurements outside gaps is due to Rx beam sweeping. It cannot be avoided during measurements. Intra-frequency measurement is very important for UE mobility and long measurement delay in FR2 has been a concern that there is dedicated WI to reduce beam sweeping factor. If the SMTC occasions are skipped then longer measurement delay is expected. Especially when any SMTC occasions overlapped with XR data is skipped then mobility performance could be highly degraded.

Proposal 2: Measurement skipping does not apply for L3 measurements without gaps.

L1 measurements

L1 measurements include RLM, BFD, CBD or L1-RSRP/L1-SINR for beam reporting. L1 measurements are performed outside gaps. Similar to L3 measurements outside gaps, in FR1, the scheduling restriction is for UE which do not support simultaneousRxDataSSB-DiffNumerology. In FR2, it is due to Rx beam sweeping for SSB based measurements. However, for CSI-RS based L1 measurements there is no scheduling restriction when certain conditions are met.

The reference signals for L1 measurements includes SSB and/or CSI-RS, which may or may not be fully overlapped with SMTC occasions outside measurement gaps. If it is not fully overlapped with SMTC occasions, the measurement is performed on the reference signal symbols not overlapped with SMTC occasions. Otherwise, in FR2 it needs to share SMTC occasions for L3 measurements outside gaps. Since scheduling restriction for L1 measurements are only on L1-RS symbols which would not be many, the benefit of enabling data transmission/reception on the symbols is very small. Furthermore, it could have big impact on L1 measurement performance, especially for RLM/BFD measurements which is essential for link quality monitoring.

Therefore, it is not preferred to enable L1-RS symbols for data transmission/reception when there are scheduling restrictions.

Proposal 3: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

L1 measurement for LTM within gap is sharing the gap for L3 measurements. When a gap occasion is skipped, the L1 measurements for LTM may also be impacted. Moreover, only Type 1 measurement gap is support for LTM. Thus, it would be no problem to support L1 measurements for LTM within gaps.

As for L1 measurements for LTM outside gaps, it does not need to be considered similar to L3 measurements outside gaps and L1 measurements for serving cells.

Proposal 4: Only L1 measurements for LTM within gaps are supported for gap skipping.

## 2.3 Impact to requirements

There were extensive discussions on the impact to requirements due to gap skipping and how the requirements are to be specified. The following issues were captured.

## Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay <Way forward>: Proposals related to measurement delay requirements: - Option 1: Proposals without distinction of XR and legacy requirements - Option 1a: As minimum requirements, cell detection, index reading, and measurement period is extended. - Option 1b: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. - Option 1c: extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network. - Option 1d: RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR. - Option 2: Proposals with distinction of conditions where measurement impact is not needed - Option 2a: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information. - Option 2b: - Apply legacy measurement delay and accuracy requirements for the following condition: - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

The most important part is whether both the cases are supported, i.e., measurement impact is expected and measurement impact is not expected due to gap skipping.

L3 measurements within gaps

For the example of measurement configuration depicted in Fig 1, measurement gap is configured together with one inter-frequency layer. For this configuration, CCSFwithin_gap,i is 1 for the frequency layer. The UE needs all of the measurement gaps to meet the requirements.

Fig 1. Configuration of measurements on one inter-frequency layer with gaps

In this case, when some of the gaps are used for data scheduling the UE cannot meeting existing measurement requirements. UE needs additional gap occasions to get enough samples to meet the accuracy requirements. Since this is a very simple case, measurement delay can just be extended by number of skipped gap occasions.

For the example of measurement configuration depicted in Fig 2, measurement gap is configured together with three inter-frequency layers where different SMTC periodicity is used. For this configuration, CCSFwithin_gap,i is 3 for all the frequency layers.

Fig 2. Configuration of measurements on multiple inter-frequency layers with gaps

In this case, some of the gaps may not always be used to meet minimum measurement period requirements. The requirements are assumed that there are always 3 candidates of MOs to be measured within a gap. For MO#1 and MO#2, it is not the case. So, some of the gaps could be left unused as long as measurement period requirements are satisfied.

Suppose 2 samples are needed for one measurement period. one of the possible measurement implementations is depicted in Fig 3. Another possible measurement implementation is depicted in Fig 4.

Fig 3. Measurement implementation example 1

Fig 4. Measurement implementation example 2

Based on above analysis, it is observed that some of the gaps may not be used for measurements and it is up to UE implementation which gaps are not used for measurements.

Observation 2: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 3: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

In the case as depicted in Fig 2, Fig 3 and Fig 4, the unused gaps are about 37.5% (9/24). It is significant gain if these gaps can be used for data transmission/reception. Since it is up to UE implementation which gaps are not used, the information can only be provided by UE. Therefore, there is obvious benefit if UE can provide assistance information about which gaps are not used for measurements.

Observation 4: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Moreover, for some advanced UE, it may not use all the gaps during the measurement period specified by minimum requirements. For example, measurement period is based on 8 samples for FR1 inter-frequency measurements. The advance UE may just use 5 of them. So, the UE either not to use some of the gaps or use all the gaps with shorter measurement period. If the former case, then the gaps not used for measurement can be reported to network to schedule the UE during the gaps. Even for the example in Fig 1, there could be unused gaps due to advanced UE implementation.

If there are enough unused gaps and it is used for data scheduling there is no impact to measurements. On the other hand, depending on measurement configuration, there may be not enough unused gaps in which case there is impact to measurements. Therefore, both of the two cases should be supported.

Proposal 5: Both of the two cases, i.e., there is impact to measurements and no impact to measurements due to gap skipping, are supported.

## Issue 3-2: Principles for measurement delay calculation <Way forward>: - Option 1: Options based on scaling factor - Option 1a: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. - Option 1b: Introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window. - Option 1c: introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be when determining the exact scaling factor. - Option 1d: pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor). - Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. - Option 2a: dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

For the case where there is no impact to measurements, the UE should meet existing measurement requirements.

For the case where there is impact to measurements, the UE cannot meet existing requirements. Based on RAN1 working assumption, dynamic indication by DCI to enable Tx/Rx in particular gap/restriction is supported. If there is no restriction on the dynamic indication, then it would be difficult, if possible, to define new requirements because it is unknown what percentage of gaps and/or which gaps are skipped. The RRM measurement requirements are based on gap periodicity and gap sharing scheme. In multi-frequency layers scenario, frequency layers would have different ‘priority’. For example, for a frequency layer configured with 160ms SMTC periodicity may have higher priority than other frequency layers with shorter SMTC periodicity when competing a measurement gap. If those gaps for the frequency layer are all indicated by network for data transmission/reception, there is no chance for the UE to measure the frequency layer. Thus, UE mobility performance will highly be degraded.

From requirements perspective, it is reasonable to use NR-U like approach to extend the delay by the number of measurement gaps skipped during the measurement period. The measurement gap skipping is not periodic so it would not be possible to figure out a scaling factor. In essence, it is similar to NR-U that certain RRM resources/gaps cannot be used for measurement.

Proposal 6: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

Proposal 7: When there is impact to measurements due to gap skipping, RRM measurement requirements are defined by using NR-U like approach with carefully deciding the max number of gaps skipped during the measurement period.

## Issue 3-3: Conditions for measurement skipping <Way forward>: - Option 1: max number of occasions cancelled during a measurement period shall be defined. - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. - Option 3: discuss MG configurations relevant for MG cancellation, e.g., - minimum MGL (e.g., MGL≥3 ms, due to low gain with MG skipping), - maximum MGL (e.g., MGL≤6 ms), - maximum MGRP (e.g., MGRP≤320 ms to avoid too long delays), etc. - Option 4: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

If NR-U like requirements is defined for measurement gap skipping, it is necessary to limit the max number of gaps skipped during the measurement period. Otherwise, the mobility performance cannot be ensured.

Additionally, it is expected that some restrictions should be put on the dynamic indication. For example, the gaps to be skipped during a time period is indicated by RRC in advance. The DCI indicated gaps are only selected from those configured by RRC. This will largely simplify UE measurement scheduler which only needs to be conducted after RRC reconfiguration and ensure mobility performance is not degraded too much. Other potential restriction(s) that can help to achieve good trade-off between mobility performance and data delivery delay can also be considered.

Proposal 8: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified.

Proposal 9: RAN4 to study possible restrictions on dynamic indication by DCI, e.g., the gaps to be skipped during a time period is indicated by RRC in advance, to achieve good trade-off between data deliver delay and mobility performance.

# UE assistance information

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [3] were sent to RAN4 to trigger further discussions in RAN4.

Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

There serval aspects about UE assistance information had been discussed in RAN1.

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

RAN4 also had some discussions in the past meetings, but no progress was made.

## Issue 5-1: General on UAI <Way forward>: - Companies are tasked to discuss the need and feasibility of UAI, based on the following options - Proposals related to measurement occasions - Option 1: Define UAI related to measurement occasions - Option 1a: number of occasions which can be skipped - Option 1b: number of measurement occasions that are needed for measurement - Option 1c: UE indicates NW a status whether it can tolerate measurement cancellation. - Option 1d: patterns of gaps positions of MG that can be skipped - Option 1e: study how UE assistance information/UE capability is reported when DRX is configured. - Option 1f: Maximum number of consecutive MGs that can be skipped - Option 2: - Option 2a: Do not define UAI related to measurement occasions. - Option 2b: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution - Option 2c: RAN4 will not specify any XR-related requirement, based on XR UAI. - Option 3: The information about gaps not used for measurement are reported as UE capability. - Option 4: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions. - Proposals related to channel conditions - Option 1: For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

As discussed in section 2.3, it is proposed to support both of the two cases, i.e., there is impact to measurements and no impact to measurements due to gap skipping.

For the case where there is no impact to measurements, UE assistance information is needed. Since it is up to UE implementation which gaps are not used, the information can only be provided by UE. The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided. Reporting number of occasions used or not used for measurement cannot ensure there is no impact to measurements. Other options also have unknown and unexpected impact on UE mobility performance and it provides no information on which gaps have higher priority, e.g., most loaded gaps or gaps for sparse reference signals.

Proposal 10: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information.

Then it comes the question that the reported gaps not used for measurement may not be enough for data transmission/reception considering QoS requirements. In some cases, there may be no gaps not used for measurement depending on measurement configuration. However, when enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, it needs to keep impact to mobility performance limited. In order to achieve this goal, it needs UE to provide information of gaps not used for measurement. If it is totally up to NW indication, the mobility performance could highly be degraded as gaps may have different priority for measurements, e.g., the most loaded gap has highest priority and it may be used to measure sparse reference signals and should not be used for data transmission/reception, or if gap sharing scheme is not equal sharing, some gaps may have high priority to meet requirements.

One possible solution is that NW indicates the percentage of gaps not used for measurement during a period of time. It is up to UE to decide which gaps are not used for measurement during the period of time. This could be the best balance between QoS requirements of data transmission/reception and UE mobility performance.

Proposal 11: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement.

If both of the cases are supported, network needs to know whether measurements are impacted or not. This can also be indicated by UE because only UE knows whether it can meet existing requirements based on measurement configurations and UE implementation.

Proposal 12: UE to indicate whether it can meet L3 measurements requirements when unused gaps reported by UE is used for data scheduling.

When DRX is configured, the UE may measure once within gaps in one DRX cycle. Only the gaps occasions overlapped with DRX on duration can be used data transmission/reception. There could be different reporting scheme for UE assistance information, e.g., only reporting gaps overlapped with DRX on duration within a time period.

Proposal 13: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

Regardless of the gaps not used for measurements is due to measurement configuration or advanced UE implementation, it is more like UE capability rather than UE assistance information. It is better that RAN4 to discuss which scheme should be used. The difference is that NW will follow UE reporting if it is UE capability, but the NW may not follow UE reporting if it is UE assistance information depending on current specification. At least for the case where measurement is not impacted, it is better to report as UE capability similar to “NeedForGaps”.

Proposal 14: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information.

Regarding the UAI related to channel conditions, the proposal on UE assistance information is that for UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

s-MeasureConfig is the threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. When serving cell RSRP is above the threshold, UE may not perform measurements on neighbour cells. In this case, measurement gaps, if configured, may not be used for measurement at all.

However, UE still needs to perform measurements on SMTC occasions for serving cell, which are also the same for all neighbour cells on the serving cell frequency, if the measurements are performed outside gap. The channel condition of serving cell RSRP does not help UE to provide information about which SMTC occasions within a time period are not used for measurements.

Moreover, when serving cell RSRP is below the threshold, UE needs to measure neighbour cells where measurement gaps could be used. The information about gaps not used for measurement should still be provided.

Proposal 15: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Additional information about gaps/SMTCs not used for measurement is still needed.

# Solutions for enabling Tx/Rx in gaps/restrictions

## 4.1 Indication solution

RAN1 concluded solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions as working assumption and an LS [3] was sent to RAN4 for confirmation. RAN4 replied the LS in [4] in the last RAN4 meeting mentioning that RAN4 is discussing other solutions.

Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

In the RAN1#118 meeting, RAN1 concluded details of DCI-based explicit indication for gap skipping.

Agreement For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”. - FFS: Interpretation of bit value equal to “0”. Agreement A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component. Agreement For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

For explicit indication by DCI, there are two options how UE would interpret different values for a gap skipping indication. UE only needs to support one option. As the example below, it could mean completely different UE behavior even if the same value is indicated by the DCIs.

Fig 5. Example of explicit DCI indication options

For option 2, UE needs to decide if a gap can be skipped by the latest DCI indication at least 5ms ahead of the start of the gap. For option 1, UE needs to decide if a gap can be skipped by the DCI indication during the gap period and 5ms ahead of the start of the gap. If there is DCI indication with value ‘1’, then the gap shall be skipped. For the example in Fig 5, the two gaps shall be skipped if it is option 1 but it shall not be skipped if it is option 2.

For both options, UE needs to decide if a gap can be skipped by monitoring DCIs contiguously. This complicates UE implementation as the UE needs to re-schedule measurements after a gap skipping. More importantly, it leaves UE less possibility to optimize measurements based on measurement configurations which will degrade mobility performance.

Observation 5: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps.

However, if gap skipping is explicit indicated by RRC it could give UE flexibility to optimize UE measurement. For example, if UE reports gap patterns that can be skipped via UE capability and NW just to indicate those gaps then it is the best to indicate via RRC with the reported gap patterns. With this, there is no impact to measurement requirements and UE doesn’t need to re-schedule the measurements when a gap is skipped. So, at least RRC based signaling should be supported for such cases.

Multiple options for semi-static schemes were proposed in the last RAN4 meeting.

## Issue 6-1: Semi-static schemes <Way forward>: - Option 1: Follow RAN1 working assumption and consider DCI only. - Option 2: also consider RRC/semi-static approach. - Option 2a: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario. - Option 2b: RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion. - Option 2c: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period. - Option 2d: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. - Option 2e: In addition to DCI solution, it is proposed to introduce semi-static schemes. - Option 3: - RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported). - RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

Our understanding is that RAN1 would not discuss this anymore. RAN4 needs to make decision whether semi-static schemes should be supported.

Proposal 16: RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported.

RAN1 designed a uniform triggering mechanism for both gap skipping and restriction skipping. However, from RAN4 perspective, it is under discussion whether skipping restrictions for L3 measurement outside gap and/or for L1 measurement is supported or not. It needs to inform RAN1 of the conclusion on this.

Proposal 17: RAN4 LS to RAN1/2 the agreements on introduction of RRC-based indication, and additionally if restriction skipping for L3 measurement outside gap and/or L1 measurement is not supported from RAN4 perspective.

## Issue 6-2: Skipping indication on frequency domain <Way forward>: - Proposals - Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3: It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s).

The measurement gap can be either per UE or per-FR. If a per UE gap is skipped, all the serving cells can be scheduled. If a per-FR gap is skipped, all the serving cells in the frequency range same as the per-FR gap can be scheduled. When a per-UE gap is configured for measurement, the DCI can explicitly indicate whether the first gap shall be skipped. However, it is a bit complicated when a per-FR gap is configured.

Per-FR gaps may be configured if a UE supports independent measurement gap configurations for FR1 and FR2. Since the DCI indication needs to be 5ms ahead of the actual gap occasion, it is not clear gap in which FR should be skipped if value ‘1’ is included in a DCI, especially when the two gaps are overlapped in some occasions as depicted in Fig 6.

Fig 6. Per-FR Configuration of measurement gaps

One possible solution is to indicated in advance via RRC configuration which gap(s) are to be skipped by further DCI indication.

Proposal 18: RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

# Summary

In this contribution, we further provided views on RRM requirements impact for XR in Rel-19 and on questions raised in the RAN1 LSs. Following proposals and observations are present.

Proposal 1: Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective.

Proposal 2: Measurement skipping does not apply for L3 measurements without gaps.

Proposal 3: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

Proposal 4: Only L1 measurements for LTM within gaps are supported for gap skipping.

Proposal 5: Both of the two cases, i.e., there is impact to measurements and no impact to measurements due to gap skipping, are supported.

Proposal 6: When only those gaps not used for measurement based on UE assistance information / UE capability are scheduled for data transmission/reception, the UE should meet existing requirements.

Proposal 7: When there is impact to measurements due to gap skipping, RRM measurement requirements are defined by using NR-U like approach with carefully deciding the max number of gaps skipped during the measurement period.

Proposal 8: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified.

Proposal 9: RAN4 to study possible restrictions on dynamic indication by DCI, e.g., the gaps to be skipped during a time period is indicated by RRC in advance, to achieve good trade-off between data deliver delay and mobility performance.

Proposal 10: The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information.

Proposal 11: NW may indicate the percentage of measurement gaps not used for measurement during a time period for UE to provide information about gaps not used for measurement.

Proposal 12: UE to indicate whether it can meet L3 measurements requirements when unused gaps reported by UE is used for data scheduling.

Proposal 13: RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

Proposal 14: RAN4 to discuss the information about gaps not used for measurement are reported as UE capability or UE assistance information.

Proposal 15: Channel condition, e.g., RSRP, does not provide information about which SMTC occasions are not used for measurements. Additional information about gaps/SMTCs not used for measurement is still needed.

Proposal 16: RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported.

Proposal 17: RAN4 LS to RAN1/2 the agreements on introduction of RRC-based indication, and additionally if restriction skipping for L3 measurement outside gap and/or L1 measurement is not supported from RAN4 perspective.

Proposal 18: RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

Observation 1: Scheduling restrictions are different in terms of conditions and restricted symbols for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements.

Observation 2: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 3: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

Observation 4: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Observation 5: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps.

- References

- R4-2420103	WF on RRM requirements for NR_XR_Ph3, Nokia

- R1-2405736	LS on UE assistance information, RAN1

- R1-2407561	LS on gaps/restrictions that are caused by RRM measurements, RAN1

- R4-2420198	Response LS on gaps/restrictions that are caused by RRM measurements, RAN4

- R4-2415198	Further discussion on RRM impacts for XR and reply LS, vivo

- R4-2418759	Further discussion on RRM impacts for XR and reply LS, vivo

- Appendix: Draft reply LS on UE assistance information

3GPP TSG RAN WG4 Meeting #114	R4-24x

Athens, Greece, 17th – 21st February, 2025

Title:	Reply LS on UE assistance information

Response to:	R1-2405736 (R4-2411007)

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 thanks RAN1 for the LS on UE assistance information.

RAN4 discussed the UE assistance information for reporting not used measurement gas/SMTC occasions to network. Following conclusions are made.

From UE measurements and mobility performance perspective, RAN4 agreed that it is beneficial if UE reports [UE assistance information / UE capability] about not used measurement gaps/SMTC occasions to network and network indicates skipping gaps/SMTC occasions based on the information.

In order for NW and UE to have same understanding on which gaps should be not used for measurement, the patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as [UE assistance information / UE capability].

RAN4 will further study the details and additional conditions of [UE assistance information / UE capability]. RAN1 will be informed once there are new conclusions.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 agreements into account in the future work.

3. Reference:

- R1-2405736	LS on UE assistance information, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #114bis	 	   April 7 – April 11, 2025			China

- Appendix: Draft LS on indication solution for gap/restriction skipping

3GPP TSG RAN WG4 Meeting #114	R4-24x

Athens, Greece, 17th – 21st February, 2025

Title:	LS on indication solution for gap/restriction skipping

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 sent a reply LS R4-2420198 in the last RAN4 meeting with following statement.

Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

RAN4 further discussed semi-static indication solutions and made following agreement.

- RRC based indication solution is supported in addition to DCI-based indication solution. It is up to UE which solution is supported.

- Gap patterns during a period for gap skipping is indicated in RRC.

RAN4 would like to kindly ask RAN2 to introduce necessary RRC signaling. RAN4 will further study how the gap patterns are specified.

2. Actions:

To: RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to introduce necessary RRC signalling for gap/restriction skipping.

3. Reference:

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #114bis	 	   April 7 – April 11, 2025			China

- Appendix: Draft LS on scenarios for gap/restriction skipping

3GPP TSG RAN WG4 Meeting #114	R4-24x

Athens, Greece, 17th – 21st February, 2025

Title:	LS on scenarios for gap/restriction skipping

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 discussed deployment scenarios for gap/restriction skipping. RAN4 made agreements that NR SA and NR carrier aggregation is supported from RAN4 requirements perspective. EN-DC and NE-DC are not supported. It is FFS for NR-DC

If a gap occasion is skipped by MN via DCI indication, there may not be information exchange between MN and SN. It means the SN cannot schedule the UE during the gap occasion cancelled. Thus, benefit to support the feature in DC scenarios is compromised.

RAN4 would like to ask RAN2 and RAN1 if NR-DC scenario is well supported for gap/restriction skipping.

2. Actions:

To: RAN WG2, WG1

ACTION: 	RAN4 kindly asks RAN2 and RAN1 to provide feedback on supporting of NR-DC scenarios for gap/restriction skipping.

3. Reference:

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #114bis	 	   April 7 – April 11, 2025			China