3GPP TSG-RAN WG4 Meeting #114	R4-2500837

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for ATG CA operation

Agenda item:	7.12.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, the list of requirements to be defined for ATG DL CA scenario was agreed [1], but there are still some open issues such as UE antenna type related requirements. In this contribution, we provide our views on open issues for ATG with DL CA operation.

Discussion

General

UE antenna type

Agreement in RAN4#112bis - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

In the last RAN4 meeting, RF session has discussed whether to support the Case 4 antenna type for ATG DL CA, but there was no agreement for the discussion since RAN4 assumed separate Rx beam for inter-band CA when array antenna is considered in FR2. Therefore, RAN4 RRM should wait for RF conclusion for Case 4 antenna type for ATG CA.

- Proposal 1: RAN4 RRM to wait for RF conclusion for Case 4 antenna type discussion

If Case 4 antenna type is considered in RRM requirements, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid RF impact since RF architecture of Case 4 antenn type which assumes common Rx beam would be that one LNA and phase shifter would be shared for two bands, and separate local oscillator were considered.

- Proposal 2: To support Case 4 antenna type in RRM requirements, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid RF impact, e.g., conditions for intra-band NCCA operation.

UE Antenna Type signaling

FFS - Proposal 1: If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side. - Proposal 2: To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed with per BC type for the bands in which UE supporting phase antenna array.

Some of requirements such as scheduling availability could be different depending on ATG UE antenna types. So, the information for antenna type used by UE side should be provided to to network.

- Proposal 3: If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side.

Timing

MRTD

To define MRTD, RAN4 should follow how to derive the MRTD value. MRTD is based on TAE/cell phase synchronication accuracy and propagation delay difference. Existing 33us MRTD requirement for inter-band CA has been defined under assumption that 30us propagation delay difference and 3 us TAE/cell phase synchronization accuracy since non co-located CA deploymenet is considered. However, for ATG inter-band DL CA, since only co-located deployment is considered, MRTD does not need to account for propagation delay differences. If only inter-band CA UE regardless of co-located or non co-located is assumed, 33usec might be considered. However, if array antenna type with common Rx beam is considered, MRTD for Case 4 antenna type should be at least 3us.

- Proposal 4: For ATG inter-band DL CA, the MRTD for Case 4 antenna type should be at least 3us.

Signalling characteristics

Interruption requirements

Agreement: - Introduce following requirements for R19 ATG - Interruptions at SCell addition/release - Interruptions at SCell activation/deactivation - Interruptions at direct SCell activation - Interruptions at fast SCell activation - Interruptions during measurements on deactivated SCC - Interruptions due to SCell dormancy - Interruptions due to UE-specific CBW change - Interruptions due to Active BWP switching Requirement - Interruptions when identifying CGI of an NR cell with autonomous gaps - Not introduce following requirements for R19 ATG - Interruptions at UL carrier RRC reconfiguration - Interruptions at inter-frequency SFTD measurement - Interruptions at NR SRS carrier based switching - Interruptions due to PUCCH SCell activation/deactivation - Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18] - FFS whether following requirements need to be introduced - Interruptions at NR SRS antenna port switching - Further discuss the potential impact from UE antenna types - Further discuss how to reflect the interruption requirement for R19 ATG

In the last RAN plenary, DL MIMO has been added in the WID [2]. Therefore, SRS antenna port switching can be considered in ATG. The existing interruption requirements at SRS antenna port switching can be resued, and only DL interruption is allowed.

- Proposal 5: Introduce interruption requirement at SRS antenna port switching, and only DL interruption should be considered based on existing interruption requirement.

Measurement procedure

Deactivated SCell measurement

In the last meeting, RAN4 agreed that beam sweeping factor N1 is needed, and FFS on L3/L1 measurement sharing factor Klayer1_measurement. RAN4 agreed to define intra-frequency measurement requirements for deactivated SCell by reusing the principle from the existing cell identification and measurement requirements with measCycleScell as baseline. So, RAN4 do not need to consider L3/L1 measurement sharing factor Klayer1_measurement.

- Proposal 6: RAN4 not to consider L3/L1 measurement sharing factor Klayer1_measurement for deactivated SCell measurement

Conclusion

In this contribution, we provide views on RRM requireiments for ATG with CA operation, and we propose

- Proposal 1: RAN4 RRM to wait for RF conclusion for Case 4 antenna type discussion

- Proposal 2: To support Case 4 antenna type in RRM requirements, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid RF impact, e.g., conditions for intra-band NCCA operation.

- Proposal 3: If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side.

- Proposal 4: For ATG inter-band DL CA, the MRTD for Case 4 antenna type should be at least 3us.

- Proposal 5: Introduce interruption requirement at SRS antenna port switching, and only DL interruption should be considered based on existing interruption requirement.

- Proposal 6: RAN4 not to consider L3/L1 measurement sharing factor Klayer1_measurement for deactivated SCell measurement

Reference

[1] R4-2420237, “WF on RRM requirements for NR_ATG_enh,” CMCC

[2] RP-243285, “Revised WID on Enhancements for Air-to-ground network for NR,” CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom