3GPP TSG-RAN WG4 Meeting # 114     	R4-2500843

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.12.3

Source: 	CMCC

Title: 			 Discussion on RRM core requirements for R19 ATG

Document for:	Discussion

- Introductions

In last meeting, RAN4 continued to identify which features and requirements will be applied and defined for Air-to-ground network enhancement, the WF has been approved [1]. In this contribution, we further discuss the RRM impact and potential enhancement considering ATG specific CA scenario.

- Discussion

  - General

Issue 1-1-1: UE antenna type for inter-band CA

FFS

- Proposal 1: To support Case 4 antenna type, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid critical receving performance degradataion, e.g., conditions for intra-band NCCA operation.

- Proposal 2: Confirm the understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

- Proposal 3: Wait for more progress from RF session.

  - Proposal 3-1: If the implementation of Case 4 is not excluded in RF session, RRM requirement and related UE capability should be defined, details can be discussed later.

Issue 1-1-2: UE Antenna Type signaling

FFS

- Proposal 1: If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side.

- Proposal 2: To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed with per BC type for the bands in which UE supporting phase antenna array.

In last meeting, RF session discussed whether to preclude case 4, we copy the approved WF as the following [2]:

Issue 3-4-1: whether preclude Case 4. WF: - Further check whether preclude case 4.

Since no consensus achieved so far, we should further wait the progress from RF before making our final decision.

If Case 4 is excluded in RF session, it should be also excluded in RRM requirement.

If Case 4 is not excluded in RF session, we think RRM should consider Case 4 for requirement definition. Besides, due to Rx beam type have impact on network scheduling, as analyzed in Issue 1-5-8 [2], the UE capability should be introduced to inform network about the UE Rx beam type. Details can be discussed later.

Proposal 1: For the feasibility of Case 4, wait for more progress from RF session.

Proposal 2: If the implementation of Case 4 is not excluded in RF session, RRM requirement and related UE capability should be defined, details can be discussed later.

  - Timing requirements

Issue 1-2-1: MRTD for ATG inter-band CA

FFS

- Option 1:

  - For the MRTD except Case#4: 33µs. Further discuss for case 4.

- Option 2:

  - For the MRTD except Case#4: 33µs.

  - If case 4 is not excluded based on RF discussion: 3µs.

- Option 3:

  - For the MRTD for all cases: 33µs.

For inter-band CA, the legacy FR1 MRTD requirement is 33µs which including 3µs TAE and propagation delay difference.

For ATG UE except antenna type Case 4, although propagation delay difference between PCell and SCell is near to 0 in ATG co-located deployment, we think 33µs MRTD can be reused, since ATG UE is more powerful than legacy hand-held UE, such approach is also more friendly to forward compatibility.

For ATG UE antenna type Case 4, after we reconsider the RF structure, that UE use common antenna panel but separate Rx chains between PCC and SCC, it may also be able to handle the MRTD 33µs. We prefer to further check it after more progress from RF session.

Proposal 3: Define MRTD requirement for ATG inter-band CA

- For ATG UE except antenna type Case 4, define the MRTD requirement as 33µs.

- For ATG UE antenna type Case 4 if introduced, further consider the MRTD requirement as 33µs.

  - Signalling characteristics requirements

Issue 1-3-1: Interruption requirements

Agreement:

- Introduce following requirements for R19 ATG

  - Interruptions at SCell addition/release

  - Interruptions at SCell activation/deactivation

  - Interruptions at direct SCell activation

  - Interruptions at fast SCell activation

  - Interruptions during measurements on deactivated SCC

  - Interruptions due to SCell dormancy

  - Interruptions due to UE-specific CBW change

  - Interruptions due to Active BWP switching Requirement

  - Interruptions when identifying CGI of an NR cell with autonomous gaps

- Not introduce following requirements for R19 ATG

  - Interruptions at UL carrier RRC reconfiguration

  - Interruptions at inter-frequency SFTD measurement

  - Interruptions at NR SRS carrier based switching

  - Interruptions due to PUCCH SCell activation/deactivation

  - Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

- FFS whether following requirements need to be introduced

  - Interruptions at NR SRS antenna port switching

- Further discuss the potential impact from UE antenna types

- Further discuss how to reflect the interruption requirement for R19 ATG

For antenna array UE, the UL-MIMO is supported, and the objective of DL MIMO has been introduced in the latest WID [3]. Network is able to configure the SRS antenna port switching to help itself to do the DL channel estimation. Hence, we propose to introduce the interruption requirement of NR SRS antenna port switching.

Proposal 4: Introduce the interruption requirement of NR SRS antenna port switching.

Based on our understanding of interruption requirements, it comes from the RF chain interaction among PCell and SCells. For example, if the RF chain of SCell need to be activated, the RF chain of PCell may be impacted by this activation. During this interruption process, we didn’t see any impact from UE antenna types.

Proposal 5: Reuse the current interruption requirements, no impact from UE antenna types.

Issue 1-3-2: SCell Activation and Deactivation Delay

FFS

- Proposal 1: The SCell activation delay requirements will not be impacted by ATG UE antenna type.

- Proposal 2: Inter-band SSB-less SCell activation requirements can apply to omnidirectional antenna type.

- Proposal 3: SCell activation requirements shall be further investigated for potential impact, considering the UE antenna types.

In last meeting, the proponents of proposal 2 thinks that the inter-band SSB-less SCell activation requirements can not apply to the ATG UE with antenna array on the SCC. The reason is that the legacy SSB-less requirement does not address beam-related issues.

However, in our understanding, in ATG scenario, UE conduct the serving cell beam steering by using the BS location assistance information, no additional time needed due to SSB-less SCell from UE perspective. From network perspective, network could acquire the Tx beam of SSB-less SCell by the Tx beam of PCell, no additional procedure needed for assisting network in determining the Tx beam direction. Therefore, the inter-band SSB-less SCell activation requirements can be applied regardless of UE antenna type on the SCC.

Proposal 6: The inter-band SSB-less SCell activation requirements can be applied regardless of UE antenna type on the SCC.

After examine the current SCell activation delay requirements, we found that only the P value of ‘TL1-RSRP, measure’ have relation with antenna type of SCC, because ATG UE with antenna array couldn’t perform neighbour cell measurement and serving cell L1 measurement simultaneously. This has been specified in R18 in TS38.133 Clause 9.5D, we copy below for information.

Therefore, for the SCell Activation and Deactivation Delay requirement of ATG, we only need to refer the definition of ‘TL1-RSRP, measure’ to the ATG Clauses.

For ATG UE with the antenna array, P value for SSB resource to be measured is defined as - Ntotal / Navailable with Navailable > 0 - Psharing factor * Ntotal / Noutside_MG with Navailable = 0 - For a window W of duration max(TL1, MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion: - Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W - TL1 is periodicity of the target SSB - Psharing factor = 3. Otherwise, for UE with one or multiple omni-directional antennas For a UE supporting concurrentMeasGap-r17 and when concurrent gaps are configured, - P value for SSB resource to be measured is defined as - Ntotal / Noutside_MG - For a window W of duration max(TL1, MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion: - Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W - TL1 is periodicity of the target SSB. Otherwise, for a UE not supporting concurrentMeasGap-r17 or when concurrent gaps are not configured, - P=11−TSSBxGP, when in the monitored cell there are GAPs configured for intra-frequency or inter-frequency, which are overlapping with some but not all occasions of the SSB; and - P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB. Where: - TSSB = ssb-periodicityServingCell of the serving cell - an SSB or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and - xRP = MGRP

Observation 1: In each components of SCell activation delay requirements, only the P value of ‘TL1-RSRP, measure’ will pending on the ATG UE antenna type on SCC, while this impact has already been identified and specified in R18.

Proposal 7: Refer the definition of ‘TL1-RSRP, measure’ to the ATG Clause 9.5D.

For the other components, due to the UE actions related to the activation command will be applied at the first opportunities once SCell is activated, and UE can perform the deactivated SCell beam steering based on the serving cell location information, we think the activation delay requirements will not be impacted by ATG UE antenna type.

Proposal 8: For SCell activation delay requirements, no other impact caused by ATG UE antenna type.

Issue 1-3-3: SCell Addition/Activation mechanism

FFS

- Proposal 1: Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

- Proposal 2: Further consider distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition

- Proposal 3: Should not consider distance triggered SCell addition and/or activation; existing activation methods are sufficient

With LOS propagation condition and co-located CA deployment, the coverage of PCell and SCell are relatively fixed, as shown in figure below.

In this case, SCell quality is highly related with distance between UE and cell center. We propose to consider distance triggered SCell addition and/or activation, which has following advantages:

1. helping UE to reduce power consumption from measurement for SCell addition and/or activation.

2. For ATG UE with antenna array in intra-band CA scenario, and for ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed in inter-band CA scenario, due to measurement for SCell addition and/or activation can be omitted, no need to apply scheduling restriction for neighbour cell measurement on candidate SCC, throughput gain can be achieved.

The distance triggered SCell addition and/or activation can be achieved through network configuration of SCell addition and/or activation condition, which similar as distance condition in location-based CHO. If the distance between UE and the configured reference point is less than the threshold, the condition is fulfilled, UE can add and/or active the SCell automatically.

After UE add and/or active the SCell, network can further active or schedule the DL/UL at this SCell, so it is necessary to inform network about the SCell state at UE side. Therefore, UE should sent the SCell addition/activation confirmation message to network after it add and/or active the SCell automatically.

Proposal 9: To save power and avoid throughput loss caused by measurement for SCell addition and/or activation, introduce distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition. After the condition is fulfilled at UE side, UE can add and/or active the SCell automatically, and then sent the SCell addition/activation confirmation message to network.

  - Measurement procedure requirement

Issue 1-4-4: Deactivated SCell measurement

Agreement:

- beam sweeping factor N1 is needed.

- FFS on L3/L1 measurement sharing factor

The deactivated SCell measurement requirement including Time period for PSS/SSS detection, Time period for time index detection, Measurement period for intra-frequency measurements without gaps for FR1 deactivated SCell.

For L1/L3 measurement sharing factor, due to there is no L1 measurement on deactivated SCell, at least for the ATG UE antenna types except for Case 4, no need to introduce L1/L3 measurement sharing factor. However, for the ATG UE antenna type Case 4, the L1/L3 measurement sharing factor is needed, since there is only one common beam, and UE need to perform L1 measurement on the PCell.

Proposal 10: Based on the legacy requirement:

- For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

- For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed

Issue 1-4-6: Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1

FFS

- Proposal 1: For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, the scheduling restriction extension to intra-band/inter-band CA should be identified for R19 ATG

  - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

  - For inter-band CA,

    - For case 2, case 3 and case 5, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to SCell/PCell will not apply to another serving cell in a different band.

    - If case 4 is considered, the scheduling restrictions on receiving PDCCH/PDSCH/TRS/CSI-RS for CQI due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

For intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements, scheduling availability of UE with the antenna array performing measurements on FR1 shall be discussed.

When intra-band carrier aggregation in FR1 is configured, due to one set of antenna and Rx beam will be applied on each carrier, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When inter-band carrier aggregation is performed:

For ATG UE antenna type Case 1, 2, 3 and 5, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

For ATG UE antenna type Case 4, i.e., ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

Proposal 11: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements:

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When inter-band carrier aggregation is performed:

  - For ATG UE antenna type Case 1, 2, 3 and 5, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

  - For ATG UE antenna type Case 4, i.e., ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

Issue 1-4-7: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

FFS

- Proposal 1:

  - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

    - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

- Proposal 2:

  - For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inta-band carrier aggregation:

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

  - For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inter-band carrier aggregation:

    - For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

    - For ATG UE with the antenna array on PCell and one or more omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCel configured in other bands than the bands in which PCell is configured.

    - For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

    - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

    - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

For RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, L1-SINR measurement, the scheduling availability was introduced when the UE performs measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1, but doesn’t support following related UE capabilities:

| simultaneousRxDataSSB-DiffNumerology Indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 Indicates whether the UE supports concurrent SSB based inter-frequency measurement without measurement gap on neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology as defined in clause 8 and 9 of TS 38.133 [5]. UE indicates support of this indicates support of interFrequencyMeas-NoGap-r16. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range where the SSB and PDCCH/PDSCH are received. | UE | No | No | Yes |

Furthermore, when intra-band contiguous carrier aggregation in FR1 is configured, due to one set of tansceiver will be applied on this carrier, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, due to UE mandatory capable of two sets of tansceivers, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured. This is also valid for Case 4 UE, which share one antenna panel and one Rx beam but two sets of Rx chains between PCC and SCC. After the antenna receiving the signals from PCC and SCC, UE is able to process them separately.

Proposal 12: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Issue 1-4-8: Sharing factor between L1 measurement and RRM measurement

FFS

- Proposal 1: Reuse the legacy sharing factor Klayer1_measurement and Psharing factor between L1 and RRM measurements

For UE with the antenna array, in single carrier case, due to the Rx beam direction of neighbour cell and serving cell is different, UE could not perform neighbour cell L3 measurement and serving cell L1 measurement simultaneously, the sharing factor between L1 measurement and RRM measurement was introduced, the definition of Klayer1_measurement and P factor are as follows, NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example:

For UE with the antenna array, Klayer1_measurement=1, - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement; Klayer1_measurement=1.5, otherwise. For ATG UE with the antenna array, P value for SSB resource to be measured is defined as - Ntotal / Navailable with Navailable > 0 - Psharing factor * Ntotal / Noutside_MG with Navailable = 0 - For a window W of duration max(TL1, MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion: - Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W - TL1 is periodicity of the target SSB - Psharing factor = 3.

For intra-frequency CA, UE only have one antenna panel and one trancevier, UE could not perform neighbour cell L3 measurement and L1 measurement of all serving cells simultaneously, the measurement resource shall be shared between all serving cells L1 measurements and the L3 measurements.

For inter-frequency CA, except for Case 4, since UE have two separate antenna panels, Rx beams and tranceviers, UE could process the signals from different serving cells separately, which means when UE performs neighbour cell L3 measurement configured by one serving cell, the serving cell L1 measurement on the other serving cell can be performed simultaneously.

For inter-frequency CA UE antenna type Case 4 if introduced, since UE only have one Rx beam, UE could not perform neighbour cell L3 measurement and L1 measurement of all serving cells simultaneously, the measurement resource shall be shared between all serving cells L1 measurements and the L3 measurements.

Therefore, the Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example):

For UE with the antenna array,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions configured by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols configured by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

For ATG UE with the antenna array,

P value for SSB resource to be measured is defined as

-	Ntotal / Navailable with Navailable > 0

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, or not overlapped with any measurement gap occasion nor any SMTC occasion configured by same serving cell within the window W when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4].

-	TL1 is periodicity of the target SSB

-	Psharing factor = 3.

Proposal 13: The Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example):

For UE with the antenna array,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions configured by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols configured by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

                  - For ATG UE with the antenna array,

P value for SSB resource to be measured is defined as

-	Ntotal / Navailable with Navailable > 0

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, or not overlapped with any measurement gap occasion nor any SMTC occasion configured by same serving cell within the window W when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4].

-	TL1 is periodicity of the target SSB

-	   Psharing factor = 3.

Measurement enhancement for ATG UE with antenna array

As we know, for ATG UE with antenna array, it can’t receive the signal from serving cell and neighbour cells simultaneously due to different Rx beam direction. Therefore, the beam sweeping factor N1 and specific L1/L3 measurement sharing factor has been introduced from R18. Besides, additional scheduling restriction also been introduced for UE with the antenna array.

In R19 CA scenario, considering co-located deployment, large cell coverage, LOS propagation condition, which are the ATG specific scenario, we further consider RRC_connected measurement improvement methodology.

In following cases, we think the measurement for neighbour cells on the carrier frequency of SCC can be omitted:

- Case 1: Network could derive the coarse neighbour cell’s quality on the carrier frequency of SCC by neighbour cell’s quality on the carrier frequency of PCC.

- Case 2: Neighbour cells has not deployed the SCC which carrier frequency are same with the serving SCC.

- Case 3: UE is in the cell center area

After omitting the neighbour cells’ measurements on the carrier frequency of SCC, in inter-band CA scenario, for ATG UE with an antenna array on the bands of SCC (except case 4), UE only need to measure the serving SCell when performing the intra-frequency measurement. The beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed at all, the measurement delay requirement and scheduling restriction of omni-directional antenna UE can be reused. The measurement delay can be tightened and throughput can be increased without UE consuming additional power.

This kind of enhancement can be achieved by network configuration, for example, if network identify neighbour cell measurement on SCC is not needed by deployment or the measurement report from PCell, then network can configure a signal which indicate UE only measure the SCell when performing the intra-frequency measurement.

Observation 2: In following cases, the measurement for neighbour cells on the carrier frequency of SCC can be omitted:

- Case 1: Network could derive the coarse neighbour cell’s quality on the carrier frequency of SCC by neighbour cell’s quality on the carrier frequency of PCC.

- Case 2: Neighbour cells has not deployed the SCC which carrier frequency are same with the serving SCC.

- Case 3: UE is in the cell center area

Proposal 14: If network identify neighbour cell measurement on SCC is not needed by deployment or the measurement report from PCell, then network can configure a signal which indicate UE only measure the SCell when performing the intra-frequency measurement. From requirement point of view, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed.

Specification structure

According to the work plan and the progress, the CR skeleton should be identified and CR split should be done in this meeting.

After examine the agreements and ongoing open issues so far, we think following clauses will be impacted after introduce CA for ATG

- 3.6.16	Applicability of requirements for ATG

- 4.2D		Cell Re-selection for ATG

- 7.3D		Timing advance for ATG

- 7.6D(new clause)	Maximum Receive Timing Difference for ATG

- 8.1D		Radio Link Monitoring for ATG

- FFS 8.2D(new clause)	Interruption for ATG

- 8.3D		SCell Activation and Deactivation Delay for ATG

- 8.5D		Link Recovery Procedures for ATG

- 8.6D		Active BWP switch delay for ATG

- 8.19D	Pre-configured measurement gap activation/deactivation delay for ATG

- 9.1D		General measurement requirement for ATG

- 9.2D		NR intra-frequency measurements for ATG

- 9.3D		NR inter-frequency measurements for ATG

- 9.5D		L1-RSRP measurements for Reporting for ATG

- 9.8D		L1-SINR measurements for Reporting for ATG

- 9.10D	CSI-RS based L3 measurements for ATG

Proposal 15: Following clauses will be impacted after introduce CA for ATG

- 3.6.16	Applicability of requirements for ATG

- 4.2D		Cell Re-selection for ATG

- 7.3D		Timing advance for ATG

- 7.6D(new clause)	Maximum Receive Timing Difference for ATG

- 8.1D		Radio Link Monitoring for ATG

- FFS 8.2D(new clause)	Interruption for ATG

- 8.3D		SCell Activation and Deactivation Delay for ATG

- 8.5D		Link Recovery Procedures for ATG

- 8.6D		Active BWP switch delay for ATG

- 8.19D	Pre-configured measurement gap activation/deactivation delay for ATG

- 9.1D		General measurement requirement for ATG

- 9.2D		NR intra-frequency measurements for ATG

- 9.3D		NR inter-frequency measurements for ATG

- 9.5D		L1-RSRP measurements for Reporting for ATG

- 9.8D		L1-SINR measurements for Reporting for ATG

- 9.10D	CSI-RS based L3 measurements for ATG

- Conclusion

In this contribution, we further identify the potential RRM impact and further enhancement of ATG enhancement. The observation and proposals are:

Proposal 1: For the feasibility of Case 4, wait for more progress from RF session.

Proposal 2: If the implementation of Case 4 is not excluded in RF session, RRM requirement and related UE capability should be defined, details can be discussed later.

Proposal 3: Define MRTD requirement for ATG inter-band CA

- For ATG UE except antenna type Case 4, define the MRTD requirement as 33µs.

- For ATG UE antenna type Case 4 if introduced, further consider the MRTD requirement as 33µs.

Proposal 4: Introduce the interruption requirement of NR SRS antenna port switching.

Proposal 5: Reuse the current interruption requirements, no impact from UE antenna types.

Proposal 6: The inter-band SSB-less SCell activation requirements can be applied regardless of UE antenna type on the SCC.

Observation 1: In each components of SCell activation delay requirements, only the P value of ‘TL1-RSRP, measure’ will pending on the ATG UE antenna type on SCC, while this impact has already been identified and specified in R18.

Proposal 7: Refer the definition of ‘TL1-RSRP, measure’ to the ATG Clause 9.5D.

Proposal 8: For SCell activation delay requirements, no other impact caused by ATG UE antenna type.

Proposal 9: To save power and avoid throughput loss caused by measurement for SCell addition and/or activation, introduce distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition. After the condition is fulfilled at UE side, UE can add and/or active the SCell automatically, and then sent the SCell addition/activation confirmation message to network.

Proposal 10: Based on the legacy requirement:

- For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

- For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed

Proposal 11: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements:

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

- When inter-band carrier aggregation is performed:

  - For ATG UE antenna type Case 1, 2, 3 and 5, the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

  - For ATG UE antenna type Case 4, i.e., ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

Proposal 12: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Proposal 13: The Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example):

For UE with the antenna array,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions configured by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols configured by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

                  - For ATG UE with the antenna array,

P value for SSB resource to be measured is defined as

-	Ntotal / Navailable with Navailable > 0

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, or not overlapped with any measurement gap occasion nor any SMTC occasion configured by same serving cell within the window W when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4].

-	TL1 is periodicity of the target SSB

-	   Psharing factor = 3.

Observation 2: In following cases, the measurement for neighbour cells on the carrier frequency of SCC can be omitted:

- Case 1: Network could derive the coarse neighbour cell’s quality on the carrier frequency of SCC by neighbour cell’s quality on the carrier frequency of PCC.

- Case 2: Neighbour cells has not deployed the SCC which carrier frequency are same with the serving SCC.

- Case 3: UE is in the cell center area

Proposal 14: If network identify neighbour cell measurement on SCC is not needed by deployment or the measurement report from PCell, then network can configure a signal which indicate UE only measure the SCell when performing the intra-frequency measurement. From requirement point of view, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed.

Proposal 15: Following clauses will be impacted after introduce CA for ATG

- 3.6.16	Applicability of requirements for ATG

- 4.2D		Cell Re-selection for ATG

- 7.3D		Timing advance for ATG

- 7.6D(new clause)	Maximum Receive Timing Difference for ATG

- 8.1D		Radio Link Monitoring for ATG

- FFS 8.2D(new clause)	Interruption for ATG

- 8.3D		SCell Activation and Deactivation Delay for ATG

- 8.5D		Link Recovery Procedures for ATG

- 8.6D		Active BWP switch delay for ATG

- 8.19D	Pre-configured measurement gap activation/deactivation delay for ATG

- 9.1D		General measurement requirement for ATG

- 9.2D		NR intra-frequency measurements for ATG

- 9.3D		NR inter-frequency measurements for ATG

- 9.5D		L1-RSRP measurements for Reporting for ATG

- 9.8D		L1-SINR measurements for Reporting for ATG

- 9.10D	CSI-RS based L3 measurements for ATG

- Reference

- R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420338, WF on NR ATG UE RF, CMCC

- RP-243285, Revised WID on Enhancements for Air-to-ground network for NR, CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom