3GPP TSG-RAN WG4 Meeting #114     	R4-2500847

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.18.3

Source: 	CMCC

Title: 			 Simulation results for BS interference suppressing with MMSE-IRC

Document for:	Discussion

- Introduction

Based on the simulation assumption approved in last meeting[1], we give our simulation results and observations.

- Simulation results

PUSCH requirements with interference (CP-OFDM/FDD)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R (1 interference) | MCS 2 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | -1.89 | -0.13 |
|  |  |  | MMSE-MRC | -0.12 | 3.28 |
|  |  |  | Gain | 1.67 | 3.41 |
|  |  | TDLB100-400 | MMSE-IRC | -1.89 | 0.13 |
|  |  |  | MMSE-MRC | -0.12 | 3.35 |
|  |  |  | Gain | 1.67 | 3.22 |
|  | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 10.53 | 12.13 |
|  |  |  | MMSE-MRC | 12.33 | 15.75 |
|  |  |  | Gain | 1.8 | 3.62 |
|  |  | TDLC300-100 | MMSE-IRC | 11.32 | 12.55 |
|  |  |  | MMSE-MRC | 12.74 | 16 |
|  |  |  | Gain | 1.42 | 3.45 |
| 1T2R (2 interference) | MCS 2 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | -0.33 | 2.16 |
|  |  |  | MMSE-MRC | 1.83 | 6.16 |
|  |  |  | Gain | 2.16 | 4 |
|  |  | TDLB100-400 | MMSE-IRC | -0.33 | 1.66 |
|  |  |  | MMSE-MRC | 1.83 | 6.07 |
|  |  |  | Gain | 2.16 | 4.41 |
|  | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 12.18 | 14.62 |
|  |  |  | MMSE-MRC | 14.51 | 18.35 |
|  |  |  | Gain | 2.23 | 3.73 |
|  |  | TDLC300-100 | MMSE-IRC | 12.92 | 15 |
|  |  |  | MMSE-MRC | 14.6 | 18.38 |
|  |  |  | Gain | 1.68 | 3.38 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 7.06 | 8.13 |
|  |  |  | MMSE-MRC | 10.49 | 13.96 |
|  |  |  | Gain | 3.43 | 5.83 |
|  |  | TDLC300-100 | MMSE-IRC | 7.47 | 8.52 |
|  |  |  | MMSE-MRC | 10.6 | 14.17 |
|  |  |  | Gain | 3.13 | 5.65 |
| 1T8R | MCS 20 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 6.6 | 7.13 |
|  |  |  | MMSE-MRC | 9.87 | 13.48 |
|  |  |  | Gain | 3.27 | 6.35 |
|  |  | TDLA30-10 | MMSE-IRC | 6.62 | 7.13 |
|  |  |  | MMSE-MRC | 10.02 | 13.48 |
|  |  |  | Gain | 3.4 | 6.35 |
|  | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 3.67 | 4.27 |
|  |  |  | MMSE-MRC | 7.15 | 10.84 |
|  |  |  | Gain | 3.48 | 6.57 |
|  |  | TDLC300-100 | MMSE-IRC | 4.11 | 4.58 |
|  |  |  | MMSE-MRC | 7.28 | 11.04 |
|  |  |  | Gain | 3.17 | 6.46 |

PUSCH requirements with interference (DFT-s-OFDM/FDD)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R (1 interference) | MCS 2 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 0.45 | 2.73 |
|  |  |  | MMSE-MRC | 1.55 | 4.5 |
|  |  |  | Gain | 1.1 | 1.77 |
|  |  | TDLB100-400 | MMSE-IRC | 0.45 | 3 |
|  |  |  | MMSE-MRC | 1.55 | 4.82 |
|  |  |  | Gain | 1.1 | 1.82 |
|  | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 12.02 | 13.23 |
|  |  |  | MMSE-MRC | 13.26 | 16.23 |
|  |  |  | Gain | 1.24 | 3 |
|  |  | TDLC300-100 | MMSE-IRC | 12.84 | 14.7 |
|  |  |  | MMSE-MRC | 13.77 | 17 |
|  |  |  | Gain | 0.93 | 2.3 |
| 1T2R (2 interference) | MCS 2 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 2.17 | 5.33 |
|  |  |  | MMSE-MRC | 3.33 | 6.59 |
|  |  |  | Gain | 1.16 | 1.26 |
|  |  | TDLB100-400 | MMSE-IRC | 2.17 | 5.03 |
|  |  |  | MMSE-MRC | 3.33 | 6.51 |
|  |  |  | Gain | 1.16 | 1.48 |
|  | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 14 | 16 |
|  |  |  | MMSE-MRC | 15 | 19 |
|  |  |  | Gain | 1 | 3 |
|  |  | TDLC300-100 | MMSE-IRC | 14.72 | 17 |
|  |  |  | MMSE-MRC | 15.55 | 19.39 |
|  |  |  | Gain | 0.83 | 1.61 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 7.53 | 8.48 |
|  |  |  | MMSE-MRC | 10.78 | 14.5 |
|  |  |  | Gain | 3.25 | 6.02 |
|  |  | TDLC300-100 | MMSE-IRC | 8.24 | 9.21 |
|  |  |  | MMSE-MRC | 11.14 | 14.9 |
|  |  |  | Gain | 2.9 | 5.69 |
| 1T8R | MCS 20 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 6.63 | 7.06 |
|  |  |  | MMSE-MRC | 10.3 | 14.15 |
|  |  |  | Gain | 3.67 | 7.09 |
|  |  | TDLA30-10 | MMSE-IRC | 6.6 | 7.06 |
|  |  |  | MMSE-MRC | 10.27 | 14.15 |
|  |  |  | Gain | 3.67 | 7.09 |
|  | MCS 16 | TDLA30-10 for HetNet and TDLB100-400 for HomNet | MMSE-IRC | 3.85 | 4.34 |
|  |  |  | MMSE-MRC | 7.48 | 11.34 |
|  |  |  | Gain | 3.63 | 7 |
|  |  | TDLC300-100 | MMSE-IRC | 4.16 | 4.77 |
|  |  |  | MMSE-MRC | 7.76 | 11.57 |
|  |  |  | Gain | 3.6 | 6.8 |

- Simulation assumption

PUSCH requirements with interference

Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD, FDD |
| TDD pattern |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  | TDD: 30kHz/10MHz FDD: 15kHz/5MHz |
| Transmit precoding |  | Disabled |
| Antenna configuration |  | 1x2 (1/2 interferes) 1x4 (2 interferes) 1x8 (2 interferes) |
| Propagation condition |  | For serving cell: Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet Option 2: different channel for different MCS: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 For Interference cell: TDLC300-100 |
| MCS |  | 2R: MCS 2/MCS 16 4R: MCS 16 8R: MCS 20/MCS 16 |
| PUSCH configuration | Mapping type | Type A/B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | DM-RS sequence generation | nSCID =0 for all cells Different NID0 for serving and interference cells NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively For DFT-S-OFDM: group hopping and sequence hopping are disabled |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| Test metric |  | SNR@70% max TP |

| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 |
| --- | --- |
| INR values for Homogeneous deployment | INRs 5.35 dB in case of 1 interference cells INRs 7.28, -2.52 dB in case of 2 interference cells |
| INR values for Heterogeneous deployment | INRs 9.82 dB in case of 1 interference cells INRs 12.38, -0.97 dB in case of 2 interference cells |
| Modulation order of interfering PUSCH | 16QAM randomly modulated symbols |
| Interference cell modeling | Interfering cell has same channel model as for serving cell and it is independently generated for each cell. |

- Reference

[1] R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, Moderator (Nokia)