3GPP TSG-RAN WG4 Meeting #114			R4-2500853

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.26.4.2

Source: 	CMCC

Title: 			   Discussion on the RRM requirement for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 achieved some general agreements for objectives of DL coverage enhancement and support of regenerative payload. The WF has been approved in [1]. In this contribution, we further discuss the RRM impact and requirements on these two objectives.

- Downlink coverage enhancements

### Issue 1-1: SSB periodicity enhancement

Agreement (online):

For SSB periodicity up to 160ms:

- No further discussion on the impact on initial access from the perspective of RRM requirement

- Discuss whether to Modify the default values for SSB and SMTC periodicities for the following if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW

  - RRC Re-establishment: TSMTC,i assumption

  - RRC connection release with redirection to NR: Trs assumption

  - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption

  - Further discuss the details on how to update the requirement.

- Further discuss the following for SSB periodicity > 20ms:

  - Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles

  - Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios

  - RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB

  - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync

  - Further discuss whether and how to update the requirement

Agreement (online):

For SSB periodicity larger than 160ms if supported by other WGs:

- Whether RAN4 will define the RRM requirements for SSB periodicity larger than 160ms is depending on whether it is supported by WGs.

According to the latest approved WID RP-243300 as below, RAN4 only need to focus on SSB periodicity equal to or smaller than 160ms.

- Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

In existing RRM requirements, 20ms SSB periodicity is used as default for RRC Re-establishment and RRC connection release with redirection to NR, 5ms SSB periodicity is used as default for Handover, CHO, RACH-less HO and satellite switch with re-sync. It seems that there is no unified SSB periodicity default assumption among requirements.

- For RRC Re-establishment, if the periodicity of the SMTC occasion for the inter-frequency carrier i is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

- For RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, there is no requirement if the SSB transmission periodicity is larger than 20ms.

- For Handover, Conditional handover, RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

Observation 1: There is no unified SSB periodicity default assumption among requirements.

Based on observation 1, we think there are three feasible ways to define requirements when network applies DL-CE feature.

- Reuse the current default assumption, if the SSB transmission periodicity is larger than the default value, then no requirement applied.

- Revise the default SSB periodicity to 160ms when network applies DL-CE feature.

- Mandate the network to configure SMRC configuration or measurement object for the frequency to be measured.

- Proposal 1: Three feasible ways to define requirements about the default value when network applies DL-CE feature, i.e., SSB periodicity larger than 20ms

- Reuse the current default assumption, if the SSB transmission periodicity is larger than the default value, then no requirement applied.

- Revise the default SSB periodicity to 160ms when network applies DL-CE feature.

- Mandate the network to configure SMRC configuration or measurement object for the frequency to be measured.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

- Proposal 1: Three feasible ways to define requirements about the default value when network applies DL-CE feature, i.e., SSB periodicity larger than 20ms

- Reuse the current default assumption, if the SSB transmission periodicity is larger than the default value, then no requirement applied.

- Revise the default SSB periodicity to 160ms when network applies DL-CE feature.

- Mandate the network to configure SMRC configuration or measurement object for the frequency to be measured.

- Reference

- R4-2420105, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)