3GPP TSG-RAN WG4 Meeting # 114                                                                R4-2500858

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.12.2

Source: 	CMCC

Title: 	Discussion on BS RF requirements for ATG with DL MIMO

Document for:	Discussion

# 1. Introduction

RAN#106 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. During the past meeting we focused on the issue of the TAE requirements, and the revised WID has approved the discussion of the TAE requirements for ATG with DL MIMO. In this paper, we will focus on this issue.

# 2. Discussion

## 2.1. ATG BS capable of multi-band operation

During RAN#106 meeting, the WID of ATG has been revised as below.

The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify the TAE requirements for support of DL MIMO for BS. [RAN4]. - Specify signaling and UE capabilities if necessary [RAN2].

In Rel-18, considering ATG did not support CA, there is no TAE requirement for ATG as below.

#### 6.5.3.1 General This requirement shall apply to frame timing in MIMO transmission, carrier aggregation and their combinations. Frames of the NR signals present at the BS transmitter antenna connectors or TAB connectors are not perfectly aligned in time. The RF signals present at the BS transmitter antenna connectors or transceiver array boundary may experience certain timing differences in relation to each other. The TAE is specified for a specific set of signals/transmitter configuration/transmission mode. For BS type 1-C, the TAE is defined as the largest timing difference between any two signals belonging to different antenna connectors for a specific set of signals/transmitter configuration/transmission mode. For BS type 1-H, the TAE is defined as the largest timing difference between any two signals belonging to TAB connectors belonging to different transmitter groups at the transceiver array boundary, where transmitter groups are associated with the TAB connectors in the transceiver unit array corresponding to MIMO transmission, carrier aggregation for a specific set of signals/transmitter configuration/transmission mode. There is no TAE requirement for ATG BS.

In our opinion, removing the limitation for TAE is a requirement for ATG to support CA, since DL MIMO for ATG is also supported in the revised WID.

Proposal 1: Remove the limitation for TAE, and there is no need to add other wording to specify ATG TAE requirements.

# 3. Conclusions

In this contribution, ATG BS requirements are discussed with the following proposals:

Proposal 1: Remove the limitation for TAE, and there is no need to add other wording to specify ATG TAE requirements.

# 4. References

[1] RP-242385	Revised WID on Enhancements for Air-to-ground network for NR