3GPP TSG-RAN WG4 Meeting #114				                      R4-2500877

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.24.2.1

Source: 	CMCC

Title: 	Discussion on A-IoT BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements for Type 1-C Ambient-IOT BS.

- Discussion

2.1 A-IoT BS Tx requirement

The conclusion on BS Tx requirements in study item are captured in following table

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES |
| Output power dynamic | RE power control dynamic range | TBD in WI phase |
|  | Total power dynamic range | TBD in WI phase |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | TBD in WI phase |
|  | Transmitter transient period | TBD in WI phase |
| Transmit signal quality | Frequency error | YES |
|  | Some requirements like EVM (Modulation quality) | YES |
|  | TAE | NA |
| Unwanted emissions | Occupied bandwidth | YES |
|  | ACLR | YES |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES |
|  | Transmitter spurious emissions | YES |
| Transmitter intermodulation |  | TBD in WI phase |

- Output power

In SI phase, micro BS is considered and used for co-existence evaluation. Hence, use NR Micro BS type 1-C as the starting point for Rel-19 ambient IOT BS.

The rated carrier output power of the BS type 1-C shall be as specified in table 6.2.1-1.

Table 6.2.1-1: BS type 1-C rated output power limits for BS classes

| BS class | Prated,c,AC |
| --- | --- |
| Medium Range BS | ≤ 38 dBm |

Proposal 1-1: For reader output power, use NR Micro BS type 1-C as baseline.

- Power related requirements

For power dynamic range related requirement,

- RE power control dynamic range: this is related to modulation schemed, according to WID, R2D only supports OOK-4 modulation, RE power control dynamic range should also be updated accordingly.

- Total power dynamic range: this is related to SCS and max transmission bandwidth configuration. Can be updated based on the agreed max transmission bandwidth of R2D.

Proposal 1-2: RE power control dynamic range and total power dynamic range requirements need to be updated according to the modulation scheme and bandwidths.

- Tx signal quality related requirement

BS spec define frequency error and signal quality related requirements.

For frequency error, UE use absolute frequency ARFCN to identify frequency so frequency error requirement is signicifent and RAN4 define strict level, i.e. 0.05/0.1ppm. For RFID, to enhance spectrum efficiency and reduce interference to other systems, there is also frequency accuracy requirement of reader for multiple and dense interrogator deployment scenario, which is almost in the range from 10ppm to 20ppm. For A-IoT, considering devices will use energy/edge detection to detect reader signal, it seems their is no need for very strict frequency error requirement if assume all A-IoT reader using same spectrum.

Proposal 1-3: current gNB frequency error can be reused.

For signal quality related requirements, gNB spec define EVM requirement which is used to evaluate phase distance. A-IOT reader only supports OOK-4 modulation. So current gNB quality related requirement is not applicable anymore. RFID[1] use three types ASK modulation types, i.e. DSB-ASK, SSB-ASK or PR-ASK and define RF envelop requirement including modulation depth, RF envelop ripple, RF pluswidth etc which can be referred for OOK modulation. Details illustration of RFID RF envelop requirement is listed as below for information.

Proposal 1-4: RAN4 could refer to RFID RF envelop related parameters to define signal transmission quality requirement as starting point, i.e. modulation depth, RF envelop ripple, RF plusewidth.

- Tx time related requirement

RFID also define Tx time related requirement, e.g. rise time, settling time, fall time.

- Rise time: the time periodicity of power-up envelop from 10% point rising monotonically to 90% point.

- Fall time: the time periodicity of power-down envelop from 90% point falling monotonically to 10% point.

- Settling time: the time periodicity of power-up envelop rise from 90% point until at least the ripple limit MI. the 90% point is rising monotonically from 10% point. During the whole settling time, envelop should not fall below the 90% max filed strength.

BS spec defines transient period which is much like rise time and fall time requirements as in RFID spec. Besides, RFID also defines settling time to evaluate signal ripple characteristics. reader support OOK signal which is much like the ASK modulated signal as in RFID spec. It’s suggested to further check whether the settling time as in RFID spec is needed or not.

Proposal 1-5: it’s suggested to define transient period related requirements for A-IoT reader. Details value can refer to RFID rise/fall time, and further consider whether settling time as defined in RFID spec is needed or not to evaluate RF envelop ripple characteristics.

- Unwanted emission requirements

For unwanted emission requirements, legacy NB-IOT requirements are used for co-existence evaluation in SI. The conclusion of evaluation shows that legacy requirements can fulfil the co-existence when NR macro BS is outdoor and A-IOT reader is indoor. One scenario that causes severe interference is when legacy NR UE is indoor, which require some existing scheduling mechanism to avoid adjacent interference or enhanced mechanism to mitigate interference (as captured in TR 38.769 clause 6.6.5.10). Consider enhanced mechanism to mitigate interference is not within Rel-19 WI scope, then legacy NB-IOT requirements can be reused for A-IOT reader.

Proposal 1-6: legacy NB-IOT ACLR, unwanted emission, spurious emission can be reused for A-IOT reader.

2.2 A-IoT BS Rx requirement

The conclusion on BS Rx requirements in study item are captured in following table

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES |
| Dynamic range |  | TBD in WI phase |
| Adjacent Channel Selectivity |  | YES |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES |
| Receiver intermodulation | General intermodulation | Wait for RAN1 conclusion. |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | TBD in WI phase |

- Reference Sensitivity

In TR 38.769, the reference sensitivity is defined as following:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

-	BW is the transmission bandwidth for D2R.

-	NF is the noise figure of A-IoT BS, equal to 10 dB for Medium Range BS.

-	IM is the implementation margin, equal to 2dB.

-	SNR is the SNR value for D2R decoding, the detailed metrics would be discussed in the WI phase.

-	Desens target is considered for CW signal interference cancellation in D1T1 scenario. The desens level for CW signal cancellation might be different for D1T1-A1/D1T1-A2 and D1T1-B. For D1T1-A2, CW signal interference is coming from self-interference and also potential CW interference from other nodes transmitting CW signal. For D1T1-A1, the CW signal interference is coming from other nodes transmitting CW inside the topology. For D1T1-B, the CW signal interference is coming from CW nodes transmitting outside topology.

For target SNR, 10% BLER can be used as the target for evaluation. RAN4 should discuss and agree on the simulation assumption to derive the target SNR.

For desens target, there is no need to consider desens target in reference sensitivity.

Proposal 2-1: Reference sensitivity is defined as:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR

- NF is 10dB (same as medium range BS)

- IM is implementation margin equal to 2dB

- SNR targets for 10% BLER for PRDCH, RAN4 will discuss the simulation assumption to derive target SNR

- ACS and blocking related requirements

Similar as ACLR, according to co-existence evaluation in SI phase, legacy NB-IOT requirements can be reused.

Proposal 2-2: For ACS and blocking related requirements, reuse legacy NB-IOT requirements.

# 3. Conclusions

In this contribution, RF requirements for gNB like reader are discussed with following observations and proposals.

Proposal 1-1: For reader output power, use NR Micro BS type 1-C as baseline.

Proposal 1-2: RE power control dynamic range and total power dynamic range requirements need to be updated according to the modulation scheme and bandwidths.

Proposal 1-3: current gNB frequency error can be reused.

Proposal 1-4: RAN4 could refer to RFID RF envelop related parameters to define signal transmission quality requirement as starting point, i.e. modulation depth, RF envelop ripple, RF plusewidth.

Proposal 1-5: it’s suggested to define transient period related requirements for A-IoT reader. Details value can refer to RFID rise/fall time, and further consider whether settling time as defined in RFID spec is needed or not to evaluate RF envelop ripple characteristics.

Proposal 1-6: legacy NB-IOT ACLR, unwanted emission, spurious emission can be reused for A-IOT reader.

Proposal 2-1: Reference sensitivity is defined as:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR

- NF is 10dB (same as medium range BS)

- IM is implementation margin equal to 2dB

- SNR targets for 10% BLER for PRDCH, RAN4 will discuss the simulation assumption to derive target SNR

Proposal 2-2: For ACS and blocking related requirements, reuse legacy NB-IOT requirements.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified