3GPP TSG-RAN WG4 Meeting # 114	                                                         R4-2500887

Athens, GR, Feb. 17 – Feb. 21, 2025

Agenda Item:      7.27.2.3

Source:	MediaTek Inc.

Title:	Discussion on conditional LTM

Document for:	Discussion

# Introduction

In this contribution, we provide our views on conditional LTM.

# Discussion

## 2.1 Conditional cell switch delay and interruption

Before discussing conditional LTM, we would like to recall the procedure and requirements of conditional handover at first. As shown in Fig. 1, CHO starts from NW configures the RRC configuration of target cell and the conditions for conditional HO. After the condition is met, UE will start to handover to the target cell.

As far as we know, there are two drawbacks of CHO. One is that target cells should start to reserve the prepared conditional HO resources and context for the UE after the configuration, as target cell does not know when the condition will be met. Another is, source cell does not know when UE will start to handover to the target cell too, so during the HO procedure, source cell may keep scheduling the UE. These two all lead to some waste of NW resources or efforts.

Fig.1

In our view, when discussing conditional LTM, we should try to solve the drawbacks of CHO. For the first one, we don’t think it is RAN4’s work. For the latter, we believe RAN4 can do some optimization. Now the interruption time includes Tprocessing, time for T/F tracking and UL sync. In LTM, early T/F tracking is introduced to reduce both the cell switch delay and interruption. In our understanding, if UE supports early T/F tracking, it is also possible to perform T/F tracking before Tprocessing. As we all know, the time introduced by T/F tracking is mainly spent on waiting for the SSB occasion. The waiting time also contributes to the interruption. If UE performs T/F tracking before Tprocessing, the delay is not reduced, but the interruption will be reduced.

Proposal 1: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM.

## 2.2 UE based TA measurement

Last meeting, RAN4 discussed whether to define requirements for UE based TA measurement. The mechanism is to derive TA of neighbor cell based on TA of serving cell and RTD between serving cell and neighbor cell as shown in Fig.2. Considering that serving cell and neighbor cell are not ideally synchronized in the field, there will be some synchronization error (denoted as y). Then TA2=TA1+2*RTD = TA1+2*(y+TP2-TP1).

Fig. 2

The errors, for each component should be considered, i.e.,

- TA1

Consider the errors: UL to DL error + TA resolution error + TA adjustment accuracy

- TP2:

Consider DL timing error

- TP1:

Consider DL timing error

The total error is UL to DL error + TA resolution error + TA adjustment accuracy + 2*(y+2* DL timing error) = UL to DL error +DL timing error + TA resolution error + TA adjustment accuracy +2y +3* DL timing error = Te + TA resolution error + TA adjustment accuracy +2y +3* DL timing error. To make sure the degradation at gNB reception is acceptable, the total error should be less than 1/2 CP, which is used as a criterion to define UE based TA measurement accuracy requirements in FR2 HST. Then it can be derived the synchronous requirement on serving cell and neighbor cell under different numerologies as below:

| SCS of SSB (kHz) | SCS of UL signals (kHz) | 1/2 CP of UL signals (64*Tc) | Te | TA resolution error 16*64*Tc/2u | TA adjustment accuracy 38.133 Table 7.3.2.2-1 | DL timing error 1/(2*BW) | y (ns) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 15 | 71.05 | 12*64*Tc | 16*64*Tc | 256 Tc | 139ns | 427.1 |
| 30 | 30 | 35.55 | 8*64*Tc | 8*64*Tc | 256 Tc | 69.4ns | 149.0 |
| 120 | 120 | 8.9 | 3.5*64*Tc | 2*64*Tc | 32 Tc | 17.4ns | 21.1 |

After calculation, it can be observed that only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

Observation 1: Only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

As the condition to use UE based TA measurement is too tight for almost all the cases, we propose not to define the related requirements.

Proposal 2: Not to define the requirements for UE based TA measurement in RAN4 in R19.

In addition, we don’t think the accuracy requirements defined for one shot large UL timing adjustment for FR2 HST can be applied directly, where at least two shots of T/F tracking or TRS based T/F tracking on the target TRP is needed.

# Conclusion

In this contribution, we provide our views for conditional LTM. A proposal is proposed as follows.

Proposal 1: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM.

Observation 1: Only when the synchronization error between serving cell and target cell is less than 427.1ns for 15kHz SCS, 149ns for 30kHz SCS, and 21.1ns for 120kHz SCS, it is possible that UL signal on target cell after adjusting through UE based TA can be within 1/2 CP at gNB.

Proposal 2: Not to define the requirements for UE based TA measurement in RAN4 in R19.