3GPP TSG-RAN WG4 Meeting#114		R4-2500897

Athens, GR, Nov 18th -22nd, 2025

Agenda item:	7.1.1.3.4

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on SRS IL imbalance for 6RX UE

Document for:	Discussion

Introduction

In RAN4#113 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

General considerations for SRS IL imbalance issue

- Sub-topic 3-1: General considerations for SRS IL imbalance issue Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on the compromised solution proposals in Issue 3-2-1.

According to TS 38.213, the transmit power of SRS is described as follows. If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasions i as

This formula takes SRS-ResourceSet as granularity, it means that the SRS power of all ports is the same or it is expected that the UE can achieve that no matter what way.

According to TS 38.101, with regard to different ports pass through different devices and PCB loss (SRS insertion loss), TS 38.101 specifies that UE is allowed to The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot with ∆TRxSRS. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

Which means that among different ports of SRS power deviation is allowed. However, according to practical experience, if SRS power is unbalanced among SRS ports, it will affect the channel estimation, which will lead to the deterioration of DL performance. Thus, in order to meet the SRS power balance of all ports and achieve better performance, UE actually made power compensation. However, when the power deviation exceeds a certain value (e.g., 5dB) due to the limited power of PA devices, it cannot be completely compensated. This leads to the final failure to achieve SRS port power balance. In our understanding, UE’s self-compensation is up to UE implementation, but when UE’s self-compensation cannot meet the power balance, it needs to rely on the compensation of gNB. In our understanding, reporting SRS insertion loss is necessary to continue to pursue a solution to this issue.

Observation 1: When the power deviation exceeds a certain value (e.g., 5dB) due to the limited power of the PA device, it cannot be completely compensated by the UE.

Proposal 1: Whether the UE perform power imbalance self-compensation depends on UE implementation.

Proposal 2: Support option2. Continue to pursue a solution to the SRS IL imbalance issue.

SRS IL imbalance issue solutions

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies are encouraged to provide comments and indication of support for the solution proposals listed below by RAN4#113. Proponent companies (in parentheses) for the general solution and solution proposals below are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Other proposals including expected performance gains and draftCRs are not precluded. - General candidate solutions - Option 1 (Intel): - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - Details of SRS IL self-compensation and assistance framework are FFS - Other options are not precluded

This issue has been discussed for many RAN4 meetings, and there has been no further progress. Many companies have provided simulation results and proved that the gain to report SRS insertion loss imbalance across SRS ports for 8RX in Rel-18, in our understanding, it is necessary to perform SRS IL reporting to NW because the UE cannot always satisfy the balance between the main branch and the diversity branch SRS port by UE self-compensation. In addition, according to the way forward, it is necessary to influence RAN1/2 and RAN4 as little as possible.

When UE supports to report semi-statically, UE just needs to report the actual SRS insertion loss with no UE self-compensation, in additional, in order to reduce the reporting frequency, UE needs to set a threshold. When the configured SRS power is higher than the threshold power, UE trigger NW to compensate according to the actual SRS insertion loss.

When the UE supports to report dynamically, there is no need to distinguish the behaviour whether performs self-compensation. The UE obtains the output power of each SRS port (e.g. by PD detection) and reports the output power to NW, then NW calculates the difference value (main branch output power and each diversity branch output power) or UE calculates the difference value of each diversity branch output power then report to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time when the configured SRS power is higher than the threshold power.

On the whole, reporting dynamically with periodicity and will needs more bits compared to static reporting. In our understanding, static reporting has less impact on RAN1 and RAN2 compared to dynamic reporting.

Regardless of the reporting method, we should consider the granularity of reporting. In our understanding, the granularity of reporting could be per SRS resource or per band.

Observation 2: Static reporting has less impact on RAN1 and RAN2 compared to dynamic reporting.

Proposal 3: If UE reports statically, report the actual SRS insertion loss with no UE self-compensation.

Proposal 4: If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time.

Proposal 5: Reporting power threshold needs to be considered for static reporting and dynamic reporting. .

Proposal 6: The granularity of reporting could be per SRS resource or per band.

Conclusions

In this paper, we provide our views on SRS IL imbalance, we have made the following observation and proposals.

Observation 1: When the power deviation exceeds a certain value (e.g., 5dB) due to the limited power of the PA device, it cannot be completely compensated by the UE.

Proposal 1: Whether the UE perform power imbalance self-compensation depends on UE implementation.

Proposal 2: Support option2. Continue to pursue a solution to the SRS IL imbalance issue.

Observation 2: Static reporting has less impact on RAN1 and RAN2 compared to dynamic reporting.

Proposal 3: If UE reports statically, report the actual SRS insertion loss with no UE self-compensation.

Proposal 4: If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time.

Proposal 5: Reporting power threshold needs to be considered for static reporting and dynamic reporting. .

Proposal 6: The granularity of reporting could be per SRS resource or per band.

- Reference

- R4-2420376 on requirements for 6Rx, AT&T,TSG-RAN4#113 meeting