3GPP TSG-RAN WG4 Meeting #114	R4-2500905

# Athens, GR, 17th – 21st February, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Rx beam factor optimization of R19 RRM enhancements

Agenda Item:	7.17.2.1

Document for:	Discussion

- Introduction

During last RAN4 meeting, around the multi-Rx based Rx beam sweeping factor optimization, the following progress were achieved and summarized in [1] as below:

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19. Issue 1-1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF Agreement: - Directly discuss the SSB processing time. Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX Agreement: - L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case. Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement: - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability: - FFS: details of the capability Issue 1-3: feature capability of L3 measurement delay reduction by optimizing Rx BSF Agreement: - Discuss this capability detail by end of the core part

In this paper, we provide our further views on the reduced Rx beam sweeping factor on L3 measurements.

- Discussion

In last meeting, companies have good discussion on the conditions of activation/deactivation of the fast L3 measurement, some progress was achieved but still some details need to determine. Besides this issue, some other issues are still suspending. We provide our further analysis on all these issues.

- Conditions to activate/deactivated L3 measurement delay reduction by optimizing Rx BSF

As concluded in 112bis meeting, the solution is the combination of NW signalling and condition triggering. Further in 113 meeting, the general principle was determined as only considering serving cell absolute channel quality, such as RSRP, RSRQ. By comparing the actual serving cell absolute channel quality with a threshold, to determine whether to enable or disable the fast L3 measurement. So the whole procedure is CHO alike. NW configures the serving cell channel quality threshold, UE starts to monitor the serving cell quality as long as receiving the NW configuration.Once the actual serving cell quality meets the threshold, UE activates the fast L3 measurement by optimizing Rx BFD. Until the current serving cell quality does not meet the threshold, UE deactivates such fast L3 measurement and fallback to legacy L3 measurement. The remaining issue is to determine the exact threshold, similar as the cases of legacy not-at-cell edge criteria in IDLE/INACTIVE and good serving cell quality criteria in CONNECTED, NW configures a RSRP threshold and/or RSRQ threshold in RRC signaling, the exact value is decided by NW.

Proposal 1: NW configuration of the threshold RSRP and/or RSRQ triggers the UE monitoring, as long as receiving the threshold, UE starts to monitor the serving cell quality to determine when to activate/deactivate the fast L3 measurement. If not any threshold configured to UE, which implies NW does not expect the UE to perform fast L3 measurement.

In last meeting, besides the above general procedure of activation/deactivation of fast L3 measurement, some companies raised the discussion on some additional quit mechanism from fast L3 measurement to legacy L3 measurement besides the serving cell quality monitoring. The main intention is to cover the case of low mobility.

Actually in both IDLE/INACTIVE and CONNECTED status, the low mobility criteria is defined to relax the intra/inter-f measurement or RLM/BFD measurement to facilitate the UE power saving. The metric of low mobility is the rate of RSRP change, i.e. comparing the delta RSRP with a threshold. If the delta RSRP is smaller than the threshold, then the low mobility is fulfilled, the relaxed measurement requirements is applied.

While during last meeting discussion, the idea of taking the legacy criteria of low mobility into consideration was discussed and excluded as highlighted below:

Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

Based on the above progress, we do not believe the consideration of low mobility should be involved in as additional quit condition.

Proposal 2: The consideration of UE mobility status based triggering condition has been excluded in last meeting, so neither support to take this as additional quit condition.

In addition, if the UE can not support the fast L3 measurement due to the power consumption issue, the legacy UAI mechanism such as overheating or preference of single panel could be used to inform NW, then NW can decide whether to configure any new RSRP/RSRQ threshold to reduce the chance of entering fast L3 measurement.

- UE capability of L3 measurement delay reduction by optimizing Rx BSF

During last meeting, it is determined that the UE capability of Rx BFD is needed, but the exact UE capability is FFS. In current spec, the Rx beam sweeping factor is melt into the whole scaling factor of M for the measurement requirements of FR2. So regarding how to reduce the Rx BSF, two solutions were proposed in last meeting:

Alt 1: UE reports the reduced N, such as N = 2, 4, 6

Alt 2: UE reports the reduced M, and M = sample*reduced Rx BFD, such as M = 12, 14, 16, 20

To our understanding, no matter defining reduced N or reduced M, there is no big difference, but considering this enhanced UE capability lies in the fast beam sweeping, it should has no impact on the sample number. Starting from this, the reporting of reduced N is preferred by us, which implies smaller Rx beam sweeping factor is assumed, no impact on the legacy assumption of sample. Considering in current FR2 measurement requirements, instead of N, the whole scaling factor M is captured in, so NW and UE should determine the corresponding reduced M based on the reduced N.

Proposal 3: Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6.

Another relevant issue is how to capture the measurement requirements impacted by the reduced N in spec. To align with our above proposal on the UE capability, in terms of how to capture the delay requirements, just replacing the original assumption of N = 8 with the reported UE capability is the most straightforward solution. For Scenario 1/2/3/4/5/7, update the the highlighted factor in the following table suggested in [3] could be as the baseline.

| Scenarios | Factors |
| --- | --- |
| 1 | Mass/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 2 | Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 3 | Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3 |
| 4 | Same as scenario 3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1. |
| 7 | RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. |

Proposal 4: For the delay requirements of Scenario 1/2/3/4/5/7, replace the original beam sweeping factor with the reduced value reported by UE capability.

Considering the transition between the fast L3 measurement mode and normal L3 measurement mode, our original view is to define the transition requirement since the transition between fast L3 measurement and normal L3 measurement have independent requirements, so the transition is a kind of similar as the transition between w gap/wo gap or w DRX/wo DRX in legacy. But as some companies raised, in R18 multi-Rx based fast beam sweeping, finally no transition requirement was defined due to the associated complexity to define the start and end of the transition. To our understanding, for the case here, actually the start and end of the transition is clear since RAN4 has defined the CHO alike mechanism, i.e. UE starts the fast L3 measurement mode when the serving cell quality condition meets, and UE ends the fast L3 measurement mode as long as the serving cell quality condition does not meet. So we can not see too much obstacle to define the transition requirements.

If no transition requirement defined, when the transition happens during the middle of one fast or normal measurement period, the UE behavior is uncertain. If the UE drops the already measured samples, it is harmful to the whole measurement period. So we prefer to define the transition requirement and follow the legacy L3 measurement transition tradition, i.e. use the longer delay between the two modes.

Proposal 5: Define the transition requirement and follow the traditional L3 measurement transition requirement, i.e. use the longer delay between the two modes.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Proposal 1: NW configuration of the threshold RSRP and/or RSRQ triggers the UE monitoring, as long as receiving the threshold, UE starts to monitor the serving cell quality to determine when to activate/deactivate the fast L3 measurement. If not any threshold configured to UE, which implies NW does not expect the UE to perform fast L3 measurement.

Proposal 2: The consideration of UE mobility status based triggering condition has been excluded in last meeting, so neither support to take this as additional quit condition.

Proposal 3: Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6.

Proposal 4: For the delay requirements of Scenario 1/2/3/4/5/7, replace the original beam sweeping factor with the reduced value reported by UE capability.

Proposal 5: Define the transition requirement and follow the traditional L3 measurement transition requirement, i.e. use the longer delay between the two modes.

- Reference

- R4-2420093 “WF for NR_RRM_Ph5_Part1”, Apple

- R4-2418294 “Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor”, NTT DOCOMO, INC