3GPP TSG-RAN WG4 Meeting #114	R4-2500909

Athens, GR, 17th – 21st February, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM aspects of UE-initiated beam management of R19 NR MIMO Phase 5

Agenda Item:	7.22.3.1

Document for:	Discussion

- Introduction

Around the objective of UE-initiated/event-driven beam management, during last RAN4 meeting, the following agreements were achieved in last meeting and summarized in [1], we list them as below:

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a first PUCCH triggered by the event. Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? < Agreement> Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion. - Start the RAN4 RRM discussion for FR2. - Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS. Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2? Case 1: no time window based Case 2: with time window based, M is configurable < Agreement> Agreement: - It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2.

In this paper, we provide our detailed analysis around the UE-initiated/event-driven beam management.

- Discussion

Ever since Rel-15, NR has supported intra-cell beam management. During Rel-17, inter-cell beam management(ICBM) was studied and specified. So actually until now, both intra-cell and inter-cell beam management are allowed. While all the legacy intra-cell and inter-cell beam management are NW-initiated. In terms of the beam management resource configuration, periodic, semi-persistent and aperiodic measurement resource are candidates.With respect to the reporting, similarly, periodic, semi-persistent and aperiodic reporting are allowed with some principle defined between the mapping of resource and reporting. All types of reporting are NW-initiated, in other words, only NW could trigger the beam measurement results reporting. Consequently, even the UE detects no need to update the current beam based on the measurement results, once the reporting is configured or activated or indicated by NW, the UE has to report the quantities configured in the corresponding report configuration.

Actually the factors which would lead to beam switching including UE movement and rotation, all these factors are not controlled by NW, so only UE is aware of these. So no matter for the sake of beam management latency reduction or to save the reporting overhead, it seems the UE-initiated beam management is more efficient and effective.

2.1 Requirements impact

The requirements of evaluation period/measurement period

In previous RAN4 meetings, warm discussion around the measurement reporting delay was held and finally it was approved as the duration between the starting point and ending point. The starting point is same as that in legacy event triggered L3 reporting, the ending point is defined as the point when UE transmits first PUCCH to request UL resource. Actually both the starting point and ending point follow same principle used in legacy event triggered L3 reporting. Such measurement reporting delay aims to verify whether UE could detect or evaluate the change of the air within a certain duration. So we need to define the upper bound of this certain duration as we did in legacy event triggered L3 reporting, this can be defined as the evaluation period.

In RAN1 latest meeting RAN1#119, besides Event-2, RAN1 has approved Event-1 and Event-7 as below:

Within Event-1, only current beam is involved in. While within Event-7, it seems UE has to monitor all beams associated to the whole active TCI state list. So the evaluation period for the three events are different. So RAN4 needs to define the evaluation period for the three events separately.

Proposal 1: RAN4 needs to determine the evaluation period for Event-1, Event-2 and Event-7 respectively. In details,

- For Event-1, only current beam should be monitored;

- For Event-2, both current beam and new beam should be monitored;

- For Event-7, besides new beam, all beams associated with the active TCI state list should be monitored.

Regarding how to define the evaluation period, RAN1’s progress on the trigger-event detection should be set as the baseline.

For Event 2, RAN1 has defined two types of trigger-event detection for reporting as below in [2][3][4]:

- Case 1: Basic solution(mandatory), once the L1-RSRP of the new beam becomes a threshold value better than the current beam(i.e. Event-2), UE initiated beam report occurs.

- Case 2: Optional solution(subject to UE capability), f within a time window, the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

- Furthermore, it should be noted that regarding the periodicity of RS, RAN1 achieved some progress and inform RAN4 in the LS[5]. The same periodicity between current beam and new beam at least for same CC case is determined, but the alternative of different periodicity is still on the table. From RAN4 perspective, whether the current beam and new beam are in the same CC or not, which is transparent for the determination of measurement requirements.

Proposal 2: Based on RAN1 progress on Event-2 until now, both same periodicity and different periodicity cases should be considered to determine the evaluation period.

- For the basic solution, which is one-shot measurement based reporting, so the evaluation period is determined by the maximum of TRS1 and TRS2, and the sample number M is set to 1.

- For the optional solution, besides TRS1 and TRS2, the sample number M is configured by NW signalling, and a timer X should also be considered.

- Other scaling factor such as collusion with other measurement or Rx beam sweeping in FR2 should also be considered.

Proposal 3: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. The maximum of current beam periodicity and new beam periodicity should be considered for both two solutions.

- For the basic Case 1, which is one-shot measurement based reporting, the sample number is 1.

- For the optional Case 2, the sample number M and timer X should be considered.

- For Event-1, since only current beam involved in, so the evaluation period could be derived from simplifying the evaluation period for Event-2, that is, ignoring the periodicity of new beam in the evaluation period for Event-2.

Proposal 4: For Event-1, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period, only the periodicity of current beam is involved in.

- For the basic Case 1, which is one-shot measurement based reporting, the sample number is 1.

- For the optional Case 2, the sample number M and timer X should be considered.

- For Event-7, the basic Case 1 is one-shot measurement based reporting, since UE needs to choose the Q-th best TCI state from the active TCI state list and set this Q-th best TCI state as reference to determine the new beam, so actually the periodicity of all beam within the active TCI state list and the periodicity of new beam are involved in the event detection. For the optional Case 2, we believe some additional clarification from RAN1 is needed, i.e. whether the Q-th best TCI state should be consistent or not during the event detection. If the Q-th best TCI state should be consistent, then as long as the Q-th best TCI state is determined at the 1st shot, then UE only needs to monitor this Q-th best beam and the new beam until the counter condition and timer condition meet, no need to monitor the beam of other TCI states. But if the Q-th best TCI state could be different beam at each measurement shot, then UE always need to monitor all beams of the active TCI state list at each measurement shot and pick the current Q-th beam.

Proposal 5: For the basic Case 1 of Event-7, the maximum between the new beam periodicity and the periodicity of all beams associated to the active TCI state list should be considered.

Proposal 6: For the optional Case 2 of Event-7, some further clarification in RAN1 is needed in terms of whether the Q-th best TCI state should be consistent or not during the event detection. RAN4 could wait for more RAN1 input or send LS to trigger the clarification.

Besides the measurement reporting delay, whether and how to define measurement period was also raised in last meeting. To our understanding, since the UEIBM is aperiodic reporting, it is hard to directly reuse the legacy L1-RSRP measurement period. We should take note of that even periodic/semi-persistent/aperiodic reporting are all allowed for the legacy L1-RSRP measurement, but actually the measurement period defined by RAN4 only covers the periodic/semi-persistent reporting, not touch aperiodic reporting. We take FR1 as example.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

It was suggested to remove Treport or set is as 0 to derive the measurement period for aperiodic reporting. But a prior question should be answered: If this way is feasible, why not to follow this way to define the measurement period for aperiodic reporting in legacy L1-RSRP measurement in R15? Furthermore, the trigger-event detection would determine when the UE would prepare the L1 reporting based on one-shot or multi-shot measurements. So to our understanding, compared with defining the requirement of measurement period, it is more straightforward to define the requirement of evaluation period. Or the evaluation period can be called as measurement period.

Proposal 7: For event triggered UEIBM, the evaluation period is equivalent to the measurement period.

The requirements of TCI state switching

RAN1 held some discussion on the reduction of TCI state switching latency after UEIBR in latest RAN1#119 meeting and the following progress were achieved:

Even no final conclusion between the 3 alternatives, it can be seen that the common point of all alternatives lies in: activate new beam(s) without additional SSB reception. From RAN4 perspective, which implies the target TCI state is of course known and maintained by UE, so the components of TL1-RSRP and Tfirst-SSB+TSSB-proc in legacy TCI state switching procedure could be ignored after confirmation/acknowledgement from NW or after receiving a TCI state activation command or after sending a UE-initiated beam report.

Proposal 8: Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

2.2 CSI-RS based measurement

In last meeting, it is approved to define CSI-RS based UEIBM L1-RSRP measurements at least for periodic CSI-RS. This aligns with the RAN1 progress, i.e. periodic CSI-RS is applicable to UEIBM. But it should be noted that different from SSB, the CSI-RS based beam management was not specially discussed for the inter-cell mTRP scenario in legacy R16/17/18. While the scope of this UEIBM has beed determined as including both intra-cell and inter-cell cases. So if applying CSI-RS based measurement into the inter-cell case, from the perspective of beam management, is there any potential issue should be addressed or clarified, still FFS. So it is better to be more cautious on the inter-cell CSI-RS based UEIBM.

After some review on the legacy R17 ICBM especially around the RAN1 background, now we believe the reason of why no discussion developed on CSI-RS based ICBM, is that the intra-cell/inter-cell CSI-RS configurations are actually transparent to UE as no PCI used for RS sequence screambling. So from RAN1 view, they believe no difference between intra-cell/inter-cell CSI-RS based beam management, no need to specially discuss CSI-RS based ICBM, that is why only SSB based ICBM was discussed in R17 ICBM. So it seems applying CSI-RS measurement in intra-cell and inter-cell mTRP are applicable.

However, during the discussion in R17 ICBM, the following points around SSB based L1-RSRP measurement were mainly identified and concrete solutions were approved:

- The requirements are only defined for a NSC who meets the known condition. The exact known condition is defined in [6] as below:

- Agreements: - Known condition for a cell with different PCI on which L1-RSRP measurement is performed. - A cell with different PCI is considered as known provided - SSBs from serving cell and the cell with different PCI for L1-RSRP measurement has the same center frequency, SCS and SFN offset; - During the last [5]s before L1-RSRP measurement is configured, the UE has sent a valid L3 measurement report for the cell with different PCI; - Timing offset between serving cell and the cell with different PCI are within CP. - A cell with different PCI is considered as unknown provided the known condition is not meet.

- NW shall configure L3 measurement on a NSC before L1-RSRP measurement is configured for UE on that cell.

- Sharing factor are introduced when any symbol of the SSBs from serving cell and NSC are overlapping or adjacent (in time domain).

- Applicable for both inter-cell BM and inter-cell mTRP scenarios.

- Referring to CSI-RS based L1-RSRP measurement, all these points should be further check. In our view, since unable to distinguish intra-cell/inter-cell CSI-RS configuration, 3) could be ignored. 4) is just the applicable scenario clarification, no impact on CSI-RS based. For 1) and 2), they should be clarified in CSI-RS based UEIBM.

Proposal 9: For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary:

-  Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

-  The prerequisite on L3 measurement on NSC.

Besides the above clarification, for CSI-RS based UEIBM, the measurement reporting delay and evaluation period/measurement period are similar as SSB based. It seems no need to additional discuss, just extend the SSB based requirements to CSI-RS based.

Proposal 10: RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

2.3 The accuracy requirements

- As we mentioned in above clause, the UEIBM largely probably lead to beam switching, so if the reporting is not accurate enough, UE would miss chance to convey the actual event. So no matter for the basic Case 1 detection or the optional Case 2 detection, at least the legacy L1-RSRP accuracy requirement should be guaranteed.

Proposal 11: At least the legacy L1-RSRP accuracy requirement should be guaranteed for both basic Case 1 and optional Case 2.

- Conclusion

In this contribution, we have the following observations and proposals:

Proposal 1: RAN4 needs to determine the evaluation period for Event-1, Event-2 and Event-7 respectively. In details,

- For Event-1, only current beam should be monitored;

- For Event-2, both current beam and new beam should be monitored;

- For Event-7, besides new beam, all beams associated with the active TCI state list should be monitored.

Proposal 2: Based on RAN1 progress on Event-2 until now, both same periodicity and different periodicity cases should be considered to determine the evaluation period.

Proposal 3: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. The maximum of current beam periodicity and new beam periodicity should be considered for both two solutions.

- For the basic Case 1, which is one-shot measurement based reporting, the sample number is 1.

- For the optional Case 2, the sample number M and timer X should be considered.

Proposal 4: For Event-1, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period, only the periodicity of current beam is involved in.

- For the basic Case 1, which is one-shot measurement based reporting, the sample number is 1.

- For the optional Case 2, the sample number M and timer X should be considered.

Proposal 5: For the basic Case 1 of Event-7, the maximum between the new beam periodicity and the periodicity of all beams associated to the active TCI state list should be considered.

Proposal 6: For the optional Case 2 of Event-7, some further clarification in RAN1 is needed in terms of whether the Q-th best TCI state should be consistent or not during the event detection. RAN4 could wait for more RAN1 input or send LS to trigger the clarification.

Proposal 7: For event triggered UEIBM, the evaluation period is equivalent to the measurement period.

Proposal 8: Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

Proposal 9: For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary:

-  Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

-  The prerequisite on L3 measurement on NSC.

Proposal 10: RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

Proposal 11: At least the legacy L1-RSRP accuracy requirement should be guaranteed for both basic Case 1 and optional Case 2.

- Reference

- R4-2420096, WF on RRM requirements for NR_MIMO_Ph5(part 1), RAN4#113

- Final_Minutes_report_RAN1#116_v100

- Final_Minutes_report_RAN1#116b_v100

- Final_Minutes_report_RAN1#117_v020

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management, RAN1#119

- R4-2207108, WF on FeMIMO inter-cell beam management, Samsung, RAN4#102e