3GPP TSG-RAN4 Meeting # 114	R4-2500939

Athens, Grace, Feb.17th – 21th, 2025

# Source: 	ZTE Corporation

# Title: 	Discussion on HPUE RedCap in FR1

# Agenda item:	7.7.1

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, the revised WID[1] of PC2 (e)RedCap FR1 to include PC2 FDD (e)RedCap was approved. In the previous meeting, there are several agreements on FR1 PC2 TDD (e)RedCap, in which the PC2 TDD (e)RedCap UE was introduced as band agnostic way and PC2 RF requirements for non-RedCap UE are re-used. Consequently, there are less impacts on the specification. Moreover, the draft CR to introduce FR1 PC2 TDD (e)RedCap was endorsed.

For PC2 FDD (e)RedCap, the situation is more complicated since it includes both FD-FDD and HD-FDD, the Rx REFSEN degradation caused by Tx noise(a.k.a RSD requirements) should be discussed.

In this contribution, we give some discussion on PC2 FDD (e)RedCap.

# 2	Discussion

In terms of the related objectives for PC2 FDD (e)RedCap:

- Specify the PC2 RF requirements for RedCap UE on both TDD and FDD bands, based on 1Tx architecture with 26 dBm PA. - For FDD bands, specify RF requirements for PC2 RedCap UE - Band specific requirements are specified for the example bands including n3 and n1 when band agnostic way is not applicable (Example bands are selected by considering different sensitivity degradation causes) - Note: Wearable device supporting HPUE is not in the scope for FDD bands in this work item

The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

For UE maximum output power and Tx power tolerance, we do not there are reasons not to reuse the requirements specified for PC2 non-RedCap UE. Noted that for (e)Redcap UE supporting PC2 TDD band, it was already agreed that the requirements specified for PC2 in clause 6.2.1 [for TDD bands] apply.

Proposal 1. For (e)Redcap UE supporting PC2 FDD band, the requirements specified for PC2 in clause 6.2.1 for FDD bands apply.

Although there are only two example bands, i.e. n1 and n3, are included in the WID and the UE maximum output power and Tx power tolerance are band specified requirements, we think it can use a more generic sentence to describe these two requirements. We propose to update the sentence in the endorsed CR [2] as:

For (e)Redcap UE supporting PC2, the requirements specified for PC2 in clause 6.2.1 [for TDD bands] apply.

Proposal 2. To update the sentence in the endorsed CR [2] as:

For (e)Redcap UE supporting PC2, the requirements specified for PC2 in clause 6.2.1 for both FDD and TDD bands apply.

According to objectives, it seems that RAN4 may need to focus on checking whether the PC2 FDD (e)RedCap requirements could be defined in a band agnostic way, and whether the existing PC2 non-RedCap FDD requirements can be reused as much as possible.

For PC2 FDD (e)RedCap, both FD-FDD and HD-FDD are included. In terms of the agreements in the WF [3]:

Issue 2: How to specify HPUE including PC2 for RedCap Way forward: - PC2 RedCap HPUE requirements can be specified in the band agnostic way at least for TDD and HD-FDD. - The band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE. - Further discuss whether the band agnostic way can be applied for PC2 FD-FDD bands with 1Rx and 2Rx, based on more analysis on the RF requirements including A-MPR and RSD. - Note 1: 1Tx architecture is assumed for RedCap PC2. Note 2: PC2 for Rel-17 Redcap is considered under this RAN task.

We think this agreement should be respected. Technically speaking, since it is HD-FDD, there are Tx noises falls into its own Rx due to no simultaneous Tx/Rx operation happen. In our understanding, HD-FDD is a like-TDD manner operation, therefore the approach of defining RF requirements for PC2 TDD can be applied to HD-FDD. The existing PC2 non-RedCap FDD requirements can be reused for PC2 HD-FDD (e)RedCap. Noted that RSD requirements does not apply for PC2 HD-FDD (e)RedCap bands. In other words, PC2 (e)RedCap HPUE requirements can be specified in the band agnostic for HD-FDD.

Proposal 3. PC2 (e)RedCap HPUE requirements can be specified in the band agnostic for HD-FDD.

For FD-FDD (e)RedCap bands, some companies show concerns that the poor filter performance capability for RedCap UE may not be as good as the non-RedCap UE, and the MPR/A-MPR may need to be revisited.

When PC3 FR1 RedCap UE was introduced in the TS38.101-1, although new clause suffix (i.e. suffix I) was introduce, almost all of the requirements (except for REFSEN) are reused from existing PC3 requirements and only the Reference sensitivity requirements are explicitly defined. For Tx requirements including MPR, the existing PC3 non-RedCap UE requirements are reused, this is not only for FR1 RedCap, but also for FR2 RedCap UE. That’s mainly because the basic assumption is that the same non-RedCap UE PAs will be used for RedCap UE, and then PA model, PA calibration and the simulation parameters should be the same and MPR requirement is also expected to the same. When RAN4 discuss the PC2 TDD (e)RedCap UE, we believe this basic assumption is still valid, which makes the PC2 TDD non-RedCap MPR requirements are reused. We think this approach should be also applied for PC2 FDD (e)RedCap UE.

For A-MPR, we think the RF filter rejection depends on implementation, and usually for A-MPR simulation assumption, there are no RF filter rejection parameters assumed. But indeed the RF filter rejection are considered for some bands to meet the out-of-band emission mask to avoid high A-MPR, but for the example band n1 and n3 in the WID, it seems the RF rejection assumption are not considered in the A-MPR simulation due to larger duplexer separation for these two bands.

Moreover, we think usually PA model is better than what we use in the simulation considering some predistortion technical such as DPD will be used to improve the PA linearity performance. In other words, the margin is somehow assumed to absorb the impacts without filter.

If there is really needed to consider the poorer RF filter rejection for (e)RedCap comparing to non-RedCap, then it may be useful to consider delta -MPR value with delta RF filter rejection between (e)RedCap and non-RedCap although that is not our preference. If RAN4 discuss and decide which bands needs RF filter rejection parameters, and do the simulation, it would be time-consuming work which will increase the workload.

Proposal 4. We prefer to reuse the existing PC2 FDD non-RedCap UE Tx requirements for PC2 FD-FDD RedCap UE for the example bands n1 and n3.

For PC2 FD-FDD (e)RedCap Rx requirements, except RSD requirements, all of the other Rx requirements should be the same as non-RedCap PC2 FDD UE with channel BW up to 20 MHz.

For RSD requirements, it based on 1Tx architecture with 26 dBm PA, so there are two cases, which are:

- 1Tx/1Rx

- 1Tx/2Rx

For 1Tx/2Rx PC2 FDD (e)RedCap UEs, adopting the same basic assumption of PA and filter performance, the Tx noise would be the same between 1Tx/2Rx PC2 FDD non-RedCap and (e)RedCap UEs, due to the PC3 REFSEN for non-RedCap and (e)RedCap UEs are same, so the PC2 1Tx/2Rx FDD non-RedCap RSD(i.e. not supporting Tx diversity) can be used as it is for PC2 1Tx/2Rx FDD (e)RedCap RSD.

Proposal 5: To reuse PC2 1Tx non-RedCap RSD for PC2 1Tx/2Rx FDD (e)RedCap RSD.

For 1Tx/1Rx PC2 FDD (e)RedCap UEs, due to the PC3 REFSEN for non-RedCap and (e)RedCap UEs are not same, and usually the noise in diversity path is lower than the main path which will cause lower RSD after MRC correlation. So the 1Tx/1Rx PC2 FDD (e)RedCap RSD would need to be re-visited.

Therefore, we think the only additional work for 1Tx FDD PC2 (e)RedCap UEs is to develop RSD for 1Rx/1Tx FD-FDD case.

Since for the 1Rx REFSEN requirements for a PC3 RedCap UE, ΔR1R is considered on top of the REFSEN for 2Rx antenna ports, and the ΔR1R requirements are defined as band agnostic, and only related to the channel BW. To simply the work, a generic rule might be a promising solution, like equation or to define ΔRSD requirements comparing with PC2 non-RedCap RSD.

Proposal 6: The only additional work for 1Tx FDD PC2 (e)RedCap UEs is to develop RSD for 1Tx/1Rx FD-FDD case.

- To adopt a generic rule, like equation or to define ΔRSD requirements comparing with PC2 non-RedCap RSD.

# 3 Conclusion

In this contribution, we give some discussion on PC2 FDD (e)RedCap, the conclusions are summarized below:

Proposal 1. For (e)Redcap UE supporting PC2 FDD band, the requirements specified for PC2 in clause 6.2.1 for FDD bands apply.

Proposal 2. To update the sentence in the endorsed CR [2] as:

For (e)Redcap UE supporting PC2, the requirements specified for PC2 in clause 6.2.1 for both FDD and TDD bands apply.

Proposal 3. PC2 (e)RedCap HPUE requirements can be specified in the band agnostic for HD-FDD.

Proposal 4. We prefer to reuse the existing PC2 FDD non-RedCap UE Tx requirements for PC2 FD-FDD RedCap UE for the example bands n1 and n3.

Proposal 5: To reuse PC2 1Tx non-RedCap RSD for PC2 1Tx/2Rx FDD (e)RedCap RSD.

Proposal 6: The only additional work for 1Tx FDD PC2 (e)RedCap UEs is to develop RSD for 1Tx/1Rx FD-FDD case.

- To adopt a generic rule, like equation or to define ΔRSD requirements comparing with PC2 non-RedCap RSD.

# 4	Reference

[1] RP-243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom

[2] R4-2420357, (NR_PC2_RedCap_UE) CR to 38.101-1: PC2 Redcap support for TDD bands, Ericsson

[3] R4-2306650, WF on RedCap HPUE, China Telecom