3GPP TSG-RAN4 Meeting # 114	R4-2500940

Athens, Grace, Feb.17th – 21th, 2025

# Title: 	Discussion on low-low band CA via switching

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.8.2

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, a new WID was approved to introduce a called switching method to enable low-low band CA due to there exists some challenges to implement the low-low band CA band combiantion due to poor antenna implementation when sharing one common antenna, or cost/size issue when separated antennas are used.

The objectives are shown below:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this contribution, we gives some initial discussions on low-low band CA via switching (i.e. downlink carrier switching) .

# 2	Discussion

In terms of the objectives, the only switching cases are:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

It means only the FDD-SDL low-low band inter-band CA scenario, which is CA_n5A-n29A, is considered in the current WID, the other band combinations will be handled via the basket WID approach.

The illustration for the above switching cases can be found in the motivation paper, which is extracted in the following:

Fig 1.Illustration for the switching cases between FDD and SDL band

In general, considering there are only four meeting left in Rel-19 timeline, we think it may not appropriate and nearly impossible to discuss and introduce complicated carrier switching approach in such short time. Instead, the Tx switching method can be used as much as possible.For switching period, there are three candidate values, i.e.: 35us, 140us, and 210us, defined for Tx switching and the values have been discussed for a long time in Rel-16, which depends on implementation of how many RF components are switched. Considering different implementations for downlink carrier switching, some of the RF components such as filter, LNA and PLL, could be shared or separated, which is similar with Tx switching.

For low-low band CA band combination, the implementations of shared antenna and separated antennas are mentioned in TR38.872[2]. In our understanding, the purpose of this WID is to generate the carrier switching method in the case of the shared antenna for the switched-from and switched-to carriers. In other words, there is no need to consider the downlink carrier switching for separated antennas implementations since the Rx chain/antennas are totally separated for the two bands low-low CA, or the carrier switching period for separated antennas implementations is nearly 0us.

Observation 1.There is no need to consider the downlink carrier switching for separated antennas implementations for low-low CA band combination.

This is also true for Tx switching but ~0us switching period is not defined. It could be interpreted that the Tx switching is not needed for separated antennas implementation. In this case, UE needs to report the capability to indicate whether UE support UL switching, the related capabilities are extracted from TS38.306 and shown below:

| ULTxSwitchingBandPair-r16, ULTxSwitchingBandPair-v1700 Indicates UE supports dynamic UL 1Tx-2Tx switching in case of inter-band CA, SUL, and (NG)EN-DC, and UL 2Tx-2Tx switching in case of inter-band CA and SUL as defined in TS 38.214 [12], TS 38.101-1 [2] and TS 38.101-3 [4]. The capability signalling comprises of the following parameters: - bandIndexUL1-r16 and bandIndexUL2-r16 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth band entry in the band combination. UE shall indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching and indicate support for 2-layer UL MIMO capabilities on both bands in each FeatureSet entry supporting UL 2T-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2 as defined in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitchingPeriod-r16 indicates the length of UL Tx switching period of 1Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. UE shall not report the value n210us for EN-DC band combinations. n35us represents 35 µs, n140us represents 140µs, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. ..... | BC | FD | N/A | FR1 only |
| --- | --- | --- | --- | --- |

For downlink carrier switching, similarly we cannot restrict the implementation for UE supporting low-low CA band combination, but UE needs to report the downlink carrier switching capability to NW. There is no need to report the capability for those UE supports low-low CA band combination does not support downlink carrier switching feature.

Proposal 1. New capability is needed to indicate whether UE supports downlink carrier switching.

In our understanding, different switching delay time or switching period for the carriers switching happen in main Tx/Rx antenna port and diversity Rx antenna ports due to different implementation between main Tx/Rx antenna port and diversity Rx antenna ports. Nevertheless, the final switching period for carriers switching should use the same Tx switching period as baseline, and meanwhile, a new capability for downlink carrier switching period should be defined accordingly.

Proposal 2. To use 35us, 140us, and 210us as baseline for the switching period, and a new capability for downlink carrier switching period should be defined accordingly.

In terms of the justification of the WID, an enhancement of the 3GPP specification is needed to enable the following solution:

-	Device needs to support switching: when the SCell operation is triggered, UE needs to switch to the SCell, and during the operation period there is no simultaneous Tx/Rx between the PCell and the SCell

-	Device switches back to the PCell after the SCell operation is finished

In our understanding, at least it is the baseline assumption that neither of the two carriers is expected to be used for receiving during the switching period.

Proposal 3. As baseline UE assumption, neither of the two carriers is expected to be used for receiving during the switching period.

As there is no simultaneous Tx/Rx between the PCell and the SCell, as illustrated in Fig 1., Rx is only happened in either switch-from carrier or switch-to carrier. In other words, only switchedDL option (similar to switchedUL for Tx switching) can be considered for low-low CA downlink carrier switching in this WID.

Proposal 4. Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID.

For the time mask, before RAN4 discussing downlink carrier switching method, there is no any time mask defined for downlink. The time mask requirements (a.k.a transmit ON/OFF time mask) are only defined for uplink/transmitter in subclause 6.3/6.3A in TS38.101-1, including ON/OFF power transient period for single carrier operation, and for inter-band CA operation, additional switching period and its location are included when uplink carrier switching period capability is present.

For the uplink switching time mask, the ON/OFF power transient period is not included in the switching period, and the semi-static configuration of switching period are located in either carrier as indicated in RRC signalling uplinkTxSwitchingPeriodLocation. In this case, the switching period are usually located in the upper TTI edge of the switch-from carrier and the lower TTI edge of the switch-to carrier. Here TTI could be slot/subslot or symbol. In other words, the switching period should not be located in both carriers.

In addition, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier.

We think the above approach can also be applied for the time mask of downlink carrier switching except the ON/OFF power transient period, which are:

For the downlink switching time mask, the semi-static configuration of switching period are located in either carrier as indicated in RRC signalling downlink carrier switching period location. In this case, the switching period are usually located in the upper TTI edge of the switch-from carrier and the lower TTI edge of the switch-to carrier. Here TTI could be slot/subslot or symbol. Illustration for the time mask for downlink carrier switching period is shown below.

Fig 1. Illustration for the time mask for downlink carrier switching period

In addition, if there is no downlink receive scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the receive on the switch-to carrier, the switching period is fully contained in the time period between the end of the receive on the switch-from carrier and the start of the receive on the switch-to carrier.

Proposal 5. For the downlink switching time mask, the switching period are located in either carrier as indicated in RRC signalling downlink carrier switching period location.

Proposal 6. If there is no downlink receive scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the receive on the switch-to carrier, the switching period is fully contained in the time period between the end of the receive on the switch-from carrier and the start of the receive on the switch-to carrier.

Moreover, besides the default UE behaviors, some optional advanced UE features were approved in Rel-18 3 and 4 bands Tx switching. For example, Introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching.

For downlink carrier switching, some optional advanced UE features can also be considered besides the default UE behaviors. For the example band combination of CA_n5A-n29A, in terms of the table 7.3.2-2 in TS38.101-1, the band n5 supports 4Rx antenna port for FWA form factor and handheld UE optionally, but for band n29, 4Rx antenna port is not supported. In addition, n5 also supports UL MIMO.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

In the case of the carriers in both n5 and n29 support 2Rx antenna ports, then all of the 2Rx antenna port in each band carrier are involved in the carrier switching. However, as mentioned above, the switching delay time or switching period for the carriers switching happen in main Tx/Rx antenna port and diversity Rx antenna ports might be different, then during the delta switching period of each involved receiver antenna port, UE can optional report the advanced UE capability to allow the carrier to receive if the switching time for this receiver antenna port is shorter. But of course the downlink receiving should be stopped in the switched-from carrier when the carrier switching period comes.

In the case of the carrier of band n5 support 4Rx antenna ports but n29 support 2Rx antenna ports, then there are 2Rx antenna ports might not be involved in the carrier switching, UE can optional report the advanced UE capability to allow the carrier to receive in the receiver antenna port which is not involved in the carrier switching.

Proposal 7: Beside the default UE behaviors, some optional advanced UE features can be considered, which needs to be further discussed.

In current RAN4 specification, unlike the other non-SDL single band, there are no RF requirements defined for SDL single band, instead they are defined associated with inter-band CA. For example, the reference sensitivity requirements for SDL shall be verified by inter-band CA combinations with SDL band, which are supported by UE. In addition to REFSEN requirements, MSD requirements and delta Tib/Rib requirements are defined for some inter-band CA band combination with SDL band.

In the WID, there is only one example band combination, the other band combination to be handled via the basket work item approach. Although it is not clear how to pursue it in basket WID approach at this stage, it seems RAN4 will introduce tables to specify which band combination can use downlink carrier switching method. If so, it will cause specification cumbersome if the downlink carrier switching method is defined as band combination manner. Actually basket WID approach to introduce additional band combination was also discussed in Rel-18 3/4 bands Tx switching discussion. But in the end, no basket WID approach is introduced for 3/4 bands Tx switching since it will be subjected to basket WID procedure/guidance, which needs at least 3 months delay when operator request it, and also it will increase RAN4 workload. Instead it depends on whether the switching capability is present for a certain band combination.

In addition to that, it seems the work for basket WID approach for band combination supporting downlink carrier switching would overlap/conflict with the normal PC3/HPUE basket WID since the MSD work may be needed for a band combination introduced via carrier switching basket WID, but in general, such MSD work is discussed in the normal PC3/HPUE basket WID.

Moreover, when downlink carrier switching happens, whether the UL interruption is allowed should be defined per band combination.

Therefore, we think RAN4 should try to define generic downlink carrier switching requirements for all the band combinations, rather than defining band combination specified requirements.

Observation 2. No band combination tables for Tx switching are included in the spec, instead it depends on whether the switching capability is present for a certain band combination.

Proposal 8: RAN4 should try to define generic downlink carrier switching requirements, rather than band combination specified requirements.

# 3 Conclusion

In this contribution, we give some initial discussion on the downlink carrier switching to enable low-low band CA band combination.

The observations, proposals and conclusions are:

Observation 1.There is no need to consider the downlink carrier switching for separated antennas implementations for low-low CA band combination.

Observation 2. No band combination tables for Tx switching are included in the spec, instead it depends on whether the switching capability is present for a certain band combination.

Proposal 1. New capability is needed to indicate whether UE supports downlink carrier switching.

Proposal 2. To use 35us, 140us, and 210us as baseline for the switching period, and a new capability for downlink carrier switching period should be defined accordingly.

Proposal 3. As baseline UE assumption, neither of the two carriers is expected to be used for receiving during the switching period.

Proposal 4. Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID.

Proposal 6. If there is no downlink receive scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the receive on the switch-to carrier, the switching period is fully contained in the time period between the end of the receive on the switch-from carrier and the start of the receive on the switch-to carrier.

Proposal 7: Beside the default UE behaviors, some optional advanced UE features can be considered, which needs to be further discussed.

Proposal 8: RAN4 should try to define generic downlink carrier switching requirements, rather than band combination specified requirements.

# 4	Reference

[1] RP-243317,New WID on low band carrier aggregation via switching, Apple, and etc.

[2] TR38.872, Study on enhancement for 700/800/900MHz band combinations