3GPP TSG-RAN4 Meeting # 114	R4-2500947

Athens, Grace, Feb.17th – 21th, 2025

# Title: 	Views on 6Rx SRS antenna IL

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.1.3.4

# Document for:	Approval

# 1 Introduction

In last meeting, the agreements on SRS IL imbalance issue captured in the WF[1] are:

### Topic 3: SRS IL imbalance issue #### Sub-topic 3-1: General considerations for SRS IL imbalance issue Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1. Issue 3-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1. #### Sub-topic 3-2: SRS IL imbalance issue solutions Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: RAN4 to further discuss the candidate solution framework below and present proposals concerning the details. Companies are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed.. Candidate solution framework: - UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on whether condition would introduce any additional RAN4 performance requirement. No RAN1 specification impact. - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others - Details of SRS IL self-compensation and assistance framework are FFS

For the issues on the reporting SRS antenna IL imbalance, there were no concrete progress up to the last RAN4 meeting comparing to Rel-18. According to the previous discussion, the solutions proposed by companies were still diverse.

In this contribution, we continue discussing the SRS IL imbalance issues and reiterate some proposals in [2].

# 2	Discussion

The impacts on DL performance caused by SRS IL imbalance have already been identified by the majority companies in either in RAN1[3] and RAN4, where the non-negligible DL performance degradation in case SRS IL is not compensated at the UE and/or gNB sides. However, there were no conclusions on how to report the SRS IL imbalance in either RAN1 and RAN4 in Rel-18, even few of companies suspect the needs on reporting the SRS IL imbalance.

Observation 1. Majority companies shows the SRS IL imbalance would cause non-negligible performance degradation according to the past discussions.

In our understanding, current NW has no knowledge about the UE SRS IL imbalance, instead the NW can dynamically adjust the power control parameters based on network conditions and UE performance, this helps in managing the power imbalance and optimizing the overall system performance. The UE adjusts the transmission power of SRS based on the power control parameters provided by the base station.

Generally speaking, the SRS transmission power should be sufficient to overcome the path loss and channel impairments. Too weak SRS transmission power would lead to poor measurement quality, but excessive power can lead to interference with other UEs or even affect network performance. Hence, in our understanding, the UE should adjusts its power dynamically to manage imbalance and maintain proper signal balance. Meanwhile, if the SRS transmission power has an imbalance (up to >10dB for PC2 UE defined in RAN4 spec), the channel estimate will be biased. This leads the gNodeB to overestimate or underestimate the gain on specific antennas or spatial paths in the DL channel, causing errors in channel state information (CSI).

1n current RAN1 TS38.213 spec, the SRS transmission power are depends on the configured maximum output power PCMAX,f,c limit, and some parameters such as p0 , α, IL and etc.

For the term of Pcmax,f,c, it is defined in the TS38.101-1, i.e. PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c; wherein the ∆TRxSRS parameter is only incorporated in the lower boundary, i.e. PCMAX_L,f,c. It should be noted that ∆TRxSRS is not exactly the SRS insertion loss, it is  actually the delta SRS insertion loss values under the worse case where SRS powers are transmitted from the main antenna path and the antenna path other than main antenna who usually has longest RF path and includes more switchers. With more Rx branches introduced, the ∆TRxSRS requirements become larger and more serious.

In terms of the previous discussion on how to derive the ∆TRxSRS parameter for 4Rx/6Rx/8Rx, it can be found different companies assumed different example RF antenna switcher architectures and different switcher loss and RF cable loss, also different implementation margin for the final value. We believe ∆TRxSRS parameter could be better than the requirements defined in the RAN4 specification.

In addition, we believe UE has ability to do some power compensation to overcome the larger IL imbalance among all the Tx/Rx SRS antenna port, especially UE has more power to be transmitted or in the scenario where the SRS transmission power (PSRS) is below PCMAX,L, i.e. PSRS,PC ≤ PCMAX,L as mentioned in [5]. The UL self-compensation  depends on the UE implementation and we think it is optional, rather than mandatory. This compensation is aimed at maximizing the transmission of SRS power through antennas other than the primary Tx antenna, ensuring that as much power as possible is utilized efficiently.

For example, when the SRS transmission power is below the UE configured maximum output power (i.e. PCMAX,L), then UE can do the power compensation to compensate the IL and also there exist +2dB power tolerance for the Pcmax. Even UE can re-calculate the PCMAX,L with the new ΔIL to replace ∆TRxSRS to achieve higher boundary of PCMAX,L, as similar proposal 3 in [5]:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

But of course, if the UE has already achieve the PCMAX,H, and there is no power headroom, then there are no room left for UE to perform the IL compensation. Once the UE performs this power compensation, the ∆TRxSRS between antennas will be significantly reduced, leading to a more balanced and optimized signal distribution. Once the NW requests such information, it is advantageous for UE to inform the Network (NW) of this compensation information as it facilitates more precise CSI estimation.

When UE performs self-compensation of SRS IL up to its configured maximum output power, we think it would not introduce any additional RAN4 performance since SRS is optional configured only in UL timing adjustment, although SRS can be used to obtain the CSI information and beamforming, and higher SRS power, less BLER, there is no performance requirements related to SRS.

Observation 2. UE performs self-compensation of SRS IL up to its configured maximum output power would not introduce any additional RAN4 performance requirement.

Another way would be dynamic report although it is admittedly more complicated than the UE power compensation. It offers a wealth of real-time information that is invaluable in certain scenarios. This is particularly true when the RF routing for SRS changes over time, yet the RFFE/RF routing information remains unknown to the NW. UE can report actual SRS IL in dynamic reporting, providing crucial insights into the real-time performance of the system. Furthermore, the establishment of several thresholds for these actual SRS IL may be necessary to ensure that the NW can effectively monitor and manage the system's performance. Even the consideration of different capability classes based on the reported actual SRS IL could be explored taking into account the diverse capabilities and operational characteristics of individual UEs, which is to use the similar methodology of lower-MSD capability reporting.

In Rel-19, RAN1 and RAN2 are not involved in the WID, instead only RAN4 is included. In our understanding, the solutions discussed in RAN4 should little impact the RAN1/2 work as much as possible, which means the possible solution in RAN4 shall be in the scope of the existing RAN1/2’s framework. LS can be sent to RAN1 and/or RAN2 for the necessary changes once there are concrete agreements in RAN4. Therefore, we think per SRS resource reporting would little impact the RAN1’s protocol.

Moreover, Due to ∆TRxSRS requirements are defined for 2Rx, 4Rx, 6Rx and 8Rx, we think the problem would be similar for all of them although the problem is minor for 2Rx. Thus if ∆TRxSRS indication from UE to NW is optionally introduced for 6Rx, it can also apply to 2Rx/4Rx/8Rx case.

With the above consideration, we have the following proposals:

Proposal 1: Introduce reporting for the SRS IL imbalance issue in Rel-19.

Proposal 2. UE SRS IL power compensation is up to UE implementation and optional, and if UE hasn’t reached its Tx power upper limit, UE should indicate the power compensation behaviour to NW once the NW requests such information.

Proposal 3. If dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern is considered, and several thresholds associated with capability class for the actual SRS IL reporting can be considered.

Proposal 4: When UE provides assistance to the network on SRS IL compensation, it should per SRS resource reporting.

Proposal 5. Adjust the existing equations on PCMAX_L,f,c can also be considered.

Proposal 6. The SRS IL imbalance reporting mechanism in 6Rx discussion should be also specified for 2Rx, 4Rx and 8Rx.

# 3 Conclusion

In this contribution, we give some further discussions on the SRS IL imbalance related to 6Rx. The conclusions are:

Observation 1. Majority companies shows the SRS IL imbalance would cause non-negligible performance degradation according to the past discussions.

Observation 2. UE performs self-compensation of SRS IL up to its configured maximum output power would not introduce any additional RAN4 performance requirement.

Proposal 1: Introduce reporting for the SRS IL imbalance issue in Rel-19.

Proposal 2. UE SRS IL power compensation is up to UE implementation and optional, and if UE hasn’t reached its Tx power upper limit, UE should indicate the power compensation behaviour to NW once the NW requests such information.

Proposal 3. If dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern is considered, and several thresholds associated with capability class for the actual SRS IL reporting can be considered.

Proposal 4: When UE provides assistance to the network on SRS IL compensation, it should per SRS resource reporting.

Proposal 5. Adjust the existing equations on PCMAX_L,f,c can also be considered.

Proposal 6. The SRS IL imbalance reporting mechanism in 6Rx discussion should be also specified for 2Rx, 4Rx and 8Rx.

# 4	Reference

[1] R4-2420376, WF on 6Rx requirements, AT&T

[2] R4-2418694, Views on 6Rx SRS antenna IL,, ZTE, Sanechips

[3] R1-2308458 FL summary on UE SRS IL imbalance issue Moderator (Huawei)

[4] R4-2415313, On the SRS insertion loss imbalance reporting, Ericsson

[5] R4-2415686, Views on SRS insertion loss compensation and reporting enhancements, Intel