3GPP TSG-RAN4 Meeting # 114	R4-2500948

Athens, Grace, Feb.17th – 21th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	On R19 increase power limit left issue and MSD rule

# Agenda item:	7.1.1.1.1

# Document for:	Approval

# 1 Introduction

In previous meeting, except SAR solution, it seems the other issues have been reached consequence. Meanwhile, MSD rule discussed was jumped into this WID from other thread and discussed for recent two meetings.

In this contribution, we give some further discussions on the left issues and MSD rule for R19 Increasing UE transmission power limit.

# 2	Discussion

- 2.1 Duty Cycle solution

- For the duty cycle solution for SAR compliance, the agreements in the past meetings were captured in the WFs[1][2][3][4]. In the latest WF[4], how to reflect the duty cycle solution in the specifications are also given as staring point:

## 4.2 Duty cycle solution for SAR compliance Agreement: - Introduction of new duty-cycle IE is not considered - For ENDC (FDD+FDD), only reply on P-MPR, and there is no further spec changes needed - Following as starting point - <......>

- From the above starting point, we can observe that the similar texts of PC1.5 single CC are added in the example duty cycle wordings for PC1.5 TDD UL intra-band contiguous and non-contiguous CA in the WF.

- Observation 1: For PC1.5 TDD UL intra-band contiguous and non-contiguous CA, the PC1.5 single CC capabilities/approach (i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1) are reused.

- However, for PC1.5 inter-band UL CA, it become a bit complicated. As we discussed in [5], we think only consider P-MPR scheme for the HPUE 2Tx/3Tx band combination including PC2 FDD band, i.e. FDD+FDD, FDD+TDD. For PC1.5 inter-band UL CA with pure FDD+FDD configuration, it was already agreed that only reply on P-MPR for ENDC. We think it is the same for inter-band UL CA, which means the only P-MPR is applied for both PC1.5 inter-band UL CA and PC1.5 ENDC.

- Proposal 1: For inter-band UL CA (FDD+FDD), only reply on P-MPR.

- For FDD-TDD/TDD-TDD configuration, the example texts are included in the WF as starting point. However, it seems it also includes 3Tx in the example texts but there are two clauses for 2Tx (clause 6.2A.1.3) and 3Tx inter-band UL CA(clause 6.2H.3.1), respectively.

- In addition, we think the descriptions for PC2 and PC1.5 2Tx should be separated in clause 6.2A.1.3 by borrowing the idea of 3Tx in clause 6.2H.3.1.

- Considering the above, the duty cycle description for 2Tx in clause 6.2A.4.1.3 look likes:

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.3-1 and the supported power class enables the higher maximum output power than that of the default power class: If a UE supports power class 2 for the band combination listed in Table 6.2A1.3-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or – if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else; – shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 50% (DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; . maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.306. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%. If a UE supports power class 1.5 for the band combination listed in Table 6.2A1.3-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or – if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38. 306 (The exact evaluation period is no less than one radio frame); or – if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is between 23dBm and 26dBm; – shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else; – shall apply all requirements for the supported power class 1.5 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 50% (DutyNR,x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR,x, DutyNR,y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, represent the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

- Proposal 2: To consider the modifications for PC1.5 2Tx inter-band UL CA duty cycle solution.

- 2.2 Pcmax tolerance

- For Pcmax tolerance for inter-band NR-CA, it was agreed to use 31dBm as the upper bound, which is shown below. where the 31dBm=26dBm+29dBm to compliance with the case when the higher power limits capabilities is enabled, and it is for 3Tx inter-band NR CA:

Agreement: - Modify PCMAX tolerance for inter-band NR-CA as following PCMAX (dBm); Tolerance TLOW(PCMAX) (dB); Tolerance THIGH(PCMAX) (dB). 23 ≤ PCMAX ≤ 31; 3.0; 2.0. 22 ≤ PCMAX < 23; 5.0; 2.0. 21 ≤ PCMAX < 22; 5.0; 3.0. 20 ≤ PCMAX < 21; 5.0; 4.0. 16 ≤ PCMAX < 20; 5.0. 11 ≤ PCMAX < 16; 6.0. -40 ≤ PCMAX < 11; 7.0. - Provide draft CR for TS 38.101-1 (a revision of R4-2417096 which was endorsed in RAN4#112bis) in Feb meeting to reflect above agreement.

- However, for 2Tx inter-band ENDC, the Pcmax tolerance is defined in current specification TS38.101-3, and it is applied for 3Tx inter-band ENDC.

- In our understanding, the upper bound 33dBm comes from the Pemax upper bound, and also it is true to use 33dBm here because the tolerance for Pcmax≥23dBm are the same. However, 33dBm cannot be never achieved in current methodology for HPUE 3Tx ENDC.

- In theory, the upper bound for both PC1.5 inter-band ULCA and PC1.5 inter-band ENDC should be the same due to increased power limits feature is enable for both. However, there exist inconsistency between inter-band ULCA and ENDC. Therefore, from specification perspective, it should keep consistency between HPUE inter-band ULCA and HPUE ENDC.

- Proposal 3: From specifications perspective, it should keep consistency for Pcmax tolerance between HPUE inter-band ULCA and HPUE ENDC.

- 2.3 MSD rule

- In terms of the previous discussion, it can be seen that companies have strong will to simplify the existing MSD table for HPUE band combination considering the huge work for HPUE artificial MSD calculation based on the worse case or different assumptions which cause high MSD value. In general, the high MSD value derived from the worse case is hardly happened in the practical.

- In last RAN4 meeting, there are some high level agreements in the WF[6] :

Issue 1.2.4-2: MSD rules Agreement: - Work on the feasible unified Equations/LUT/Simplified guidelines for MSD for HPUE band combinations as normative requirements - Targeting at introduction of unified Equations/LUT/Simplified guidelines to replace the existing method using HPUE MSD tables at the end of Rel-19 or earlier if agreed. - The existing MSD requirements for band combinations for HUPE won’t be changed. - Discuss whether the request procedure for HPUE band combinations should be kept - For Rel-19, follow tradition procedure, including the request of HPUE band combinations, until the work on the feasible unified Equations/LUT/Simplified guidelines is finished.

- One of the issue is to discuss whether the request procedure for HPUE band combinations should be kept. In our understanding, after the MSD rule for HPUE band combination is agreed, it will large impact on the HPUE band combination work since there are no technical issue for the HPUE band combination, which makes no TP to TR work for HPUE band combination, instead only draft CR work to introduce the new band combination. It will largely reduce the workload.

- If the request procedure for HPUE band combinations is not kept, then somehow it means there is no need to generate the HPUE basket WID in later release (i.e. Rel-20 and onward).

- Observation 2: With MSD rule, there are no technical issue for the HPUE band combination which makes it feasible to introduce the newly HPUE band combination via draft CR.

- Observation 3: Without request procedure for HPUE band combinations, there is no need to generate the HPUE band combination basket WID in later release (i.e. Rel-20 and onward).

- In other words, without request procedure for HPUE band combination, all of the PC3 band combination will imply support HPUE band combination automatically.

- For the MSD rule, there are two options for the harmonic, receiver harmonic mixing and cross-band isolation interference, which are:

WF on 1UL HPUE MSD simplification： Agreement: - For new Rel-19 inter-band CA combinations within FR1, and for MSD due to harmonic, receiver harmonic mixing and cross-band isolation interference, the 1UL HPUE MSD requirements are specified using a delta MSD relative to the default power class 3 MSD as: PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5, - Companies are invited to evaluate two options, - option 1: RAN4 keeps on specifying two sets PC2 requirements [4,5]: - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB, - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB, - option 2: RAN4 specifies a single set PC2 requirement [3]: - one PC2 LUT/equation PC2 MSD = PC31Tx MSD + PC2, e.g. plateau at ~6dB.

- Although either option can workable, we slight prefer Option 1 to use two sets PC2 requirements for the feasibility, and also 1Tx is the baseline assumption for the harmonic, receiver harmonic mixing and cross-band isolation MSD requirements, and not all of band combinations support 2Tx.

- Moreover, the MSD rule should be applied to all 2Tx/3Tx HPUE band combinations.

- For 3Tx HPUE band combination, PC3 3Tx band combinations were supported by default. But there were no discussions how to derive PC3 3Tx MSD, instead the MSD requirements for HPUE 3Tx band combination were reused from the corresponding HPUE 2Tx band combination, we think this rule shall also be applied for all the applicable 3Tx HPUE band combination in R19.

- We have the following proposals:

- Proposal 4: For 3Tx PC2/PC1.5 inter-band UL CA, the MSD requirements are reused from the corresponding HPUE 2Tx band combination.

- Proposal 5: RAN4 keeps on specifying two sets PC2 requirements for 1Tx and 2Tx, respectively.

# 3 Conclusion

In this contribution, we give further discussions on PC1.5 Intra-band contiguous and non-contiguous UL CA. The proposals and conclusions are:

duty cycle:

- Observation 1: For PC1.5 TDD UL intra-band contiguous and non-contiguous CA, the PC1.5 single CC capabilities/approach (i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1) are reused.

- Proposal 1: For inter-band UL CA (FDD+FDD), only reply on P-MPR.

- Proposal 2: To consider the modifications for PC1.5 2Tx inter-band UL CA duty cycle solution.

- Pcmax tolerance:

- Proposal 3: From specifications perspective, it should keep consistency for Pcmax tolerance between HPUE inter-band ULCA and HPUE ENDC.

- MSD rule:

- Observation 2: With MSD rule, there are no technical issue for the HPUE band combination which makes it feasible to introduce the newly HPUE band combination via draft CR.

- Observation 3: Without request procedure for HPUE band combinations, there is no need to generate the HPUE band combination basket WID in later release (i.e. Rel-20 and onward).

- Proposal 4: For 3Tx PC2/PC1.5 inter-band UL CA, the MSD requirements are reused from the corresponding HPUE 2Tx band combination.

- Proposal 5: RAN4 keeps on specifying two sets PC2 requirements for 1Tx and 2Tx, respectively.

# 4	Reference

[1] R4-2410750, WF on high power UE, Samsung

[2] R4-2414277, WF on HPUE for CA in TN, Samsung

[3]R4-2417098, WF on HPUE RF requirements, Samsung

[4]R4-2420329, WF on HPUE for CA in TN, Samsung

[5] R4-2415904 Discussion on R19 HPUE duty cycle and MSD rule, ZTE Corporation, Sanechips

[6] R4-2420380, WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm