3GPP TSG-RAN4 Meeting # 114	R4-2500949

Athens, Grace, Feb.17th – 21th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	On release independence for 3Tx inter-band UL CA

# Agenda item:	7.1.1.1.3

# Document for:	Approval

# 1 Introduction

In last meeting, we provide some discussion on release independence for 3Tx inter-band UL CA for handheld UE and FWA UE in [1]. A\

The agreements related to release independence of 3Tx are included in the WF [2]:

- Release independence of 3Tx Agreement: - For FWA, Rel-17 - For handheld UE, FFS in next meeting

It can be seen that there is only agreement for FWA 3Tx, but it is still FFS for handheld UE.

We submit this contribution based on [1] with removing some contents which are already achieved agreements in the WF.

# 2	Discussion

In Rel-19, the valid UL configurations were agreed for 3Tx inter-band CA, which are:

- Don not consider 3ULCC with 3Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario

- Do not consider 4UL CC with PC2/PC1.5 2Tx/3Tx inter-band UL NR CA/ENDC scenario.

The above agreements are applied for both FWA UE and handheld UE.

However, for the DL band number, there are no restriction during the discussion in R19 NR_ENDC_RF_Ph4 WID , instead it can be referred to HPUE_NR_CADC_SUL_R19 WID. However, there are no explicit information for the DL band number in revised WID [3]. Ideally, up to 6 DL bands can be considered but it depends on the proponent’s request.

In the draft big CR[4], still there are only two bands 2 DL/UL bands with 2UL CCs are introduced for 3Tx inter-band CA configurations. However, there exists 3DL bands or 2 DL bands with 3DL CC were requested in the excel sheet in the HPUE_NR_CADC_SUL_R19 WID [3].

Due to the request for new configurations for 3Tx inter-band CA will continue for the whole R19, it is impossible to predict which types of DL/UL configurations will be requested by the proponents, and even there are requests but it is impossible to predict when to introduce the specification via basket WID work.

Observation 1. It is impossible to predict which types of DL/UL configurations will be requested, and also it is impossible to predict when to introduce the specification via basket WID work.

But the discussion in R19 NR_ENDC_RF_Ph4 WID should be general to applicable for all of the possible/valid configurations. Unpredictable DL/UL configurations in the observation 2 will make it quite complicated for the release independence discussion if considering the detail information for DL/UL as mentioned in observation 1.

To void the complicated situation, we propose to apply the same release independence for all the configurations of 3Tx inter-band CA for handheld UE and FWA UE, respectively.

Proposal 1. To apply the same release independence for all the configurations of 3Tx inter-band CA for handheld UE.

For handheld UE, it seems it would be impossible for 3Tx band combination handheld UE to be released independence from R16, the earliest release should be the same as FWA UE, i.e. R17. To make the 3Tx inter-band CA band combinations are valid in the filed as earlier as possible, it is proposed to be release independent from Rel-17.

would be R17, which is same as FWA UE. Meanwhile, from R18, even R19 could also be the options. So we have following proposal.

Proposal 2: For the 3Tx inter-band UL CA/ENDC configurations for handheld UE, the release independence is from Rel-17.

With the above proposals, the only changes are to introduce the new introduced configuration in the table. A companion CR can be found in [3] in this meeting.

# 3 Conclusion

In this contribution, we give the discussions for the release independence for 3Tx band combination for handheld UE and FWA UE. The proposals and conclusions are:

Observation 1. It is impossible to predict which types of DL/UL configurations will be requested, and also it is impossible to predict when to introduce the specification via basket WID work.

Proposal 1. To apply the same release independence for all the configurations of 3Tx inter-band CA for handheld UE.

Proposal 2: For the 3Tx inter-band UL CA/ENDC configurations for handheld UE, the release independence is from Rel-17.

A companion CR can be found in [5] in this meeting.

# 4	Reference

[1] R4-2418692, On release independence for 3Tx inter-band UL CA for handheld UE and FWA UE, ZTE Corporation, Sanechips

[2] R4-2420329, WF on HPUE for CA in TN, Samsung

[3] RP-242003_Revised WID for HPUE_NR_CADC_SUL_R19, China Telecom, China Unicom

[4] R4-2414978, DraftBig CR to 38.101-1 new combinations for HPUE_NR_CADC_SUL_R19, China Telecom, China Unicom

[5] R4-2500950, Draft CR to TS 38.307 Release independence for R19 3Tx band combination, ZTE Corporation, Sanechips