3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500975

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Discussion on adding HAPS abbreviation and definition of HAPS BS classes to TS38.141-1/2

Agenda item:	4.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on adding HAPS abbreviation and definition of HAPS BS classes to TS 38.141-1/2.

- Discussion

For Rel-17 NR_NTN_solutions work item, in RAN4#102-e meeting, the HAPS core requirements [1] including HAPS abbreviations, HAPS BS classes and HAPS operating bands were captured into TS 38.104, But the HAPS BS classes were not introduced to TS 38.141-1/2. The definition of HAPS BS classes in TS 38.104 is copied as below:

For BS type 1-C, 1-H and 1-O, HAPS BS class is defined as indicated below: - HAPS Base Stations are characterised by requirements derived from High Altitude Platform scenarios with a BS to ground UE minimum distance of typically around 20km. - Unless otherwise stated, HAPS BS class would refer to Wide Area BS class, which is specified in clause 4.4.

As above, for HAPS core requirement, the definition of HAPS BS class in TS 38.104 indicated that the core requirements for HAPS should refer to core requirements for Wide Area BS class for TN. So for HAPS conformance requirements, it is necessary to capture the definition of HAPS BS class in TS 38.141-1/2 to indicate that conformance requirements for HAPS should refer to conformance requirements for Wide Area BS class for TN.

Observation 1: For Rel-17 NR_NTN_solutions work item, it is reasonable to capture the definition of HAPS BS class in TS 38.141-1/2 to indicate that conformance requirements for HAPS should refer to conformance requirements for Wide Area BS class for TN.

Meanwhile, since for TN BS, the Base station (BS) classes were captured in both TS 38.104 and TS 38.141-1/2. So like TN BS classes, the definition of HAPS BS class in TS 38.104 is also needed to be captured in TS 38.141-1/2.

Observation 2: For Rel-17 NR_NTN_solutions work item, like TN BS classes, the definition of HAPS BS class in TS 38.104 is also needed to be captured in TS 38.141-1/2.

For Rel-18 TEI18 work item, in RAN4#112 meeting, RACH format 1 demodulation requirements for HAPS [2,3] were introduced to TS 38.141-1 and TS 28.141-2.  For example, PRACH format for HAPS scenario declaration in Table 4.6-1 of TS 38.141-1 is copied as below:

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. D.125; PRACH format for HAPS scenario; Declaration of supported PRACH format(s) for HAPS scenario, i.e., format 1.; x; x.

As above, For Rel-18 TEI18 work item, since the “HAPS” abbreviation is used in TS 38.141-1/2, but the HAPS abbreviation is missing in sub-clause 3.3 (Abbreviations) of TS 38/141-1/2.

Observation 3: For Rel-18 TEI18 work item, since the “HAPS” abbreviation is used in TS 38.141-1/2, HAPS abbreviation is missing in sub-clause 3.3 (Abbreviations) of TS 38/141-1/2.

As above, For Rel-18 TEI18 work item, since the “HAPS scenario” is used in TS 38.141-1/2, but the definition of HAPS BS class corresponding to HAPS scenario is missing in sub-clause 4.3 (Base station classes) of TS 38/141-1/2.

Observation 4: For Rel-18 TEI18 work item, since the “HAPS scenario” is used in TS 38.141-1/2, but the definition of HAPS BS class corresponding to HAPS scenario is missing in sub-clause 4.3 (Base station classes) of TS 38/141-1/2.

Based on above analysis, we propose to add HAPS abbreviation and definition of HAPS BS class to TS 38.141-1/2 from Rel-17 NR_NTN_solutions to correct the issue of  HAPS BS class definition missing in TS 38.141-1/2.

Proposal 1: Add HAPS abbreviation and definition of HAPS BS class to TS 38.141-1/2 from Rel-17 NR_NTN_solutions to correct the issue of  HAPS BS class definition missing in TS 38.141-1/2.

Four CRs [1, 2, 3, 4] on HAPS abbreviation and definition of HAPS BS class for TS 38.141-1/2 are provided for approval.

- Conclusion

This contribution provides analysis on HAPS abbreviation and definition HAPS BS class and the following proposal is concluded as follows:

Observation 1: For Rel-17 NR_NTN_solutions work item, it is reasonable to capture the definition of HAPS BS class in TS 38.141-1/2 to indicate that conformance requirements for HAPS should refer to conformance requirements for Wide Area BS class for TN.

Observation 2: For Rel-17 NR_NTN_solutions work item, like TN BS classes, the definition of HAPS BS class in TS 38.104 is also needed to be captured in TS 38.141-1/2.

Observation 3: For Rel-18 TEI18 work item, since the “HAPS” abbreviation is used in TS 38.141-1/2, HAPS abbreviation is missing in sub-clause 3.3 (Abbreviations) of TS 38/141-1/2.

Observation 4: For Rel-18 TEI18 work item, since the “HAPS scenario” is used in TS 38.141-1/2, but the definition of HAPS BS class corresponding to HAPS scenario is missing in sub-clause 4.3 (Base station classes) of TS 38/141-1/2.

Proposal 1: Add HAPS abbreviation and definition of HAPS BS class to TS 38.141-1/2 from Rel-17 NR_NTN_solutions to correct the issue of HAPS BS class definition missing in TS 38.141-1/2.

Four CRs [4, 5, 6, 7] on HAPS abbreviation and definition of HAPS BS class for TS 38.141-1/2 are provided for approval.

- References

- R4-2207341, CR for TS 38.104: capturing HAPS requirements, Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat, RAN4#102-e

- R4-2413563, (TEI18) [NR NTN HAPS] CR for 38.141-1 on PRACH format 1 demodulation requirements, Ericsson, NTT DOCOMO, RAN4#112

- R4-2413564, (TEI18) [NR NTN HAPS] CR for 38.141-2 on PRACH format 1 demodulation requirements , Ericsson, NTT DOCOMO, RAN4#112

- R4-2500976, CR for TS 38.141-1, Adding HAPS abbreviation and definition of HAPS BS class for conformance requirements.

- R4-2500977, CR for TS 38.141-1, Adding HAPS abbreviation and definition of HAPS BS class for conformance requirements.

- R4-2500978, CR for TS 38.141-2, Adding HAPS abbreviation and definition of HAPS BS class for conformance requirements.

- R4-2500979, CR for TS 38.141-2, Adding HAPS abbreviation and definition of HAPS BS class for conformance requirements.