3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500995

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Discussion on BS RF requirements for enhancements for Air-to-ground network for NR

Agenda item:	7.12.2

Document for:	Approval

- Introduction

In RAN#106 meeting, a revised WID [1] on Enhancements for Air-to-ground network for NR was agreed, the objectives are copied as below:

### 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify the TAE requirements for support of DL MIMO for BS. [RAN4]. - Specify signaling and UE capabilities if necessary [RAN2].

In this contribution, we provide our views on BS RF requirements for enhancements for Air-to-ground network for NR.

- Discussion

As the revised WID [1], the core requirements for intra-band co-located and inter-band co-located DL CA and TAE core requirements for support of DL MIMO for BS need to be supported by ATG BS.

For TAE requirement, to support DL CA and DL MIMO for ATG BS, the TAE requirements for intra-band CA, inter-band CA, and MIMO are needed to be specified for ATG BS. It is sufficient that the existing TAE requirements for intra-band CA, inter-band CA, and MIMO for NR BS can be reused for ATG BS. The sentence “There is no TAE requirement for ATG BS.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) indicates that there is no TAE requirements specified for ATG BS in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2). It is reasonable to remove the sentence “There is no TAE requirement for ATG BS.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support TAE requirements for intra-band CA, inter-band CA, and MIMO for ATG BS.

Proposal 1: Remove the sentence  “There is no TAE requirement for ATG BS.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support TAE requirements for intra-band CA, inter-band CA, and MIMO for ATG BS.

As the objectives for the core requirements for intra-band co-located and inter-band co-located DL CA in revised WID [1], to support the intra-band co-located DL CA, requirements for contiguous and non-contiguous spectrum are needed to be specified for ATG BS. It is sufficient that the existing requirements for contiguous and non-contiguous spectrum for NR BS can be reused for ATG BS. The sentence “There are no CA requirements for ATG BS in the present specification.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) indicates that there are no intra-band CA requirements specified for ATG BS in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2). It is reasonable to remove the sentence “There are no CA requirements for ATG BS in the present specification.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support intra-band co-located DL CA for ATG BS.

Proposal 2: Remove the sentence “There are no CA requirements for ATG BS in the present specification.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support intra-band co-located DL CA for ATG BS.

As the objectives for the core requirements for intra-band co-located and inter-band co-located DL CA in revised WID [1], to support the inter-band co-located DL CA, requirements for BS capable of multi-band operation are needed to be specified for ATG BS. It is sufficient that the existing requirements for BS capable of multi-band operation for NR BS can be reused for ATG BS. The sentence “There is no multi-band operation requirement for ATG BS.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) indicates that there are no multi-band operation requirements specified for ATG BS in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2). It is reasonable to remove the sentence “There is no multi-band operation requirement for ATG BS.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support inter-band co-located DL CA for ATG BS.

Proposal 3: Remove the sentence “There is no multi-band operation requirement for ATG BS.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support inter-band co-located DL CA for ATG BS.

Three draft CRs [2, 3, 4] on Introduction of R19 ATG enhancement BS RF requirements for TS 38.104, 38.141-1 and TS 38.141-2 are provided for endorsement.

- Conclusion

This contribution provides analysis on BS RF requirements for ATG BS, and the following proposal is concluded as follows:

Proposal 1: Remove the sentence  “There is no TAE requirement for ATG BS.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support TAE requirements for intra-band CA, inter-band CA, and MIMO for ATG BS.

Proposal 2: Remove the sentence “There are no CA requirements for ATG BS in the present specification.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support intra-band co-located DL CA for ATG BS.

Proposal 3: Remove the sentence “There is no multi-band operation requirement for ATG BS.” in existing specifications (TS 38.104, TS 38.141-1, and TS 38.141-2) to support inter-band co-located DL CA for ATG BS.

Three draft CRs [2, 3, 4] on Introduction of R19 ATG enhancement BS RF requirements for TS 38.104, 38.141-1 and TS 38.141-2 are provided for endorsement.

- References

- RP-243285, Revised WID on Enhancements for Air-to-ground network for NR, CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom, RAN#106

- R4-2500996, Draft CR for 38.104, Introduction of R19 ATG enhancement BS RF requirements, CATT, RAN4#114.

- R4-2500997, Draft CR for 38.141-1, Introduction of R19 ATG enhancement BS RF requirements, CATT, RAN4#114.

- R4-2500998, Draft CR for 38.141-2, Introduction of R19 ATG enhancement BS RF requirements, CATT, RAN4#114.