3GPP TSG-RAN WG4 Meeting #114	                                                                                            R4-2501025

Athens, GR, 17 Feb –21 Feb, 2025

Source: 	CATT

Title: 	Discussion on SBFD general issues

Agenda item:	7.23.1

Document for:	Approval

- Introduction

This contribution provides further discussion on the open issues for SBFD parameters according to the WF [1] in last RAN4 meeting. Channel BW, subband configuration and guard band size are discussed.

- Discussion

  - DU/UL subband configuration

In last RAN4 meeting, the progress for the DL/UL subband configuration is good that the general principle for the DL/UL subband configuration was agreed as captured in WF [1],

============================================

Sub-topic 1-3: DL/UL subband configurations support for SBFD

Issue 1-3-1: Clarification on declaration and flexibility

Agreement:

- The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared

- The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested.

  - FFS on how to capture this in the specification.

  - This restriction shall at least be applied to FR1 WA BS

  - FFS other BS classes

    - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations.

      - FFS how to capture this in the specifications

Issue 1-3-2: DL/UL subband configurations support for SBFD

Agreement:

- For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration:

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD

    - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

  - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

    - FFS different sizes for specific UL subband for DUD and DU/UD.

    - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth.

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

  - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

===============================================

Some of the agreements are strange that it’s the first time for base station to limit the declaration number and mandatory to test all of the cases. In our understanding, the declaration approach is invented to leave the flexibility for operation and manufacturer not to set any restrictions. For SBFD feature, it’s so new that large scale deployment is not available that the specification should have sufficient flexibility to make sure that choices are in the hands of operators to use the feature’s gain. So if the agreements can’t be reversed, at least the flexibility left on the table should be kept as much as possible.

For the declaration, current limitation for WA class is too strict. We think the limitation should not be applied for MR and LA because the implementation is not as difficult as WA class. We have the following proposals,

Proposal 1: No limitation for DL/UL sub-band declaration for MR and LA class.

For the limitation for WA class, FFS the UL SB size declaration difference for DUD and DU in last meeting. Our understanding is that DUD and DU can be different because when the whole BS is the same, the DUD and DU SB sizes are naturally different. So we have the following proposal,

Proposal 2: DUD and DU pattern can have different UL SB size declaration.

  - Specification

For the specification, we prefer to add sub-clauses to current NR specification. This approach also follows the guideline of current WID in which there’s no new specification proposed. Another reason for not preferring new specification is that SBFD is a feature for NR base station not like repeater and relay, they’re very different network nodes. New requirements for SBFD are very limited so reusing NR specification for SBFD is a more proper way.

Proposal 3: Reuse TS 38.104 and TS 38.141-1/38.141-2 as the specifications to capture the SBFD RF requirements.

- Conclusion

This contribution provides our further analysis for SBFD general issues. We have the following observation and proposals.

Proposal 1: No limitation for DL/UL sub-band declaration for MR and LA class.

Proposal 2: DUD and DU pattern can have different UL SB size declaration.

Proposal 3: Reuse TS 38.104 and TS 38.141-1/38.141-2 as the specifications to capture the SBFD RF requirements.

- References

[1] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General, Samsung, RAN4#113