3GPP RAN WG4 Meeting #114	R4-2501038

Athens, GR, Feburary 17th – Feburary 21st, 2025

Title: 	Discussion on testing framework and procedure for one-sided models

Source: 	CATT

Agenda item:	7.20.2

Document for:	Discussion

Introduction

In RAN4#113 meeting, RAN4 discussed some issues related to testability and interoperability for one-sided model in Rel-19 AI/ML for NR air interface WI and the agreements were captured in [1]. In RAN#106 meeting, the study on testability and interoperability for both one-sided model and two-sided model was split and approved as a new SI project [2]. The RAN4 objectives are reproduced below:

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

According to the SID, RAN4 needs to finalize the testing framework and procedure for one-sided models and continue the study on test methodology for two-sided models. In this contribution, we will further discuss some test-related issues.

Discussion

CSI prediction test

In RAN4#113 meeting, RAN4 agreed to use TDL channel model for CSI prediction tests while CDL channel model is still on the table for generalization test. Another open issue is whether to have non-static testing conditions, which, from our understanding, is also for generalization tests. Therefore both issues can be discussed together.

Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

Based on the above agreements, RAN4 actually has determined the principle for defining tests, i.e., the tests use TDL channel model with static condition during one test. When plural tests with different static conditions are defined, the generalization performance of CSI prediction can also be verified. Hence, if non-static conditions are agreed to be used in tests, the workload will increase, which is unnecessary. But we are okay with non-static conditions if this method is adopted to define some tests in performance part. RAN4 needs to avoid that using both static and non-static conditions to define tests for same test purposes.

Regarding CDL channel model, the proponents’ proposal is that CDL channel is used for generalization tests. However, there are several parameters that can be altered for generalization tests except for channel modes. We do not see a strong motivation to use CDL channel model for generalization tests at current stage. But we are open to use CDL channel for generalization tests if the complexity and workload is controllable.

Proposal 1: CDL channel mode and non-static conditions are used only if they are proved necessary.

RSRP accuracy in BM test

As one of the options for beam prediction KPIs, the definition of RSRP accuracy was discussed for many meetings and divided into absolute RSRP accuracy and relative RSRP accuracy. In RAN4#112bis meeting, the definition of absolute RSRP accuracy was agreed with a bracket:

RAN4#112bis Agreement: Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on ground truth L1-RSRP].

According to the agreement, the absolute RSRP is derived based on predicted L1-RSRP and ground truth of L1-RSRP for the same beam. This definition applies for any beam, nevertheless it belongs to top-K beams or not. Therefore the bracket and the related contents can be removed.

Proposal 2: Absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on ground truth L1-RSRP].

Regarding the definition of relative RSRP accuracy, there were three options in last meeting:

RAN4#113: Issue 3-1: relative RSRP accuracy - Proposals - Option 1: The definition of relative RSRP accuracy shall be: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j - Option 2: The definition of relative RSRP accuracy shall be: predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams.

For option 1, the relative RSRP accuracy is derived based on ground truth of predicted beam i and the ground truth of top-1 ideal beam j. Since RSRP results of both beams are ground truth and predicted RSRP values are not considered, this definition is not suitable for RSRP prediction tests.

For option 2 and option 3, they are similar and the only difference is whether to choose the predicted beam having the largest RSRP as the benchmark. Between these two options, we slightly prefer option 3 because it aligns with the legacy method of calculating relative RSRP.

Proposal 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams (Option 3).

Positioning case 1 test

In last meeting, proposals of whether to define a requirement for case 1 were brought up and they were quite divergent.

- Proposals - Option 1: Consider to define a requirement using a test metric of 3-D distance - Option 2: Defining a general requirement is not feasible / do not define - Option 3: Postpone decision on requirements until RAN 1/2 signalling is clear

The main issue is if it is feasible to define a performance requirement like other performance requirements, i.e., is it possible to find a relation between the absolute position error and side conditions, e.g., SINR, RS bandwidth, etc. Based on our observations, the position error spans from smaller than 1 meter to a few meters in different InF scenarios with generalization considered. It seems not feasible to define a general performance requirement that applies for different scenarios. Therefore we prefer not to define requirements for case 1.

Proposal 4: RAN4 not to define performance requirements for case 1.

Positioning case 3a/3b test

Considering the AI/ML models are deployed in gNB in case 3a and LMF in case 3b, no test cases need to be defined for case 3a/3b.

Proposal 5: RAN4 does not discuss testability aspects related to positioning case 3a/3b.

Conclusions

This contribution discussed some testability and interoperability issues for AI/ML one-sided models and some proposals are provided:

CSI prediction:

Proposal 1: CDL channel mode and non-static conditions are used only if they are proved necessary.

Beam management:

Proposal 2: Absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on ground truth L1-RSRP].

Proposal 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams (Option 3).

Positioning:

Proposal 4: RAN4 not to define performance requirements for case 1.

Proposal 5: RAN4 does not discuss testability aspects related to positioning case 3a/3b.

References

[1] R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated, RAN4#113.

[2] RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, RAN#106.