3GPP TSG RAN WG4 Meeting #114		R4-2501075

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Further discussion on BS RF requirements for LP-WUS

Agenda Item:	7.26.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

For BS RF requirements for LP-WUS, RAN4#113 has made some progress and still left some issues open [1]:

Issue 1-1: Concept of LP-WUS power boosting - Adhoc WF - “The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition. - FFS whether other cases can be additionally considered. E.g., single value declaration. Issue 1-2: How to comprehend the de-boost on the NR transmission power - WF - Consider the de-boosting behaviour together with other relevant issues - Companies are encouraged to think about the draft spec language to reflect corresponding proposals #### Issue 1-3: Core requirement for LP-WUS power boosting - WF - FFS the following options - Option 1: Leave the power boosting level to manufacturer declaration - Option 2: If the manufacturer declares power boosting, the minimum LP-WUS power boosting level should be X (>0) dB for Y MHz CBW - Other options are not precluded #### Issue 1-6: General consideration on other requirements - Adhoc WF - Discuss on whether transmit signal quality should be defined - FFS whether equivalent EVM requirement could be considered, - E.g. NR needs to pass 64QAM EVM test with LP-WUS embedded in the NR carrier - FFS whether no other new RF requirements than the dynamic range or potential transmit quality requirement identified for LP-WUS

In this contribution, we provide our views on these open issues.

# 2 Discussion

Concept of LP-WUS power boosting

Since RAN4 has decided the concept of LP-WUS power boosting is defined similar to that of NB-IoT, i.e., ratio between average power of LP-WUS REs and average power of all REs (including both LP-WUS REs and NR REs),

And during the discussion, one point was raised that a single value in terms of EPRE ratio between LP-WUS and NR may be declared additionally.

First of all, declarations shall be aligned with the definition/concept, no matter whether it is a single value or not, otherwise it makes no sense.

Proposal 1: If making a declaration on LP-WUS power boosting, it must be aligned with the concept/definition of LP-WUS power boosting, i.e., EPRE ratio between LP-WUS and NR shall not be declared.

As calculated in our previous paper [2], the table below illustrates LP-WUS power boosting Vs different channel bandwidth in terms of EPRE ratio between LP-WUS and NR:

Table – 1 EPRE ratio for LP-WUS

| LP-WUS power boosting (dB) | CBW 3MHz | CBW 5MHz | CBW 10MHz | CBW 15MHz | CBW 20MHz | CBW 25MHz | CBW 30MHz | CBW 35MHz | CBW 40MHz | CBW 45MHz | CBW 50MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +3dB | N/A | 9.7 | 4.37 | 3.78 | 3.54 | 3.42 | 3.34 | 3.29 | 3.25 | 3.22 | 3.20 |
| +6dB | N/A | N/A | 13.12 | 8.91 | 7.87 | 7.39 | 7.11 | 6.92 | 6.78 | 6.69 | 6.61 |
| +9dB | N/A | N/A | N/A | 14.97 | 11.54 | 10.38 | 9.78 | 9.40 | 9.14 | 8.96 | 8.82 |

Following the agreed LP-WUS power boosting definition, if a single value is declared, it is associated with different EPRE ratios between LP-WUS and NR for different channel bandwidth and SCS.

Proposal 2: If a single value for LP-WUS power boosting is declared, multiple values of EPRE ratio between LP-WUS and NR should also be declared at the same time for different channel bandwidth and SCS.

How to comprehend the de-boost on the NR transmission power

For the de-boost / degradation on the NR Tx power, first of all, the group may need to agree on an essential principle, that is, if LP-WUS power is boosted, the maximum total transmission power of all REs including LP-WUS and NR should keep unchanged.

Proposal 3: LP-WUS power boosting does not change the maximum total transmission of all REs including LP-WUS and NR.

Under such agreement, the corresponding specification texts could be like this:

For an LP-WUS embedded within a wider NR carrier, if power boosting for the LP-WUS is enabled at a declared level, the NR transmission power shall be reduced to ensure that the total maximum transmission power across all REs, including both LP-WUS and NR, remains the same.

Proposal 4: RAN4 to specify the de-boost/degradation on the NR transmission power as:

For an LP-WUS embedded within a wider NR carrier, if power boosting for the LP-WUS is enabled at a declared level, the NR transmission power shall be reduced to ensure that the total maximum transmission power across all REs, including both LP-WUS and NR, remains the same.

Core requirement for LP-WUS power boosting

To facilitate the declaration, only non-zero power boosting is allowed to be declared, and a minimum power boosting level for a certain channel bandwidth, is needed, however, the maximum power boosting level is up to declaration thus no need to introduce a cap for this purpose.

Proposal 5: RAN4 to specify the declaration of LP-WUS power boosting as the minimum LP-WUS power boosting level should be X (>0) dB for Y MHz CBW if the manufacturer declares power boosting.

Requirement for transmit signal quality

Introducing a requirement for transmit signal quality, such as EVM, is not a new concept. However, it is closely tied to the LP-WUS signal design in RAN1. The group can engage in this discussion once RAN1 has finalized the LP-WUS signal design.

Proposal 6: RAN4 to hold on the discussion on requirement for transmit signal quality until RAN1 has finalized the LP-WUS signal design.

# 3 Conclusion

In this contribution, we have the following proposals on BS RF requirements for LP-WUS:

Proposal 1: If making a declaration on LP-WUS power boosting, it must be aligned with the concept/definition of LP-WUS power boosting, i.e., EPRE ratio between LP-WUS and NR shall not be declared.

Proposal 2: If a single value for LP-WUS power boosting is declared, multiple values of EPRE ratio between LP-WUS and NR should also be declared at the same time for different channel bandwidth and SCS.

Proposal 3: LP-WUS power boosting does not change the maximum total transmission of all REs including LP-WUS and NR.

Proposal 4: RAN4 to specify the de-boost/degradation on the NR transmission power as:

For an LP-WUS embedded within a wider NR carrier, if power boosting for the LP-WUS is enabled at a declared level, the NR transmission power shall be reduced to ensure that the total maximum transmission power across all REs, including both LP-WUS and NR, remains the same.

Proposal 5: RAN4 to specify the declaration of LP-WUS power boosting as the minimum LP-WUS power boosting level should be X (>0) dB for Y MHz CBW if the manufacturer declares power boosting.

Proposal 6: RAN4 to hold on the discussion on requirement for transmit signal quality until RAN1 has finalized the LP-WUS signal design.

# Reference

[1] R4-2419802, “WF for [113][308] NR_LPWUS”, Huawei

[2] R4-2417860, “Further discussion on BS RF requirements for LP-WUS”, CATT