3GPP TSG RAN WG4 #114	R4-2501326

Athens, Greece, Feb 17 – 21, 2025

Agenda item:		7.23.2.1

Source:	Nokia

Title:	On potentially new BS RF requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#113 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the potentially new BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the potentially new BS RF requirements for SBFD operation.

# Discussion

## BS RF Specification structure for SBFD requirements

The following was agreed in RAN4#112bis:

Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

#### How to introduce BS RF new requirements for SBFD-capable BS has also been under discussion during the last meetings. The WF presents two options and we consider as best solution to choose option-3. SBFD would be better handled in a completely new specification as is already done for example for IAB, repeaters and satellite access nodes. SBFD is not only just an enhancement to TDD but it is also combining the full duplexing of FDD systems. SBFD-capable FR1 WA BS must use separate antenna panels for transmitting and receiving and there is no agreed implementation for SBFD-capable BS. SBFD system also requires highly advanced and complex interference cancellation mechanisms and isolation materials to be used between antenna panels and sectors to protect the receiver from excessive interference. This makes it very crucial to ensure there is no mix-up in the requirements between TDD and SBFD. SBFD has also limitations where it is feasible to use and there are discussions ongoing for different RF requirements for different scenarios which might be very confusing to the reader. One could not be sure what requirements are valid for which systems and possibility for error would be highly increased. Therefore, we see that the best option for SBFD would be to have a new specification for it. There should not be bigger effort with maintenance of new specifications, as with separate specifications all SBFD related requirements will be in one place, maintenance even may be easier with separate specifications.

For introduction of new BS RF requirements for SBFD operation, solution with creating new specification is proposed (option 3).

## Requirements for transient period between SBFD and non-SBFD

The following was agreed in RAN4#113:

Way Forward: - FFS whether the transmitter transient period shall be located within the SBFD symbol or not: - FFS it is an implementation issue and no RAN4 standard impact

In RAN4#113 there was a text proposal from Samsung in R4-2418098 stating the following:

=================== Start of Text Proposal ===================

#### 6.4.2B			Transmitter transient period for SBFD

##### 6.4.2.1B			General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases antenna configuration:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

In our view this text proposal captures what is necessary regarding the transient period. The text proposal only needs to be updated regarding the BS type 1-C as it is precluded from SBFD as was agreed in RAN4#113.

On the transmitter transient period for SBFD, adopt the text proposal from R4-2418098 after removing the sentence related to BS type 1-C which has been precluded for SBFD operation.

## In-channel adjacent subband leakage ratio requirements

The following was agreed in RAN4#113:

Agreement: - New requirement for in-channel adjacent subband leakage ratio should be defined - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline.

RAN4 agreed in RAN4#113 that a new requirement for in-channel adjacent subband leakage ratio should be defined. In our view this is an important requirement to have to guarantee that the adjacent sub-band leakage to other nodes in the network is sufficiently low. The exact value of the requirement needs more discussion, since in our view it would depend on the ratio between the bandwidths of the DL and the UL subbands but the baseline value should be the existing ACLR requirement.

Existing ACLR requirement should be used as baseline for the in-channel adjacent subband leakage ratio depending on the ratio between the bandwidths of the DL and the UL subbands.

## In-channel adjacent subband blocking requirements

The following was agreed in RAN4#113:

Agreement: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration.

RAN4 also agreed in RAN4#113 that a new requirement for in-channel adjacent subband blocking should be defined. In Nokia’s view, this is very important to ensure that blocking requirement is also covered in the presence of co-site inter-sector and inter-site interference. The starting point for the requirement should be without gNB-to-gNB CLI handling schemes as there are many limitations on applying CLI mitigation. Also, as we would like to make sure the performance is guaranteed in all cases, we should define the requirement as close to worst case as possible. This would also mean to select the expected gNB-to-gNB isolation to be the worst case as well. It will be very difficult to agree what is reasonable expectation here. If reasonable compromise is not reached, there may be a need to add a note saying that the performance is achieved with some limitations.

Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes.

Reasonable expected gNB-to-gNB isolation should be the worst-case scenario to ensure functionality in the field.

# Conclusion

In this contribution, our views on the potentially new SBFD BS RF requirements are presented. The following proposals were listed:

- For introduction of new BS RF requirements for SBFD operation, solution with creating new specification is proposed (option 3).

- On the transmitter transient period for SBFD, adopt the text proposal from R4-2418098 after removing the sentence related to BS type 1-C which has been precluded for SBFD operation.

- Existing ACLR requirement should be used as baseline for the in-channel adjacent subband leakage ratio depending on the ratio between the bandwidths of the DL and the UL subbands.

Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes.

Reasonable expected gNB-to-gNB isolation should be the worst-case scenario to ensure functionality in the field.

# References

- R4-2419907 “Way Forward for [113][306] NR_duplex_evo_general”, RAN4#113, Orlando, Florida, USA, 18th-22nd November, 2024