3GPP TSG-RAN WG4 Meeting # 114															R4-2401410

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.8.2

Source:	Samsung

Title:	Views on UE RF requirements for LB CA switching

Document for:	Discussion

1	Introduction

At the latest RAN plenary meeting, RAN#106, a new RAN4-led Work Item (WI) on low band (LB) carrier aggregation (CA) via switching was approved to address a business demand for LB enhancements. The WID includes following objectives that need to be discussed within the rest of Rel-19 meetings [1].

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In terms of UE RF requirements, RAN4 is supposed to specify the time mask for carrier switching, and define corresponding UE capabilities to allow the switching between Case 1 and Case 2:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

In this contribution, given the switching scenario between Case 1 and Case 2 under LB CA, and that most of deployment constraints to define physical layer procedures and/or UE RF requirements are stable as noted in the WID in our understanding, we would like to provide our initial views focusing on the UE RF aspects considering the agreed assumption and restriction in the Rel-19 WID [1].

2	Discussion

RAN4 has experienced to enable switching mechanism with the start of NR enhancements from Rel-16. In short, it started with the uplink scenario between Case 1 (1T+1T) and Case 2 (0T+2T) with 2 bands inter-band uplink CA in Rel-16. Case 3 (2T+0T) was added in Rel-17 to enable the Tx switching between different cases across carriers based on SUL (Case 2 and 3) and NR inter-band uplink CA (Case 1, 2 and 3). Furthermore, in Rel-18, the feature extended to more configured uplink bands than its simultaneous transmission capability up to 4 bands, and added both single TAG and multiple TAGs configurations to support dynamic Tx carrier switching across the configured bands. Figure 1 summarizes the discussion history of uplink Tx switching scenarios by release.

Figure 1: Discussion history of UL Tx switching

Now, the discussion heads to another carrier switching option between low bands. Although the motivation of LB CA switching would be somewhat different with the former uplink Tx switching, the baseline framework across specs should be very similar to each other to support the carrier switching for both uplink Tx and downlink Rx. In addition, the time limitation for this WI should also be considered, which was also the main constraint for the WI scope minimized as much as possible. Therefore, it is proposed that RAN4 should leverage the uplink Tx switching work as much as possible for LB-LB CA with carrier switching.

Observation 1:	Based on the previous work items, baseline framework across specs should be very similar to each other to support the carrier switching.

Proposal 1:		RAN4 should leverage the uplink Tx switching work as much as possible for LB-LB CA with carrier switching.

In order to have a common understanding before the Rel-19 discussion in RAN4, it would be also good to summarize the target scenario, bands and assumption or restriction for this work item based on [1]. First of all, the switching scenario between LB CA cases are limited to Case 1 and Case 2 with band n5 and n29, respectively, which can also be described in Figure 1 below:

Figure 2: Switching scenario for Rel-19 (Case 1 and Case 2)

In addition, it is also required that only a semi-static switching pattern based on RRC configuration is considered to utilize the switching-based LB CA in Rel-19, while the exact switching pattern can be discussed and decided in other WG. By having the semi-static switching instead of dynamic switching, the switching timing, location would be fixed by a standardized switching pattern by RRC configuration, which would be more beneficial than ‘dynamic switching’ in terms of complexity and efficiency.

Observation 2:	Only a semi-static switching pattern based on RRC configuration is considered to utilize the switching-based LB CA in Rel-19.

Observation 3:	Switching pattern via RRC configuration could be discussed by other WG

Regarding the band combination considered for the switching-based CA in this WI, one example band combination, i.e., CA_n5A-n29A, is selected to derive general requirements with following condition as noted in [1]:

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers

For switching period, similar to the uplink Tx switching, and also described in [1], the UE supports LB CA through the switching might be necessary to support a new UE capability, e.g., [lowBandSwitchingBandPair] per band combination, to indicate the switching capability between LBs. Similar to the Tx switching, one of the parameters under the UE capability can also indicate the length of carrier switching period (X) of the LB-LB switching with [lowBandSwitchingPeriod]. Although the semi-static switching pattern may require less transient time for carrier switching than the dynamic UL Tx switching, as discussed in RAN4 previously, at least single or multiple OFDM symbol length for 15kHz SCS can be considered for [lowbandSwitchingPeriod] given the SCS assumption for both carrier in Rel-19. However, in order to enjoy the LB advantage with switching, it is recommended that the switching period (X) should be less than 210 us, e.g., [70, 140, 210], which are up to 3-symbol length for 15kHz SCS, at any case of switching between LB carrier, e.g., FDD to SDL and vice versa.

Proposal 1:		UE supports LB CA through the switching might be necessary to support a new UE capability, e.g., [lowBandSwitchingBandPair] per band combination, to indicate the switching capability between LBs.

Proposal 2:		Similar to the Tx switching, one of the parameters under the UE capability can also indicate the length of carrier switching period (X) of the LB-LB switching with [lowBandSwitchingPeriod]

Proposal 3:		Switching period (X) should be less than 210 us, e.g., [70, 140, 210], which are up to 3-symbol length for 15kHz SCS, at any case of switching between LB carrier, e.g., FDD to SDL and vice versa.

Regarding the switching location where carrier change happens, the uplink switching feature from Rel-16 defined uplinkTxSwitchingPeriodLocation, which is used to indicate whether the location of UL Tx switching period is configured in this uplink carrier or the other uplink carrier. For example, if network configures this field to TRUE for the uplink carrier(s) on one band and configures this field to FALSE for the uplink carrier(s) on the other band.

However, for the LB-LB CA switching case, the situation would not be the same with the uplink Tx switching case given the PDCCH location. From UE’s perspective, since the downlink control channel should be received from the beginning of the downlink carrier, the switching period would better not to be located at least on first three symbols of any downlink slot, but to be at the end of carrier. Consequently, given the points considered above, we proposed Figure 3 as a proper way of the switching period and location for switching mask of LB-LB CA in Rel-19.

Figure 3: Proposed switching period and location for switching mask of LB-LB CA

Observation 4:	From UE’s perspective, the downlink control channel (PDCCH) should be received from the beginning of the downlink carrier.

Proposal 4:		Switching period would better not to be located at least on first three symbols of any downlink slot, but to be at the end of carrier as Figure 3.

3	Conclusion

In this contribution, we provide our initial views focusing on the UE RF aspects considering the agreed assumption and restriction in the Rel-19 WID [1].  Following observations and proposals were derived.

Observation 1:	Based on the previous work items, baseline framework across specs should be very similar to each other to support the carrier switching.

Proposal 1:		RAN4 should leverage the uplink Tx switching work as much as possible for LB-LB CA with carrier switching.

Observation 2:	Only a semi-static switching pattern based on RRC configuration is considered to utilize the switching-based LB CA in Rel-19.

Observation 3:	Switching pattern via RRC configuration could be discussed by other WG

Proposal 1:		UE supports LB CA through the switching might be necessary to support a new UE capability, e.g., [lowBandSwitchingBandPair] per band combination, to indicate the switching capability between LBs.

Proposal 2:		Similar to the Tx switching, one of the parameters under the UE capability can also indicate the length of carrier switching period (X) of the LB-LB switching with [lowBandSwitchingPeriod]

Proposal 3:		Switching period (X) should be less than 210 us, e.g., [70, 140, 210], which are up to 3-symbol length for 15kHz SCS, at any case of switching between LB carrier, e.g., FDD to SDL and vice versa.

Observation 4: 	From UE’s perspective, the downlink control channel (PDCCH) should be received from the beginning of the downlink carrier.

Proposal 4: 		Switching period would better not to be located at least on first three symbols of any downlink slot, but to be at the end of carrier as Figure 3.

Reference

[1] 	RP-243317, New WID on low band carrier aggregation via switching, RAN#106