3GPP TSG-RAN WG4 Meeting #114	R4-2501415

# Athens, Greece, 17 Feb. – 21 Feb., 2025

Source: 	vivo

Title: 	Further consideration on RRM requirements for UE-to-UE CLI handling

Agenda Item:			7.23.3.1

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD based on [2] and [3].

- Discussion

This WI has been discussed at RAN4 112 and RAN4 112bis meeting. At previous meeting, based on the WF [2], the following agreements are available:

Issue 1-1-2: Measurement methods

- Agreements (from Thu AH session)

  - RAN4 shall specify L1 CLI measurement requirements at least for below methods:

    - Method#1: UE measures RSSI within DL subband

    - Method#2: UE measures RSRP of aggressor UE within UL subband

  - Additional methods can be considered based on further RAN1 agreements.

  - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

Issue 1-1-3: Rx beam

- Agreements (from Thu AH session)

  - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec.

  - Rx beam sweeping is not needed for L1 CLI measurement.

Issue 1-1-6: Measurement period

- Agreements (from Wed AH session)

  - For L1-SRS-RSRP, further discuss the following options based on simulation results:

    - Option 1: requirement specified with [1] sample measurement assumption

    - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement

  - For L1-CLI-RSSI,

    - Confirm measurement period requirements are defined based on 1 sample, and

    - Clarify in the requirements that the measurement is based on DL timing.

Issue 1-1-9: Scheduling and measurement restriction

- Agreements (from Thu AH session)

  - For scheduling restriction,

    - For L1-SRS-RSRP,

      - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols considering the impact of timing assumption

      - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination, and timing assumption

    - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing.

  - For measurement restriction,

    - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

      - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

In this contribution we discuss the following open issues.

#### Issue 1-1-4: Measurement resources

For the measurement resource beam, the following assumptions from RAN1 are available.

Agreement (RAN1#118)

For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported

- Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband

- Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands

- FFS: Number of resources that can be configured

- FFS: UE behavior for measurement

Agreement (RAN1#118)

For L1 UE-to-UE CLI measurement and reporting, the following are supported

- Wideband CLI-RSRP reporting

- Wideband CLI-RSSI reporting

- FFS: Subband CLI-RSSI reporting

Agreement (RAN1#118-bis)

For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs.

- A single wideband RSSI measurement report is supported

  - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

  - Proposal 1 (CMCC, ZTE, MTK, Samsung):

    - RAN4 firstly define requirements based on wideband reporting, and further discuss sub-band reporting pending on RAN1 progress.

  - Proposal 2 (Nokia):

    - RAN4 should define new requirements for the sub-band CLI-RSSI reporting supported by RAN1. Sub-band CLI-RSSI reporting is applicable for both measurement resource type #1 and type #2.

  - Proposal 3 (CATT, MTK):

    - RAN4 to evaluate whether the legacy CLI-RSSI measurement accuracy requirements are applicable for L1 CLI-RSSI measurements, when one CLI-RSSI resource occupies two discontinuous DL subbands (measurement resource type#2)

  - Proposal 4 (vivo):

    - For the wide-band reporting when measurement results from different discontinuous DL subbands need be combined (measurement resource type#2), how to combine the measurement results could be up to UE implementation.

  - Proposal 5 (LGE):

    - RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

  - Proposal 6 (HW):

    - RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource or reporting configuration.

  - Proposal 7 (vivo):

    - RAN4 needs to discuss the applicability of requirements when the number of RBs used for measurement is less than 48.

  - Proposal 8 (MTK):

    - For measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband, existing requirements are applicable provided that the minimum number of resources are satisfied.

  - Proposal 9 (Samsung):

    - For measurement resource type, RAN4 shall cover semi-static, aperiodic and periodic resource types following the configuration in CSI-ResourceConfig.

From RAN1 agreement, a single wideband RSSI measurement report is supported and FFS on the scenario where wideband report in each DL subband when two DL subband are available. The method on how to handle the scenario when 2 DL subband are available are also discussed in the other section of this WI. It is preferred to define single wideband requirements firstly for L1-CLI-RSRI and L1-SRS-RSRP for semi-static, aperiodic and periodic resource types.

Proposal 1: Preferred to define single wideband requirements firstly for L1-CLI-RSRI and L1-SRS-RSRP for semi-static, aperiodic and periodic resource types.

#### Issue 1-1-5: Side condition

- Proposals for timing offset

  - Proposal 1 (Nokia):

    - For intra-cell scenario, remove cell phase error from Rel-16 SRS-RSRP side condition.

    - For inter-cell scenario, RAN4 shall define the requirement only if sensible accuracy performance can be achieved based on the simulation results.

  - Proposal 2 (vivo):

    - From UE perspective, the differentiation on inter-cell aggressor UE or intra-cell aggressor UE maybe not possible even the NW side has this capability. The 3us cell phase synchronization error may still need be considered for the worst case.

  - Proposal 3 (Samsung):

    - Further discuss residual timing error side condition pending on RAN1 feedback with following candidate options:

      - Option 1: RAN4 specify two sets of requirements based on different residual timing error conditions

      - Option 2: RAN4 specify two sets of requirements based on different residual timing error conditions with NW indication

      - Option 3: RAN4 specify one set requirement based on case 2 for residual timing error condition

- Proposals for SRS Es/Iot

  - Proposal 1 (CATT, Samsung, ZTE):

    - Re-use Rel-16 side condition (SRS Es/Iot = 1dB) as baseline.

  - Proposal 2 (Nokia, QC):

    - Define a higher value of SRS Es/Iot as side condition.

- Proposals for SRS BW

  - Proposal 1 (CATT):

    - Use 24 PRB as baseline and wait for more progress in RF group

  - Proposal 2 (Rel-16 side condition):

    - 48 PRB

- Recommended WF

  - For timing offset, wait for RAN1 reply, and at same time check the simulation results, e.g. whether performance with 1-sample and Case 1 condition is acceptable.

  - For SRS Es/Iot, further discuss. Encourage companies to provide simulation results to check the performance with different Es/Iot conditions.

  - For SRS BW, further discuss. Encourage companies to provide simulation results to check the performance with 24 RB.

In the reply LS from RAN1 [R1-2410890], the following descriptions are available:

When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios.

From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

- For the time offset between DL timing and SRS arrival timing, the LS reply from RAN1 indicated that the two scenarios, intra-cell vs. inter-cell UE-to-UE L1 CLI measurement, has not been differentiated during RAN1 discussion. However RAN1 also indicated that the intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized.

- To our understanding the main concern is whether the 3us cell phase synchronization error should be considered or not. One possibility is to have different requirements where one requirement considers the 3us and the other does not consider 3us cell phase synchronization error.

- Proposal 2: For timing offset, multiple requirements can be defined where one requirement considers the 3us and the other does not consider 3us cell phase synchronization error.

#### Issue 1-1-6: Measurement period

- Agreements (from Wed AH session)

  - For L1-SRS-RSRP, further discuss the following options based on simulation results:

    - Option 1: requirement specified with [1] sample measurement assumption

    - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement

  - For L1-CLI-RSSI,

    - Confirm measurement period requirements are defined based on 1 sample, and

    - Clarify in the requirements that the measurement is based on DL timing.

For L1-SRS-RSRP measurement period, we support option 1 since a quick feedback is the intention to have this kind of measurement. In addition the accuracy of L1-SRS-RSRP should be kept and the corresponding side conditions of L1-SRS-RSRP could be relaxed.

- Proposal 3: For L1-SRS-RSRP measurement period, support option 1. In addition the accuracy of L1-SRS-RSRP should be kept and the corresponding side conditions of L1-SRS-RSRP could be relaxed if necessary.

Issue 1-1-7: Measurement reporting

- Proposals

  - Proposal 1 (CATT, Nokia, CTC, vivo, HW, ZTE, E///, MTK):

    - At least L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

  - Proposal 2 (LGE, CMCC, vivo, HW, ZTE, MTK, Samsung):

    - Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

- Recommended WF

  - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

  - For aperiodic reporting, L1-RSRP measurement reporting requirement for aperiodic reporting can be re-used as baseline.

Proposal 4: For measurement reporting, support both proposal 1 and proposal 2.

Issue 1-1-8: Measurement accuracy

- Proposals for L1-SRS-RSRP

  - Proposal 1 (LGE, CTC, ZTE, MTK):

    - RAN4 need to define the measurement accuracy requirements for L1-SRS-RSRP based on the simulation results at agreed side condition and measurement sample.

    - RAN4 not to reuse legacy UE-to-UE CLI accuracy requirements directly.

  - Proposal 2 (vivo):

    - The accuracy of L1-SRS-RSRP should be kept and the corresponding side conditions of L1-SRS-RSRP could be relaxed if necessary.

  - Proposal 3 (Samsung):

    - With residual timing error assumption case 2, single shot measurement can be specified with unified L1-SRS-RSRP accuracy for all SCSs i.e., 3dB with RF margin.

- Recommended WF

  - Further discuss L1-SRS-RSRP accuracy based on agreed side condition(s) and measurement sample(s).

For L1-SRS-RSRP measurement period, the intention of the functionality is to ensure a prompt and reliable feedback to NW hence we think the accuracy, although on one hand depending on simulation results, should not and also not necessary to be relaxed compared with what be defined in legacy UE-to-UE CLI accuracy requirements. Considering the possible reduction of number of measurement samples/periods (in issue 1-1-6) and the accuracy is the same as that of legacy as we suggested, the side conditions may need be relaxed if necessary.

Proposal 5: For measurement accuracy for L1-SRS-RSRP, consider relax the side conditions of accuracy requirements when using 1 samples and kept the accuracy requirements the same as that of the legacy UE-to-UE CLI accuracy requirements.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: Preferred to define single wideband requirements firstly for L1-CLI-RSRI and L1-SRS-RSRP for semi-static, aperiodic and periodic resource types.

- Proposal 2: For timing offset, multiple requirements can be defined where one requirement considers the 3us and the other does not consider 3us cell phase synchronization error.

- Proposal 3: For L1-SRS-RSRP measurement period, support option 1. In addition the accuracy of L1-SRS-RSRP should be kept and the corresponding side conditions of L1-SRS-RSRP could be relaxed if necessary.

Proposal 4: For measurement reporting, support both proposal 1 and proposal 2.

Proposal 5: For measurement accuracy for L1-SRS-RSRP, consider relax the side conditions of accuracy requirements when using 1 samples and kept the accuracy requirements the same as that of the legacy UE-to-UE CLI accuracy requirements.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2420097, WF on RRM requirements for NR_duplex_evo, RAN4 113, Huawei, RAN4 113

[3] R4-2418281, Topic summary for [113][223] NR_duplex_evo, Huawei, RAN4 113