3GPP TSG-RAN WG4 Meeting #114							        	R4-2501422

Athens, GR, 17th-21st February 2025

Title:	RRM Core requirements on CSI-RS based L1 measurement for LTM

Source:	China Telecom

Agenda item:			7.27.2.2

Document for:	Discussion

- Introduction

In the RAN4 #113 meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on CSI-RS based L1 measurement for LTM.

- Discussion

Issue 3-1-2: Whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet

<Candidate solutions> - Option 1: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2. (Apple, Huawei, vivo) - Option 2: Remove Rel-18 LTM SSB-based L1-RSRP measurements. (CMCC, Nokia, Samsung, MTK, Ericsson) - Option 2a: Remove L1 measurement for FR2-1 as side condition and consider “The CSI-RS from the target cell configured for L1 measurement remains measurable” as requirement applicability. (Xiaomi) - Option 3: Keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress. (ZTE) <Recommended WF> - Continue discussion.

We think this issue is related to the measurement procedure. If we agree with that before CSI-RS based L1 measurements, UE needs to perform SSB-based L1 measurements and reports on the same candidate cell For FR2-1, SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2 is necessary.

Or else, if the SSB from the target cell configured for L1 measurement is not detectable, there will be impact on subsequent measurements. So, we think we should keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2.

Proposal 1: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2.

Issue 3-5: Measurement on CSI-RS with repetition ON

<Candidate solutions> - Option 1: Considering the potential large overhead, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition ON in FR2-1. (MTK) - Option 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. (Huawei, CATT) - Option 2a: Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1. (vivo) <Recommended WF> - Continue discussion.

For CSI-RS resources with repetition ON in FR2, UE needs to perform L1 CSI-RS measurement with RX beam sweeping, which will introduce extra latency and overhead. This is unbenefited with the objective of LTM to reduce the delay. So we do not support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Proposal 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Issue 3-6: Measurement on CSI-RS with repetition OFF

<Candidate solutions> - Option 1: From RRM requirements perspective, for CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1. (vivo) - Option 1a: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. Considering that there is no gain compared to SSB based L1 measurement, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition OFF in FR2-1. (MTK) - Option 1b: In FR2, RAN4 will define the requirements for the following case, (CATT) - CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB. - Option 2: Define first requirements for the case where the repetition is ‘OFF’ (Nokia) <Recommended WF> - Continue discussion.

For the case which measurement on CSI-RS with repetition OFF, UE perform measurements on the configured CSI-RS resources of candidate cell where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement. and UE performs L1 CSI-RS based measurement without RX beam sweeping.

So when the CSI-RS L1-measurement resources are configured as repetition OFF, previous SSB L1-RSRP measurement is necessary. We support that for CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1, and we could define related requirements.

Proposal 3: For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1. And define the related requirements.

Issue 3-10: CSI-RS based L1 measurement w.r.t active BWP

<Candidate solutions> - Option 1: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM. (CATT) - Option 2: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK, Xiaomi, OPPO) - Option 3: Support CSI-RS based L1-RSRP measurement within active BWP and outside active BWP, and same principle can be used as start point, e.g., defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement. (ZTE) - Option 4: If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s). (vivo) - Option 5: From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: - Option 5a: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. - Option 5b: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. - Option 5c: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP. <Recommended WF> - Continue discussion.

If only support CSI-RS based L1-RSRP measurement within active BWP, many cases will be limited. From the deployment point of view, it is normal that the CSI-RS resources of neighbor cells are outside active BWP. In addition, RAN1 also agreed that there was no restriction on the frequency location of CSI-RS used for L1-measurement. So, we think that CSI-RS based L1-RSRP measurement outside active BWP should be considered in R19 LTM.

Proposal 4: CSI-RS based L1-RSRP measurement outside active BWP should be considered in R19 LTM.

Issue 3-12: Scenarios to be considered w.r.t. RTD

<Candidate solutions> - Option 1: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case. (MTK) - Option 2: Either Alt 1 or Alt 2 is ok. RTD reporting might be needed if option 2 is adopted. (Apple) - Option 3: Support RTD>CP case for CSI-RS based L1-RSRP measurement. (CT) - Option 3a: define the requirements for RTD>CP. Need further clear definition of RTD. - Option 4: Define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP. (vivo) - Option 5: RTD < CP, and RTD > CP (Ericsson, ZTE) - Option 5a: RTD < CP, and RTD > CP UE capability for supporting RTD>CP can be introduced. (CMCC, vivo) - Option 6: As the network does not know UE RTD conditions, requirements for RTD > CP shall be defined for rel-19 LTM. Whether the support of these requirements is up to UE capability or not can be FFS. (Nokia) - Option 7: RAN4 to limit the applicable condition for CSI-RS-based L1-RSRP measurement to scenarios where RTD is not larger than a certain value, e.g., CP length. The detailed side condition can be discussed together with the solution for direct/indirect RTD value reporting to NW in RAN4#114. (Qualcomm) - Option 8: RAN4 to consider introducing a solution to address the ambiguity issue regarding RRM requirement applicability due to RTD being larger than CP. The detailed solution can be discussed in RAN4#114. (Qualcomm) <Recommended WF> Continue discussion.

If restrict the RTD between serving cell and neighbor cell to be within CP, some cases with higher SCS are also restricted. So, we think it better that support RTD>CP case for CSI-RS based L1-RSRP measurement, and UE capability for supporting RTD>CP can be introduced.

Proposal 5: Support RTD < CP and RTD > CP. UE capability for supporting RTD>CP can be introduced.

Issue 3-15: Whether to consider CSI-RS based L1-SINR measurement

<Candidate solutions> - Option 1: RAN4 doesn’t need to define the RRM requirements for CSI-RS based L1-SINR measurement. (CATT) - Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1. (Apple, Qualcomm, ZTE) - Option 3: RAN4 focus on CSI-RS based L1-RSRP measurement. (Samsung) <Recommended WF> - Continue discussion.

As we know, in real field, the CSI-RS measurement is not widely used. So, we think RAN4 could focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1.

Proposal 6: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2.

Proposal 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Proposal 3: For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the SSB in FR2-1. And define the related requirements.

Proposal 4: CSI-RS based L1-RSRP measurement outside active BWP should be considered in R19 LTM.

Proposal 5: Support RTD < CP and RTD > CP. UE capability for supporting RTD>CP can be introduced.

Proposal 6: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement until it is officially confirmed by RAN1.

- References

[1] R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.