3GPP TSG-RAN WG4 Meeting #114		R4-2501465

Athens, GR, 17th – 21st February, 2025

Agenda item:		7.23.3.2

Source:		Samsung

Title:	Views on RRM requirements for Rel-19 SBFD operation

Document for:		Discussion

- Introduction

The objectives of Rel-19 “Evolution of NR duplex operation: Sub-band full duplex (SBFD)” described in latest WID [1].

In previous RAN4 meetings, RAN4 achieved some agreements and latest WF is approved in [2]. There are still some open issues in [2]. In this contribution, we continue to discuss the open issues of RRM requirements for SBFD operation.

- Discussion

CSI-RS based L1 measurement

  - For CSI-RS colliding with UL transmission:

- Agreements (from online session) - For CSI-RS colliding with UL transmission - For aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission. - For periodic and semi-persistent CSI-RS resource, RAN4 will send RAN1 LS for clarification if needed.

RAN4 has already sent an LS to RAN1 in [3] in last RAN4#113 meeting and we are waiting for reply from RAN1.

However, we want to emphsize our understanding for this issue.

From Rel-15, the UE behavior has been introduced in 38.213:

(From TS38.213 v15.) For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, or the CSI-RS if the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS in the set of symbols of the slot.

In this single carrier unpaired spectrum, the collison handling principle is: dynamic scheduled UL always is prioritized over the semi-persistent CSI-RS reception.

In RAN1 discussion in RAN#116

RAN1#116 agreements Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded. Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

In the RAN1 discussion for Case 2:

Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - Option 1: reuse the existing collision handling principles in NR for operation on a single carrier /single cell in unpaired spectrum - Support: Huawei, Spreadtrum, IDC, ZTE (for same priority case), vivo, CATT, Tejas, Samsung, Langbo, CMCC, Ericsson (FFS ULCI), OPPO, Apple, NEC, Panasonic, Sony, ETRI (baseline), QC, DOCOMO - Option 2: support additional indication from the gNB for link direction determination in upcoming slots/certain slots - Support: Nokia

Finally in the same meeting, the agreements has been achieved based on option#1:

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

In this collision priciple, it should be the same as that in Rel-15 spec which is that UL always is prioritized over DL reception including CSI-RS reception.

Proposal 1: For CSI-RS collision with dynamic UL transmission: dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement.

  - For CSI-RS puncturing in frequency domain in SBFD symbols:

The general rule and open issues are captured in WF[2] as below:

#### Issue 1-2-2: Requirements for CSI-RS based L1 measurement - Agreements (from online session) - For CSI-RS puncturing in frequency domain in SBFD symbols, - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs. - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs. - FFS for the DUD configuration. - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions. - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold. - FFS the measurement behavior for DUD configuration. - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement. - For CSI-RS distributed over SBFD and non-SBFD symbols, - Ask RAN1 clarification on whether related RAN1 agreement applies for L1 measurement (L1-RSRP/SINR measurement and RLM/BFD/CBD measurement). Further discuss the wording when drafting the LS.

For the CSI-RS distributed over SBFD and non-SBFD symbols, RAN4 has already sent an LS to RAN1 in [3] in last RAN4#113 meeting and we think it’s hard to further discuss with no reply, especially for RLM/BFD/CBD measurement.

Proposal 2: For CSI-RS distributed over SBFD and non-SBFD symbols, need RAN1 reply LS.

We can continue the discussion about the number of PRBs and subbands in frequency domain.

In legacy L1-RSRP based on CSI-RS, the bandwidth of CSI-RS is 48PRBs. In last meeting, companies are discussed whether the condition can be also applied.

If DUD configuration, the potential RRM impacts can be further discussed:

- Whether the number of X PRB can be decreased to a value is less than 48PRBs

- Whether the condition is related to one of X1 PRBs (DL subband1) and X2 PRBs (DL subband2), or X1+X2 is larger than the condition.

According to the latest agreements in RF session [4]:

##### Issue 1-1-4: Specification implementation of BS channel bandwidth support for SBFD Agreement: - The following note is introduced in Table 5.3.2-1 and Table 5.3.5-1 in TS 38.104: - In the bands applicable to SBFD operation, the channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise. ##### Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

For the DUD case, the X1 and X2 are the same or +-1RB.

Observation 1: There is no case in Rel-19 for asymmetrical two DL subbands.

Proposal 3: For CSI-RS on SBFD symbols for the DUD case, if the CSI-RS total RB bandwidth is at least X (X1+X2) PRBs but it is split across the two subbands, the existing accuracy requirements for CSI-RS based measurement can also be applied, because X1 and X2 are the same or different by only 1PRB.

In legacy L1-measurement requirements like L1-RSRP/L1-SINR, the threshold of number of PRBs of CSI-RS is 48 PRBs.

According to the agreed 20MHz channel bandwidth for band n39, to cover the scenarios of SCS = 30kHz/60kHz, 48 PRBs cannot be applied.

Proposal 4: RAN4 to support 24 PRBs of CSI-RS PRBs to cover more deployment scenarios like 20MHz channel BW.

- Conclusion

In this contribution, we provide our proposals for SBFD operation:

Proposal 1: For CSI-RS collision with dynamic UL transmission: dynamic UL transmission shall be prioritized over than L1 CSI-RS based on measurement.

Proposal 2: For CSI-RS distributed over SBFD and non-SBFD symbols, need RAN1 reply LS.

Observation 1: There is no case in Rel-19 for asymmetrical two DL subbands.

Proposal 3: For CSI-RS on SBFD symbols for the DUD case, if the CSI-RS total RB bandwidth is at least X (X1+X2) PRBs but it is split across the two subbands, the existing accuracy requirements for CSI-RS based measurement can also be applied, because X1 and X2 are the same or different by only 1PRB.

Proposal 4: RAN4 to support 24 PRBs of CSI-RS PRBs to cover more deployment scenarios like 20MHz channel BW.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei

[3] R4-2420165, LS on CSI-RS measurement with SBFD operation, MediaTek

[4] R4-2419907, Way Forward for [113][306] NR_duplex_evo_General, Samsung