3GPP TSG-RAN WG4 Meeting #114	R4-2501519

# Athens, Greece, 17th – 21st Feb, 2025

Title: 	Draft TP introduction of comparison of spatial channel models for SU-MIMO cases

Source: 	Huawei, HiSilicon

Agenda item:       7.16.2

Document for:	Approval

Background

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.

- Draft TP

# 6  Comparison of spatial channel Models

- 6.1  PDSCH performance under SU-MIMO scenario

This section provides comparison for different spatial channel models with simulation assumptions captured in Table 6.1-1.

Table 6.1-1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| TDD Slot Configuration Pattern |  | 7D1S2U |
| FR / Carrier frequency |  | FR1,3.5GHz |
| UE speed |  | 3km/h |
| Antenna configuration |  | Rank4: 4T4R Rank8: 8T8R |
| Number of layers |  | 8,4 |
| PMI |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook [Fixed] |
| Waveform |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |
| MCS |  | 13 (64 QAM table) |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | Resource allocation type | Type 0 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 for rank 8 1 for rank 4 |
| Codebook configuration | CodebookType | Single Panel Type I |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI |
| Number of HARQ Processes |  | 8 |
| Maximum HARQ transmissions |  | 4 |
| Channel Models |  | CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,8,2,8,4) CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,2,2,8,1) CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,8,2,1,1) CDL-UMa-C channel with (M,N,P,Ms,Ns)=(1,2,2,1,1) Enhanced TDL-C channel [TDL-C, XPL, High] |
| UE receiver type |  | MMSE-IRC |
| Test metric |  | SNR(dB) @ 70% of max throughput |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |
|  | UE Antenna Radiation Pattern | Omnidirectional |
|  | Antenna Panel Placement | YZ Plane |

The summary of simulation results are captured in Table 6.1-1

Table 6.1-1: Summary of simulation results

| Channel model | SNR (dB)@70%@ of max throughput |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Company 7 |
| CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,8,2,8,4 |  |  |  |  |  |  |  |
| CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,2,2,8,1) |  |  |  |  |  |  |  |
| CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,8,2,1,1) |  |  |  |  |  |  |  |
| CDL-UMa-C channel with (M,N,P,Ms,Ns)=(1,2,2,1,1) |  |  |  |  |  |  |  |
| Enhanced TDL-C channel |  |  |  |  |  |  |  |
| [TDL-C, XPL, High] |  |  |  |  |  |  |  |

Observations: TBD

- Conclusion

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.