3GPP TSG-RAN WG4 Meeting #114	  R4-2501531

Athens, Greece, February 17 – 21, 2025

Agenda Item:	7.20.2

Source:	Huawei, HiSilicon

Title:	Discussion on testing framework and procedure for one-sided models

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #113 [1], we discuss the remaining issues related to the RRM core requirement for AIML positioning.

# Test setup for AI-BM

RAN4 #113 WF [1]: Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. RAN4 #112 WF [1]: Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

For AI BM Case 1 and Case 2, measured RSRP is the model input. Both AoA and AoD are not considered as the model input. Therefore in RAN4, only the RSRP related to different beam needs to be emulated.

Observation 1: AoA and AoD are not used as model input in AI BM Case 1 and Case 2.

Proposal 1: Use single AoA as the channel model for AI BM testing.

RAN4 #112bis WF [2]: Issue 2-9: Test setup requirements - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

For TDL channel model, the derivation of Set A should take TE capability into account, including the  power range, the maximum number of beams and the number of probes.

RAN4 #112 WF [3]: Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered

The selection of the number of set A/B Tx beams need to take account of the configuration of set A/set B from other WGs. RAN4 can wait more progress of RAN1.

RAN4 #112bis WF [2]: Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

In practical, only the measured RSRP rather than the ideal RSRP can be obtained by DUT. Therefore, the measurement error should be considered when defining the requirement of prediction accuracy. RAN4 can first evaluate the relationship between SNR and RSRP measurement error. Then the measurement error will be captured in the model input during model training and model inference. Considering that the labels for model training is also obtained from DUT measurement, the measurement error should also be captured in the model output.

Observation 2: Only the measured RSRP rather than the ideal RSRP can be obtained by UE both for model training and model inference.

Proposal 2: Take the measurement error into consideration for generating the labels for model training.

According to RAN4 #112bis WF, the simulation assumption for beam prediction was agreed as a starting point. Companies are encouraged to investigate whether and how the measurement error will impact the RSRP prediction accuracy with system-level simulation assumptions. However, the measurement error  modeling is generated by link-level simulations. We understand that in RAN4 legacy, only the link-level channel model is used for defining test cases and requirements. Since there may exist a gap between the impact derived from system-level simulations and the impact derived from link-level simulations, how to use the evaluation results with system-level simulation assumptions for requirement definition study is still an open issue.

Proposal 3: For the impact of measurement error on BM performance evaluation, RAN4 to study how to use the evaluation results with system-level simulation assumptions for requirement definition study under link-level channel models.

It is observed that the performance of the model may also be impacted by other factors, including the gNB-side beampattern (which is gNB-side proprietary information), the channel conditions (e.g., the scatter in the physical environment), the receiving gain at UE (i.e., different UE implementation, UE rotation). Therefore, whether and how to define a common training/testing dataset is an open issue.

If the training dataset is not specified, how to ensure the consistency between training and testing needs to be studied. One possible solution is to let the DUT to collect training dataset and complete the model training, followed by performing model test. However, according to R18 study outcome, the training procedure is not considered in RAN4 test due to many reasons, such as test cost/time.

Even if RAN4 revisits the training procedure, how to ensure the following principle identified in R18 is also an open issue:

The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field. [2]

This is due to the reason that the parameters considered in the test set up which limit the model generalization may be totally different from real deployment. In this case, a DUT has an AI model to pass RAN4 test does not mean it can work well in the field.

Proposal 4: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to avoid that a UE can pass the test but performs poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

After achieving clear answers of the aforementioned questions, we can start to discuss how to define the setup, where the exiting FR2 OTA test set up should be considered as the baseline. Any enhancements on top of existing fancy enough framework should be justified.

Proposal 5: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

# Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1: AoA and AoD are not used as model input in AI BM Case 1 and Case 2.

Proposal 1: Use single AoA as the channel model for AI BM testing.

Observation 2: Only the measured RSRP rather than the ideal RSRP can be obtained by UE both for model training and model inference.

Proposal 2: Take the measurement error into consideration for generating the labels for model training.

Proposal 3: For the impact of measurement error on BM performance evaluation, RAN4 to study how to use the evaluation results with system-level simulation assumptions for requirement definition study under link-level channel models.

Proposal 3: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to avoid that a UE can pass the test but performs poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

Proposal 4: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

# References

- R4-2420513, “WF on AI/ML air interface part 1”, 3GPP RAN4 #113, Orlando, USA 18th ‒ 22nd November, 2024.

- R4-2414323, “WF on AI/ML”, 3GPP RAN4 #112, 19th – 23rd August, 2024.

- R4-2417212, “WF on AI/ML”, 3GPP RAN4 #112bis, Oct 14th – 18th, 2024.