3GPP TSG-RAN WG4 Meeting #114	R4-2501533

# Athens, GR, 17 - 21 Feb, 2025

Title: 	Discussion on NTN testing for NGSO channel model

Source: 	Huawei, HiSilicon

Agenda item:	7.10.4

Document for:	Discussion

Background

During RAN4#113 meeting, Way Forward [1] for NTN_testing_NGSO_channel_model was approved. In this contribution, we share our view on NTN testing for NGSO channel model.

Discussion

TE emulated channel model for satellite mobility

Time varying Doppler shift and propagation delay derivation procedure

- WF - For Keplerian definition motion base method, consider the following general procedure to generate the time varying Doppler shift and propagation delay. The detail formulars for each step can be considered as reference in Appendix 1.4. - Step 1: Set the initial ephemeris information parameters with format of position/velocity state vector in Earth-centered earth-fixed frame (ECEF) format according to the existing RAN5 test and initial UE location parameters with the following tables - Step 1.1: covert the initial ephemeris information in ECEF format in table 1 into the format of orbital parameters (Keplerian orbital elements) in ECI (Earth-centered inertial) format in table 2 for orbit generation based on two-body Keplerian model at epoch time T0 - Step 1.2: covert the configured UE location in a format of latitude, longitude, and altitude in table 3 into the ECEF format in table 4 to align with the satellite position/velocity - FFS on the value in table 1, 3 for initial ephemeris information and UE location information Table 1: Initial ephemeris information in format of position/velocity state vector in ECEF PositionVelocity-r17; Unit; Value; Abbreviation/ Symbol; Note. positionX-r17; meter; TBD; rx(0); 26 bits with 1.3 m resolution, i.e., actual value = field value*1.3. positionY-r17; meter; TBD; ry(0); 26 bits with 1.3 m resolution, i.e., actual value = field value*1.3. positionZ-r17; meter; TBD; rz(0); 26 bits with 1.3 m resolution, i.e., actual value = field value*1.3. velocityVX-r17; meter/second; TBD; vx(0); 18 bits with 0.06 m/s resolution, i.e., actual value = field value*0.06. velocityVY-r17; meter/second; TBD; vy(0); 18 bits with 0.06 m/s resolution, i.e., actual value = field value*0.06. velocityVZ-r17; meter/second; TBD; vz(0); 18 bits with 0.06 m/s resolution, i.e., actual value = field value*0.06. Note: The symbol represents the actual value derived from the value for the corresponding to the parameter. Table 2: Initial ephemeris information in format of orbital parameters (Keplerian orbital elements) in ECI Orbital-r17; Unit; Value; Abbreviation/ Symbol; Note. semimajorAxis-r17; Meter; TBD; a; 33 bits with 4.249*10-3 m resolution, i.e., actual value = 6500000+field value*4.249*10-3. eccentricity-r17; NA; TBD; e; 20 bits with 1.431*10-8 resolution, i.e., actual value = field value*1.431*10-8. periapsis-r17; Radian; TBD; ω; 28 bits with 2.341*10-8 rad resolution, i.e., actual value = field value*2.341*10-8. longitude-r17; Radian; TBD; Ω; 28 bits with 2.341*10-8 rad resolution, i.e., actual value = field value*2.341*10-8. inclination-r17; Radian; TBD; i; 27 bits with 2.341*10-8 rad resolution, i.e., actual value = field value*2.341*10-8. meanAnomaly-r17; Radian; TBD; M; 28 bits with 2.341*10-8 rad resolution, i.e., actual value = field value*2.341*10-8. Table 3: Initial UE location parameters in a format of latitude, longitude, and altitude Parameters; Unit; Description. UElatitude; degree; Initial UE latitude. UElontitude; degree; Initial UE longitude. UEaltitude; km; Initial UE altitude. Table 4: UE location parameters in a format of position state vector in ECEF Parameters; Unit; Description. UEt,x; km; UE position state vector at time t in Earth-centered earth-fixed frame (ECEF). UEt,y; km; UE position state vector at time t in Earth-centered earth-fixed frame (ECEF). UEt,z; km; UE position state vector at time t in Earth-centered earth-fixed frame (ECEF). Table 5: Constant value Parameters; Abbreviation/ Symbol; Unit; Value. earth gravitational constant; μ; m3/s2; 3986004.418e8. velocity of light; c; m/s; 299792458. Earth radius; aE; km; 6378.137. Square of Earth eccentricity; eE2; N/A; 6.6943799014 x 10-3. Earth angular speed; ωE; rad/s; 7.2921151467 x 10-5. - Step 2: Generate the estimated satellite position/velocity vector based on Keplerian model at the time T1 with the initial ephemeris information provided in step 1 - Step 2.1: Calculate the Mean Anomaly value - Step 2.2: Derive the Eccentric Anomaly at time T1 by solving Kepler’s equation - Step 2.3: Derive the True Anomaly at time T1 - Step 2.4: Covert the orbital elements in ECI format into the satellite position/velocity state vector in ECEF format with considering the earth rotation - Note: The detail numerical methods to solve the Kepler’s equation in step 2.2, including number of iterations, accuracy of requirement, can be up to implementation, for example - Numerical method - Newton-Raphson method - Number of iterations - Option 1: 4 - Accuracy requirement E-E0>ϵ - Option 1: ϵ=10-8 - Option 2: ϵ=10-6 - Note: Satellite position/velocity estimation period, T1-T0, is less than or equal to the satellite ephemeris information update period in Issue 1-1-3-4 - Step 3: Obtain the propagation delay and Doppler shift trajectory based on the interpolation operation at the anytime between T0 and time T1. - Step 3.1: Obtain the satellite position/velocity at time T1 - Step 3.2: Obtain the propagation delay and Doppler shift trajectory between time T0 and time T1, (T0 <= T1) - Note: The update granularity of propagation delay and Doppler shift should be [1] slot - Note: The details interpolation method can be up to implementation, with meets some accuracy requirement, for example - Numerical method - Option 1: cubic spline interpolation - Option 2: linear interpolation - Option 3: calculate propagation delay and Doppler shift at any time instant - Other options are not precluded - Accuracy requirement - Option 1: - Max¦delay-error¦ ~ 3e-13 s - Max¦delay-rate error¦ ~ 3.28e-12 - Other options not precluded - Note: RAN4 will conduct the calibration work for each step in post-meeting. Companies are encouraged to provide the calibration and comparison results with GAMT before RAN4#114 meeting. Based on calibration and comparison results, further discuss how to capture the time varying Doppler shift and propagation delay derivation procedure and formulars into specification in next meeting. - The proponents to use the equation of motion to derive the satellite position/velocity in Steps 1/2, as an alternative method as the Keplerian model, is encouraged to provide more detail procedure and the comparison results with the Keplerian model in the next meeting

For the accuracy requirement to solve the Kepler’s equation in step 2.2, more accuracy value is preferred, i.e. ϵ=10-8 should be fulfilled.

For the accuracy requirement to solve the Kepler’s equation in step 2.2, ϵ=10-8 should be fulfilled.

For the accuracy requirement for the delay-error and delay-rate error if interpolation method is used, the proposed value seems reasonable, i.e. max|delay-error| < 3e-13 s, max|delay-rate error| < 3.28e-12 should be fulfilled.

For the accuracy requirement for the delay-error and delay-rate error if interpolation method is used, max|delay-error| < 3e-13 s, max|delay-rate error| < 3.28e-12 should be fulfilled.

Assumption of Elevation angle range at time 0

- WF - Set more satellite position/velocity depending on test purpose and pre-existing test configuration - For demodulation test - Option 1: starting from [30°] for both NR and IoT NTN - Option 2: - starting from [ 30°] for NR NTN - starting from [10°] for IoT NTN - For RF test - starting from [10°] for both NR and IoT NTN with considering maximum positive/negative doppler - starting from [60°] for both NR and IoT NTN with considering the half maximum positive/negative doppler - For RRM test - Option 1: starting from [30°] - Option 2: starting from [90°]

For demodulation, the minimum test time can be founded in current TS 38.521-5 and 36.521-4 for NR and IoT NTN respectively. Some values are copied as following Table 2.1.3-1 and Table 2.1.3-2.

Table 2.1.3-1: Minimum Test time for PDSCH demodulation

| Test number | Reference Channel | Propagation condition | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- |
| 1-1 | R.PDSCH.1-1.1 FDD | NTN-TDLA100-200 | 30000 (Note 1) | 1.0526 | 32000 |
| 1-2 | R.PDSCH.1-2.1 FDD | NTN-TDLC5-200 | 72000 (Note 1) | 1.0526 | 76000 |
| 1-3 | R.PDSCH.1-1.1 FDD | NTN-TDLC5-200 | 80000 (Note 1) | 1.0526 | 85000 |
| 1-4 | R.PDSCH.1-1.1 FDD | NTN-TDLA100-200 | 30000 (Note 1) | 1.0526 | 32000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNAS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |

Table 2.1.3-2: Minimum Test time for PDSCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 | 845 (Note 1) | 239 | 288 | 243360 |
| 8.3.1.1.1 Test 2 | R.NB.2 FDD (200kHz, QPSK,1/3) (1x1) NTN TDLA100-10 | 523 (Note 1) | 879 | 1152 | 602496 |

We can see that for some cases under NTN fading channel, the minimum test time can be about 85s and 602.496s for NR and IoT NTN respectively. However, based on the GMAT simulation results in R4-2413333, the time duration is about 250s and 520s for 30°~150° and 10°~170° elevation angle range respectively. That means elevation angle ranges starting from either 10° or 30° is feasible for NR NTN while elevation angle range starting from either 10° or 30° is not feasible for IoT NTN. Therefore, for IoT NTN demodulation test, RAN4 should solve the problem of long test time.

For the tests other than IoT NTN demodulation test, we propose to use the elevation angle range starting from 30° to align with Rel-17/18 configuration and align RF/RRM/demod as much as possible.

The elevation angle ranges from either 10° or 30° is not feasible for IoT NTN demodulation test.

For IoT NTN demodulation test, RAN4 should solve the problem of long test time.

For the tests other than IoT NTN demodulation test, use the elevation angle range starting from 30° to align with Rel-17/18 configuration and align RF/RRM/demod as much as possible.

Initial ephemeris information at time 0

- WF - Option 1 - Consider the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point to specify the TE-emulated Doppler shift and propagation delay model Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler) Derivation Path: TS 38.508-1, Table 5.6.3.1-1. Information Element; Value/remark; Comment; Condition. EphemerisInfo-r17 ::= CHOICE {. positionVelocity-r17 SEQUENCE {. positionX-r17; -2717617; -3532.9021 (km). positionY-r17; 4550419; 5915.5447 (km). positionZ-r17; 852799; 1108.6387 (km). velocityVX-r17; 6164; 0.36984 (km/s). velocityVY-r17; -19424; -1.16544 (km/s). velocityVZ-r17; 124281; 7.45686 (km/s). }. }. NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm.. Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions) Derivation Path: Table 5.6.3.1-1. Information Element; Value/remark; Comment; Condition. SIB19-r17 ::= SEQUENCE {. ntn-Config-r17 SEQUENCE {. epochTime-r17; Not present. ephemerisInfo-r17 CHOICE {. positionVelocity-r17 SEQUENCE {. positionX-r17; -2771542. positionY-r17; 4509243. positionZ-r17; 2455572. velocityVX-r17; 20837. velocityVY-r17; -46176. velocityVZ-r17; 109528. }. }. }. }. NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm. - Option 2 - GMAT - Other options are not precluded

In our view, the initial ephemeris information depends on discussion on the elevation angle range in section 2.1.3. Also considering that RAN5 test procedure use Option 1, which is based on the example files provided in R4-2413333, we propose to decide initial ephemeris information based on the example files provided in R4-2413333 and the outputs in the discussion of elevation angle range.

Decide initial ephemeris information based on the example files provided in R4-2413333 and the outputs in the discussion of elevation angle range.

RAN4 requirements and tests

UE assumption on propagator model for satellite motion for RAN4 simulation purpose

- WF - Option 1: Consider Keplerian model for UE assumption on the propagator model - Option 2: - Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span. - Option 3: It is up to UE implementation what method is used to estimate the doppler shift and propagation delay based on the signalled ephemeris information

In our view, UE side propagator model is up to UE implementation, it is not necessary to be aligned, so Option 2 or Option 3 can be adopted.

For UE assumption on propagator model for satellite motion for RAN4 simulation purpose, Option 2 or Option 3 can be adopted.

    - Impact on RAN4 demodulation requirements

- WF - Option 1: - RAN4 NTN UE demodulation requirements specification need to configure the period of ephemeris information update - RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

In our view, whether to add additional margin depends on the channel model issues discussed in section 2.1. Once the channel model is determined, simulations can be immediately running to decide the value of the additional margin.

Once the channel model is determined, simulations can be immediately running to decide the value of the additional margin.

Inclusion in Specification

Specification of Channel Model

- WF - RAN4 to incorporate the TE-emulated channel model with time varying Doppler shift and propagation delay into the annex of TS 38.101-5 and TS 36.102 - FFS on how to capture into specification

We think how to capture into specification can be discussed in the CR drafting. The specific TE emulated timing varying Doppler shift and propagation delay derivation procedure should be captured in the specification as did for the simplified TDL channel model in the B.2 of TS 38.101-4 to ensure all TE vendors are generating the same channel models for NTN testing for unified test environment.

How to capture into specification can be discussed in the CR drafting.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- For the accuracy requirement to solve the Kepler’s equation in step 2.2, ϵ=10-8 should be fulfilled.

For the accuracy requirement for the delay-error and delay-rate error if interpolation method is used, max|delay-error| < 3e-13 s, max|delay-rate error| < 3.28e-12 should be fulfilled.

- The elevation angle ranges from either 10° or 30° is not feasible for IoT NTN demodulation test.

For IoT NTN demodulation test, RAN4 should solve the problem of long test time.

For the tests other than IoT NTN demodulation test, use the elevation angle range starting from 30° to align with Rel-17/18 configuration and align RF/RRM/demod as much as possible.

Decide initial ephemeris information based on the example files provided in R4-2413333 and the outputs in the discussion of elevation angle range.

For UE assumption on propagator model for satellite motion for RAN4 simulation purpose, Option 2 or Option 3 can be adopted.

Once the channel model is determined, simulations can be immediately running to decide the value of the additional margin.

How to capture into specification can be discussed in the CR drafting.

Reference

R4-2419786, Way Forward for [113][322] NTN_testing_NGSO_channel_model, RAN4#113, Samsung