3GPP RAN4 Meeting #114		R4-2501613

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.14.2.1

Source:	Ericsson

Title:	Further discussion on XR test method

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Approval

# Introduction

During RAN4 #113 meeting, the following agreements were made in WF [1].

This contribution provides our views related to the remaining issues regarding XR devices performance metrics and measurement time reduction solutions.

# Discussion

## System QZ size for XR device

The size of XR head-mounted devices can vary significantly based on their model. However, Generally, these devices are bigger than XR glasses and mobile phones. Furthermore, the antenna location in these devices can also vary. With the different possible sizes and without knowing the antenna location, it is worth to specify a larger Quite Zone (QZ) for testing XR head-mounted devices.

Observation 1: XR head-mounted devices are larger than XR glasses and mobile phones and their antennas may be in different locations, thus requiring a larger QZ compared to the other type of devices.

Proposal 1: Specify a minimum measurement distance for AC system with 50cm QZ.

## Performance metric for XR devices

The performance of an antenna (radiation pattern, radiation efficiency, insertion loss, etc.) depends on its size, the used material, and the interactions with the near objects. Given the different form factors of the two XR device types (head-mounted devices and glasses), the different materials and the different positions on the head phantom, their antennas will have different performances. Hence, the two device types should have different performance requirements, as a baseline, pending the measurement campaign results. Furthermore, since we are studying OTA requirements, it is worth using some antenna related parameters (e.g. maximum directivity, beamwidth, front-to-back ratio, etc.) as performance metrics.

Observation 2: The antennas of the two XR device types will have different performances. Hence, the two device types might have different performance requirements, pending the measurement campaign results.

Proposal 2: The two XR device types (head-mounted devices and glasses) might have different performance requirements. The difference in performance requirements (delta) can be decided depending on the measurement campaign results.

Proposal 3: Use some antenna related parameters (e.g. maximum directivity, beamwidth, front-to-back ratio, etc.) as performance metrics.

## Testing time reduction solutions for XR OTA

### Coarse measurement grids for XR device testing

#### TRP measurement

According to TS 38.161, “the TRP of the DUT is measured by sampling the radiated transmit power of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step of 15 degrees in both theta (θ) and phi (φ) axes for TRP measurement”. However, TS 38.161 is mainly focused on handheld UEs with a different shape factor and antenna environment compared to XR glasses.  Furthermore, as seen from TR 38.870, even for small size UEs that have quasi-omnidirectional antennas, TRP measurement error increases when increasing the grid step size as shown below:

XR glasses have different form factor and different position compared to handheld UEs. Thus, they can have different antennas that are more sensitive to the sampling grid step. Hence, XR glasses’ TRP measurement should not use coarse measurement grids.

Observation 3: Considering the results given in TR 38.870 and the potential different antenna patterns for XR glasses compared to hand-held UEs’, the XR glasses’ TRP should be measured without using a coarse grid measurement.

Proposal 4: For XR glasses TRP testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 15 degrees.

#### TRS measurement

According to TS 38.161, “the TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step of 30 degrees in both theta (θ) and phi (φ) axes for TRS measurement”. Given, the same arguments as in the case of TRP measurement, XR glasses’ TRS measurement should not use coarse measurement grids.

Proposal 5: For XR glasses TRS testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 30 degrees.

# Conclusions

This contribution provides our views on the remaining issues regarding XR devices performance metrics and measurement time reduction solutions. The contribution makes the following proposals.

Proposal 1: Specify a minimum measurement distance for AC system with 50cm QZ.

Proposal 2: The two XR device types (head-mounted devices and glasses) might have different performance requirements. The difference in performance requirements (delta) can be decided depending on the measurement campaign results.

Proposal 3: Use some antenna related parameters (e.g. maximum directivity, beamwidth, front-to-back ratio, etc.) as performance metrics.

Proposal 4: For XR glasses TRP testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 15 degrees.

Proposal 5: For XR glasses TRS testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 30 degrees.

# References

R4-2419808, “Way Forward for [113][325] TRP_TRS_Ph3” vivo, 3GPP RAN4 #113.