3GPP TSG-RAN WG4 Meeting #114                                        R4-2501675

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:              General discussion on AI mobility regarding testability and interoperability

Agenda item:			7.21.3

Document for:	Discussion

1. Introduction

Some AI mobility regarding testability and interoperability were discussed extensively in last meeting. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Considering multiple cells in the testing setup

- For mobility, neighbouring cells are of the same-level significance as the serving cell since handover is a multi-cell related behaviour. The cross-cell RRM measurement contains inter-frequency (Inter_F_C) and intra-frequency should be studied with certain spatial consistency.

- For scenarios FR1 to FR1 intra-frequency temporal domain case B and FR1 to FR1 inter-frequency inter-cell (frequency domain), cell specific and cluster approaches are considered. Based on RAN2 evaluation results, cluster approach outperforms the cell-specific approach not only in temporal domain prediction but also in frequency domain prediction. While, cluster approach refers to the prediction method where the number of cells for measurements and/or predictions is more than one.

- With the input data from two frequencies (e.g., serving cell X at frequency f1 and neighbour cell Y at frequency f2), the AI/ML model can be trained to learn the correlation between the measurements in the two frequencies. Based on RAN2 simulations, if the cell to be measured and the cell to be predicted are co-located (located in the same sector/site with the same antenna configuration), the correlation is high, the RSRP difference between the two cells is relatively stable and a good prediction accuracy is obtained. While, non-collocated inter-frequency prediction is under discussion

Proposal 1: RAN4 to consider the following testability aspects for AI mobility

- Consideration of serving and neighbour cells

- Multi-cell setup and number of cells to measure

  - The legacy number of cells per carrier that the UE is able to monitor for intra-frequency and inter-frequency can be considered as the baseline

2.2 Cell-level RSRP prediction accuracy definition (Ground truth definition of L3-RSRP for FR1 and FR2)

One key issue for RRM test methodology is the procedure of RSRP prediction accuracy testing. From our understanding, there are two directions to define ideal RSRP:

- Direction 1: Settled by TE. Since TE is able to control the RSRP level at the centre of quiet zone, the ideal RSRP can be derived from TE for noise-free or very high SNR conditions. And in this sense, the ideal RSRP is of the transmit power.

  - FFS: The power level at which SS-RSRP measurement is conducted

- Direction 2: Settled by measurement report. Since the RSRP measurements are done at UE BB side, which shall contain RF gain and IL (RSRPBB = RSRPRP + GUE + ILUE), while TE does not have such information on ideal RSRP definition UE see. From this, prior to RSRP prediction accuracy test under noise limited conditions, the TE shall first obtain the ideal RSRP. And the ideal RSRP can be derived based on the RSRP measurements reported by UE for the noise-free or very high SNR conditions and under the assumption of single signal source. There are two methods to define the ideal RSRP following legacy discussions

  - Method 1: UE is used as a reference for itself (basic method). Drawback: RF uncertainties are cancelled out, only the baseband accuracy is verified

  - Method 2: Determined based on minimum and maximum beamforming gain (finally agreed)

For the both directions above, emulation of noise-free test conditions/Noise-free link to the DUT and assumption of single signal source are supported.

However, above directions, especially direction 1, are suitable for FR1, since FR1 RRM testing is in conducted test environment and the input of useful signals to the UE baseband can be known, but very controversial to FR2 (FR2 OTA tests), for which the ideal RSRP would highly depends on UE antenna/beamforming gain, which is up to UE implementation, i.e., the uncertainty due to UE antenna gain is very large for FR2.

Proposal 2: Two directions can be considered to define ideal RSRP.

- Direction 1: Settled by TE. The ideal RSRP is of the transmit power.

- Direction 2: Settled by measurement report. The ideal RSRP can be derived based on the RSRP measurements reported by UE for the noise-free or very high SNR conditions.

For the both directions above, emulation of noise-free/high SNR test conditions and assumption of single signal source are supported

Proposal 3: For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition.

- FFS: The power level at which SS-RSRP measurement is conducted

- FFS: The exact SNR level emulated

2.3 Principle of FR2 testing in AI/ML mobility

Prior to the start of the FR2 RSRP accuracy prediction test, we’d better to study the methodology of FR2 testing in AI/ML mobility. From our understanding, at least the following aspects

- Emulation of the propagation conditions. In AI mobility L3 cell-level prediction, it is highly possible to emulate the multiple beam environment. In this sense, multi-path propagation conditions, i.e., the multi-path fading conditions between the gNB emulator and test chamber probe may need to be emulated to ensure the verification of MIMO algorithms, where TDL can be modelled. Besides, since RRM requirements are defined with AWGN as side condition, suitable AWGN implementation in an OTA environment is necessary.

- Emulation of channel models. The key parameters can contain: Maximum CBW in one carrier; Maximum CCs (if CA implementation is considered); Total number of faded paths; Number of taps for each faded path; Number of transmitters; Number of UE Rx receivers in different evaluation scenarios (temporal domain, frequency domain, cell-specific approach, cluster approach, intra-frequency cells, inter-frequency cells, different frequencies) for the proposed Sub-use-case (s).

- The number of emulated transmission reception points (TRxP). As a reference, NR cell corresponds to one or multiple TRxPs, which may be co-located or non-collocated.

- Emulation of noise and EPRE. How the emulated noise is generated/transmitted. And the SNR/SINR in various evaluation scenarios, the test will define the exact noise/SNR level per TRxP at the reference point.

- AoA setup. For OTA testing, one of the primarily part is AoA setup. The baseline RRM setup assumes that measurement equipment shall be able to emulate the signals from 2 AoAs. However, considering the aims of different tests, we have: verification of TX/RX beamforming is needed (2 AoAs/3 AoAs, 3AoAs is for multi-Rx simultaneous reception), verification of TX/RX beamforming is not needed (1AoA). While, if we focus on intermediate KPI, i.e., RSRP difference and/or measurement overhead reduction test, the assumptions of AoA setup should be further discussed.

- Rx beam direction assumption. RRM tests will consist of tests with signals transmitted from different directions that have to be identified and reported by UE. And Rx antenna gain is an important factor that to enable the search of signals from different directions by improving the link budget. Actually, the Rx antenna gain experienced by the UE will depend on the Rx beamforming codebook selected, and the exact orientation of the DUT is controlled by the TE turntable. That is to say, on the proposed AoA, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage. While, antenna gains corresponding to different directions are different, can be found in Annex B in TS 38.133. Besides, Noc level for signals coming from the two directions (beam peak and non-beam peak) are different also.

  - Considering the test uncertainties and complexities the non-beam peak direction may bring, we have concern that if our RRM tests at current stage are mainly to verify the RRM RSRP prediction performance without involving beamforming performance, whether test in non-beam peak direction is not necessary. And another reason is that since the cells to predict and train are co-located cells so far, UE has the peak beam direction for the predicted/inferred neighbor cell (s) can be testable

Observation 1: For FR2 testing, on the exact AoA to be used, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage

Proposal 4: For FR2 testing in AI/ML mobility, at least the following aspects can be studied:

- Emulation of the propagation conditions

    - AWGN and TDL

- Emulation of channel models

- The number of emulated transmission reception points (TRxP)

- Emulation of noise and EPRE

- AoA setup

- Rx beam direction assumption

Proposal 5: For FR2 testing in AI/ML mobility, multiple beam environment could be emulated

However, considering that there are some ongoing discussions on the testability and interoperability in AI/ML for NR air interface. Considering the commonality and difference of AI/ML based beam management and AI/ML mobility: FR2 VS FR1+FR2; Spatial and temporal beam prediction, serving cell (intra-cell) prediction VS temporal, spatial, and frequency domain predictions, cross-cell prediction; BM, predicted L1-RSRP VS Cell, predicted L3-RSRP; The Top-1/K (%) beam prediction accuracy KPI VS intermediate KPIs (Average L3 cell level RSRP difference) + system KPIs (HOF, RLF rate, throughput, etc) . From this, for FR2, to avoid the overlapping of similar discussions in the two WIs, especially intra-frequency intra-cell beam level prediction, we’d better to wait for more progress from AI/ML-BM

Proposal 6: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline

- RAN4 to discuss what test parameters/methodologies on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to L3 cell level measurement prediction testing to avoid the duplicate study

Proposal 7: For FR2 testing in AI mobility, if the test is to mostly verify the RRM RSRP prediction performance, RAN4 to study the feasibility of testing in beam peak direction on the exact AoA to be used

2.4 Testing of Generalization aspects

AI Model generalization is a very important and challenging issue in deep learning, which refers to the ability of the trained model to perform well on new inputs unseen during the training. From our understanding, compared to the PHY prediction, the generalization/adaptability of the AI models for mobility is more worth studying. The reasons are:

- For AI mobility, various deployment scenarios (e.g., UMa, UMi, etc.), channel environments, UE movement patterns and speeds, and frequencies exist. Meanwhile, different RRC configurations and conditions for measurement (events) need to be considered in practice. All of them will impact generalization performance of an AI/ML model.

- Multiple cells would be considered in L3 AI mobility WI. While, across different cells may capture variations in NW conditions and local radio conditions. In this sense, the risk of model overfitting to specific cell characteristics may arise, which may decrease the adaptability to different cells

- Since the prediction measurement results are L3 filtered. Different L3 filtering factors may incur different prediction accuracy. While, when training datasets collected with fixed L3 filtering factor, the corresponding AI models may have worse generalization capability in different cells, especially.

- The measurement event prediction may benefit of cell-, site- or area-specific models and the study on AI model generalization for measurement event prediction is needed

During the Rel-18 studies about AI/ML for PHY, the generalization evaluation performance of AI/ML models is verified by testing its performance with different training and test datasets (evaluation methodology in TR38.843 for generalization study), we can reuse such principle/methodologies.

Proposal 8: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

- Reuse the principle/methodologies on generalization achieved in AI/ML for NR air interface as much as possible

3 Conclusion

In this contribution, we provided our initial viewpoints to General discussion on AI mobility regarding testability and interoperability. The following observations and proposals are obtained:

Observation 1: For FR2 testing, on the exact AoA to be used, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage

Proposal 1: RAN4 to consider the following testability aspects for AI mobility

- Consideration of serving and neighbour cells

- Multi-cell setup and number of cells to measure

  - The legacy number of cells per carrier that the UE is able to monitor for intra-frequency and inter-frequency can be considered as the baseline

Proposal 2: Two directions can be considered to define ideal RSRP.

- Direction 1: Settled by TE. The ideal RSRP is of the transmit power.

- Direction 2: Settled by measurement report. The ideal RSRP can be derived based on the RSRP measurements reported by UE for the noise-free or very high SNR conditions.

For the both directions above, emulation of noise-free/high SNR test conditions and assumption of single signal source are supported

Proposal 3: For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition.

Proposal 4: For FR2 testing in AI/ML mobility, at least the following aspects can be studied:

- Emulation of the propagation conditions

    - AWGN and TDL

- Emulation of channel models

- The number of emulated transmission reception points (TRxP)

- Emulation of noise and EPRE

- AoA setup

- Rx beam direction assumption

Proposal 5: For FR2 testing in AI/ML mobility, multiple beam environment could be emulated

Proposal 6: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline

- RAN4 to discuss what test parameters/methodologies on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to L3 cell level measurement prediction testing to avoid the duplicate study

Proposal 7: For FR2 testing in AI mobility, if the test is to mostly verify the RRM RSRP prediction performance, RAN4 to study the feasibility of testing in beam peak direction on the exact AoA to be used

Proposal 8: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

- Reuse the principle/methodologies on generalization achieved in AI/ML for NR air interface as much as possible

4 References

[1] R4-2420095 “WF on RRM requirements for FS_NR_AIML_Mob”, Nokia