3GPP TSG-RAN WG4 Meeting #114                                        R4-2501680

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:              General discussion on RRM requirements for LB CA switching

Agenda item:			7.8.3

Document for:	Discussion

1 Introduction

In RAN #106, the following objectives for low band (LB) carrier aggregation (CA) via switching were agreed in RP-243317

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this paper, we would like to give a recap from Tx switching, and share some preliminary understandings in terms of impact on 	RRM requirements for LB CA switching

2 Background on Tx switching

- Actually, the discussions of Tx switching can be back to Rel-15, while in such release, inter-band UL CA, SA SUL and inter-band EN-DC RF requirements are specified assuming 0us switching period between 2 UL carriers when TDM operated, it leads to that UE cannot support 2Tx transmission when UE switch to one carrier if no switching period is reserved for the UE.

- From Rel-16~Rel-19

| Release | Scenarios to discuss |
| --- | --- |
| Rel-16 | 1Tx-2Tx UL switching with inter-band CA 2 bands |
| Rel-17 | 2Tx-2Tx UL switching with inter-band CA 2 bands |
| Rel-18 | 1/2Tx-1/2Tx UL switching with inter-band CA 3/4 bands incl. SUL bands |
| Rel-19 | LB CA switching (FDD+SDL: Tx/Rx FDD-Rx SDL) |

The detailed cases/scenarios counterpart:

- Rel-16

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | - Tx on carrier 1 and 2 Tx on carrier 2 |

- Rel-17

For Tx switching based on SUL band combination, or uplink CA band combination and For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |
|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

For Tx switching based on SUL band combination, or uplink CA band combination and For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
|  | Number of Tx chains in WID (band A + band B) |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |
|  | Number of Tx chains in WID (band A + band B) |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

- Rel-18 (multi-carrier enhancements)

Inter-band UL-CA without SUL band; Inter-band UL CA for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s); Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands; {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band}

- Rel-19

The switching scenario between LB CA cases are limited to Case 1 and Case 2 with band n5 and n29, respectively, which can also be described in Figure 1 below

3 Discussion on potential requirement impact

Go through all the discussions happened in Tx switching no matter in which release, the scope and timeline can be summarized as below:

- Start from RF requirement, then RRM requirement

  - The scope in RF session mainly contains: Time mask; Switching period including length and location; Timing issue for collocated deployment of the UL carriers; UE capabilities and RRC signaling for Tx switching, the RRC signalling can be used to indicate the switching period location and length; Applicability of DL interruption; UL outage due to switching; Other RF requirements

  - The scope in RRM session mainly contains: DL interruption requirements for different duplex mode combinations due to UL switching, the DL interruption requirements include interruption length, granularity and location; Capabilities for UEs with and without DL interruption; Test case

    - To define the switching requirement, MRTD and MTTD were considered

Based on the analyses above, from our understanding, although the research direction is very different (Tx switching VS LB CA switching), at least the following issues are common, and can be discussed in RF and RRM session of LB CA switching, respectively:

- From RF requirement perspective: Switching period; New UE capabilities and RRC signalling related to switching period; Applicability of interruption

- From RRM requirement perspective: Interruption requirements due to switching; Capabilities for UEs with and without interruption

Proposal 1: At least the following requirements discussed in Tx switching can also be focused on in LB CA switching

- From RF requirement perspective: Switching period; New UE capabilities and RRC signalling related to switching period; Applicability of interruption

- From RRM requirement perspective:  Interruption requirements due to switching; Capabilities for UEs with and without interruption

While, based on the WID, RAN1 will define semi-static switching pattern (s) based on RRC configuration, considering the flexibility of the pattern, and the mechanism to trigger the LB carrier switching

Proposal 2: It is necessary to discuss interruption requirements under different switching pattern

Proposal 3: RAN4 to define RRC based switching delay requirements between two LB carriers

Furthermore, from our understanding, before the switching happens, it is highly possible that the SDL SCell was configured. Besides, since co-located and synchronized network deployment for both carriers is considered, in theory, NW can activate SCell with short delay without using SSB of the to-be-activated SCell since it can be sure the actual receiving time difference from UE perspective of the Cells won’t be larger than CP. In this sense,

Proposal 4: RAN4 to discuss whether SSB-less SCell activation delay requirement can be applied to LB CA switching scenario

4 Conclusion

In this contribution, we provide our initial views focusing on RRM requirements for LB CA switching.  Following observations and proposals were derived.

Proposal 1: At least the following requirements discussed in Tx switching can also be focused on in LB CA switching

- From RF requirement perspective: Switching period; New UE capabilities and RRC signalling related to switching period; Applicability of interruption

- From RRM requirement perspective:  Interruption requirements due to switching; Capabilities for UEs with and without interruption

Proposal 2: It is necessary to discuss interruption requirements under different switching pattern

Proposal 3: RAN4 to define RRC based switching delay requirements between two LB carriers

Proposal 4: RAN4 to discuss whether SSB-less SCell activation delay requirement can be applied to LB CA switching scenario

5 Reference

[1] 	RP-243317, New WID on low band carrier aggregation via switching, RAN#106