3GPP TSG-RAN WG4 Meeting #113																	R4-2501699

Orlando, US, 18th – 22nd November, 2024

Title:	Text proposal on 7125 – 8400 MHz IMT parameters in TR 38.922

Source:	Qualcomm Incorporated, Nokia, Ericsson

Agenda item:	7.2.2

Document for:	Approval

# Introduction

In RAN4#113, the RF parameters for the different frequency ranges were nearly finalized. Nevertheless, the SI got extended till March’25 plenary to allow additional time to finalize the TR. In this contribution, we provide additional text proposal on different aspects for the 7125 – 8400 MHz frequency range

******************Begin changed sections******************

# 5	7125 - 8400 MHz frequency range

## 5.2	BS parameters

#### 5.2.1.2	Spectral mask

Both Category A and B spectral mask are specified. Since the frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104, it is proposed that existing spectral mask (Category B) for band n104 in TS 38.104 subclause 6.6.4 for conducted requirements and subclause 9.7.4.2 for radiated requirements are applicable for the range. Category A limits for Wide Area BS are specified in Table 5.2.1.2-1 for non-AAS BS and in Table 5.2.1.2-2 for AAS BS. In addition, more information is given in Annex A.2 on the BS spectral mask for channel bandwidths higher than the typical 100MHz (e.g., 200MHz).

******************Next Change******************

#### 5.2.1.3	ACLR

It is agreed to re-use n104 ACLR. The ACLR should be higher than the value specified in Table 5.2.1.3-1. In addition, it has been shown in Annex A.1 that the value given in Table 5.2.1.3-1 can be reused for higher than 100MHz channel bandwidth (e.g., 200MHz).

******************Next Change******************

#### 5.2.1.5	Maximum output power

The maximum base station output power/ sector (e.i.r.p.) was provided in the antenna parameter table of the LS [5], as extracted in Table 5.2.1.5-1. It was agreed to be aligned with antenna characteristics. Note that the values given in Table 5.2.1.5-1 represent the maximum typical values of the BS output power.

Table 5.2.1.5-1: Maximum typical BS output power in 7125 to 8400 MHz

|  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban |
| --- | --- | --- | --- |
| Maximum typical base station output power/sector (e.i.r.p.) (dBm) | 78.3 | 78.3 | 61.5 |

******************Next Change******************

#### 5.2.2.4	ACS

It is agreed to specify 42 dB.  In addition, it has been shown in Annex A.1 that the same value can be reused for higher than 100MHz channel bandwidth (e.g., 200MHz).

******************Next Change******************

#### 5.3.1.2	Spectral mask

The UE spectral mask is described in Table 5.3.1.2-1. In addition, more information is given in Annex A.2 on the UE spectral mask for channel bandwidths higher than the typical 100MHz (e.g., 200MHz).

******************Next Change******************

#### 5.3.1.3	ACLR

According to the previous studies and simulation results in TR 38.921 [14] sub-clause 4.3, it was concluded that 26 dB ACLR would be sufficient for 6.425 - 7.125 GHz. Thus, ACLR of 26dB can be considered for the frequency range 7.125 – 8.4 GHz. In addition, it has been shown in Annex A.1 such ACLR value can be reused for higher than 100MHz channel bandwidth (e.g., 200MHz).

The actual ACLR for this frequency range or parts there-of should be further studied in the WI phase.

******************Next Change******************

#### 5.3.2.4	ACS

According to the previous studies and simulation results in TR 38.921 [14] sub-clause 4.3, adjacent channel selectivity (ACS) is agreed as 32 dBc for 6425 – 7125 MHz.  Thus, ACS of 32dB can be considered for the frequency range 7.125 – 8.4 GHz. In addition, it has been shown in Annex A.1 that the same value can be reused for higher than 100MHz channel bandwidth (e.g., 200MHz).

The actual ACS for this frequency range or parts there-of should be further studied in the WI phase.

******************Next Change******************

## 5.4	Antenna characteristics

### 5.4.1	BS antenna characteristics

#### 5.4.1.2	Antenna parameters

The typical BS antenna parameters are listed in Table 5.4.1.2-1.

******************End changed sections******************