3GPP TSG-RAN WG4 Meeting #114	R4-2501701

# Athens, GR, 17th – 21st Feburary, 2025

Agenda Item:	7.10.2.3.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN HPUE TX RF requirements

Document for:	Approval

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable high power UE (HPUE) with defining RF requirements targeting at support of IoT-NTN UEs for FR1 single carrier scenario.

In this contribution, our views about HPUE TX RF requirements for IoT-NTN UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

In RAN4#113 meeting, the agreed WF [2] provides the following works for specifying TX requirements for IoT-NTN under HPUE operation. The partial contents are shown as follows.

NTN HPUE UE RF – General & Feasibility Issue 1-2-1: Discuss the unsettled requirements applicability for IoT-NTN: - Agreement (Ad Hoc): - No 2Tx PC1/PC2 requirement will be specified for IoT NTN in Rel-19 - Only consider non-handheld for PC1 1Tx eMTC based with full duplex IoT-NTN - FFS whether requirements should be defined for PC1 1Tx handheld UE for NB-IoT based NTN Table: Current status for requirements consideration and feasibility for IoT-NTN Power Class; Antenna Numbers; Handheld and/or non-handheld. NB-IoT based; PC1; 1Tx; FFS. PC2; 1Tx; Both. eMTC based with half duplex; PC1; 1Tx; FFS. PC2; 1Tx; Both. eMTC based with full duplex; PC1; 1Tx; Non-handheld. PC2; 1Tx; Both. NTN HPUE UE RF – Tx requirements Issue 1-1-8: The nominal MOP tolerance Background: HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP. From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread. As for IoT-NTN eMTC and NB1/NB2, the actual NTN-IoT PC2 HD-FDD TX output power upper bound could be 28.8dBm (i.e., 26.8dBm+2dB) but exceeds conventional PC2 TX upper power bound of 28dBm. From ETSI EN 301 908, PC3 TX output power upper bound can be specified higher (i.e., 23dBm +2.7dB or 23dBm +3dB). - Proposals - As for IoT-NTN eMTC and NB1/NB2, regarding solution for PC2 HD-FDD TX MOP upper bound higher than FD-FDD TX MOP upper bound, consider a specific or general solution for all IoT-NTN bands. The solution is applicable for solving the same issue in PC3 and PC5. The options are as follows. - Option1: Add additional note to increase the upper tolerance for NTN HD-FDD operation in applicable regions. EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 26; +22, -2; 23; +22, -2; 20; +22, -2. 255; 26; +22, -2; 23; +22, -2; 20; +22, -2. 254; 26; +22, -2; 23; +22, -2; 20; +22, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When UEs recognizes and supports NTN HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD].. - Option 2: Add additional UE capability and NW signal to control NTN HD-FDD power increment. - Option 2-1: EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 26; +22, -2; 23; +22, -2; 20; +22, -2. 255; 26; +22, -2; 23; +22, -2; 20; +22, -2. 254; 26; +22, -2; 23; +22, -2; 20; +22, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB.. - Option 2-2: EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). 256; 262; +2, -2; 232; +2, -2; 202; +2, -2. 255; 262; +2, -2; 232; +2, -2; 202; +2, -2. 254; 262; +2, -2; 232; +2, -2; 202; +2, -2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: When IoT-NTN UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The reference power is increased by ΔPNTNHdFdd.. - Option 3: Add new power classes (e.g., PC4.5_NTN-HD-FDD, PC2.5_NTN-HD-FDD, PC1.75_NTN-HD-FDD). - Option 4: - Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE. - WF - FFS NTN HPUE UE RF – Rx requirements Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based) - Proposals - Option 1: 0dB for n254/n255/n256 (vivo, oppo) - Option 2: others - WF - [0] dB for n254/n255/n256 Issue 3-1-3: General for RSD Way forward: - Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

Observation 1: Based on evaluation results below, the PC1 1Tx handheld UE for supporting NTN NB-IoT seems feasible.

Pout=31dBm, 2G Mid-Band PA is used. 1610.7MHz; 1660MHz; 1674.3MHz; 1980.7MHz; 2019.3MHz. BPSK_3.75KHz Tone allocation 0/47; ACLR1; -38; -37.67; -37.03; -39.43; -40.82. BPSK_15KHz Tone allocation 0/11; ACLR1; -32.45; -32.18; -32.47; -34; -35.38. QPSK_3.75KHz Tone allocation 0/47; ACLR1; -41.78; -41.52; -41.78; -43.49; -44.71. QPSK_15KHz Tone allocation 0/11; ACLR1; -36.55; -36.3; -36.56; -38.2; -39.46. BPSK_3.75KHz; EVM; < 0.4%; < 0.4%; < 0.4%; < 0.4%; < 0.4%. BPSK_15KHz; EVM; < 0.4%; < 0.4%; < 0.4%; < 0.4%; < 0.4%. QPSK_3.75KHz; EVM; < 0.4%; < 0.4%; < 0.4%; < 0.4%; < 0.4%. QPSK_15KHz; EVM; < 0.4%; < 0.4%; < 0.4%; < 0.4%; < 0.4%.

Observation 2: Regarding using GSM 2GHz mid-band PA for NB-IoT NTN PC1, the evaluation measurement results show the single-tone PC1 ACLR1 value is better than -30dBc/200kHz. Thus, regarding at least B256, the single-tone PC1 ACLR could meet UTRAACLR of 37dBc without MPR.

Observation 3: The NTN NB1/NB2 SEM requirement is -5dBm/30kHz at +/-100kHz ΔfOOB frequency offset. By transferring PC1 ACLR1 of -28dBc/200kHz into -5.2dBm/30kHz, the SEM at +/-100kHz ΔfOOB frequency offset could be met when single-tone PC1 ACLR1 is lower than -28dBc/200kHz.

Proposal 1: Requirements for supporting PC1 1Tx NTN NB-IoT handheld UE should be defined.

It is unclear whether the potential solutions to mitigate SAR issue is for handheld UEs, non-handheld UEs, or both.

Observation 4: Regarding IoT-NTN HPUE, it is unclear whether the potential solutions to mitigate SAR issue is for handheld UEs, non-handheld UEs, or both.

Proposal 2: Regarding IoT-NTN HPUE, clarify whether the potential solutions to mitigate SAR issue is for handheld UEs, non-handheld UEs, or both.

Observation 5: The HD-FDD FE IL is around 0.8dB lower compared to FD-FDD IL and causes TX MOP difference when the same PA is used. If TX output power upper bound is not relaxed, there would be 0.8dB TX power backoff for NTN UE HD-FDD operation.

Proposal 3: Discuss possible options below for relaxing the nominal MOP or MOP tolerance upper limitation for NTN HD-FDD operation.

- Option 1:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26 | +2x, -2 | 23 | +2x, -2 |
| 255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26x | +2, -2 | 23x | +2, -2 |
| 255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 3: New sub power classes (e.g, PC2NTN-HD-FDD) to relax the TX output power upper limitation.

Observation 6: Regarding NTN FD-FDD eMTC, the PC1 TX interferer after duplexer rejection (e.g., 51dB rejection) has still quite high power (e.g., -20dBm) and could cause obvious RX desensitization.

Proposal 4: Because the PC1 TX interferer after duplexer rejection (e.g., -20dBm) is still high enough to cause RX desensitization, the RX RSD should not be 0dB for NTN PC1 FD-FDD eMTC for both handheld and non-handheld UEs.

3 Conclusion

In this paper, our views on HPUE TX RF requirements for IoT-NTN UE are provided as follows:

Observation 1: Based on evaluation results below, the PC1 1Tx handheld UE for supporting NTN NB-IoT seems feasible.

Pout=31dBm, 2G Mid-Band PA is used. 1610.7MHz; 1660MHz; 1674.3MHz; 1980.7MHz; 2019.3MHz. BPSK_3.75KHz Tone allocation 0/47; ACLR1; -38; -37.67; -37.03; -39.43; -40.82. BPSK_15KHz Tone allocation 0/11; ACLR1; -32.45; -32.18; -32.47; -34; -35.38. QPSK_3.75KHz Tone allocation 0/47; ACLR1; -41.78; -41.52; -41.78; -43.49; -44.71. QPSK_15KHz Tone allocation 0/11; ACLR1; -36.55; -36.3; -36.56; -38.2; -39.46. BPSK_3.75KHz; EVM; < 0.4%; < 0.4%; < 0.4%; < 0.4%; < 0.4%. BPSK_15KHz; EVM; < 0.4%; < 0.4%; < 0.4%; < 0.4%; < 0.4%. QPSK_3.75KHz; EVM; < 0.4%; < 0.4%; < 0.4%; < 0.4%; < 0.4%. QPSK_15KHz; EVM; < 0.4%; < 0.4%; < 0.4%; < 0.4%; < 0.4%.

Observation 2: Regarding using GSM 2GHz mid-band PA for NB-IoT NTN PC1, the evaluation measurement results show the single-tone PC1 ACLR1 value is better than -30dBc/200kHz. Thus, regarding at least B256, the single-tone PC1 ACLR could meet UTRAACLR of 37dBc without MPR.

Observation 3: The NTN NB1/NB2 SEM requirement is -5dBm/30kHz at +/-100kHz ΔfOOB frequency offset. By transferring PC1 ACLR1 of -28dBc/200kHz into -5.2dBm/30kHz, the SEM at +/-100kHz ΔfOOB frequency offset could be met when single-tone PC1 ACLR1 is lower than -28dBc/200kHz.

Proposal 1: Requirements for supporting PC1 1Tx NTN NB-IoT handheld UE should be defined.

Observation 4: Regarding IoT-NTN HPUE, it is unclear whether the potential solutions to mitigate SAR issue is for handheld UEs, non-handheld UEs, or both.

Proposal 2: Regarding IoT-NTN HPUE, clarify whether the potential solutions to mitigate SAR issue is for handheld UEs, non-handheld UEs, or both.

Observation 5: The HD-FDD FE IL is around 0.8dB lower compared to FD-FDD IL and causes TX MOP difference when the same PA is used. If TX output power upper bound is not relaxed, there would be 0.8dB TX power backoff for NTN UE HD-FDD operation.

Proposal 3: Discuss possible options below for relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation.

- Option 1: Relaxation of TX MOP tolerance upper limitation for NTN HD-FDD

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26 | +2x, -2 | 23 | +2x, -2 |
| 255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: Additional NW signal to control relaxation of TX MOP upper limitation for NTN HD-FDD.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26x | +2, -2 | 23x | +2, -2 |
| 255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 3: New sub power classes (e.g., PC2NTN-HD-FDD) to relax the TX output power upper limitation.

Observation 6: Regarding NTN FD-FDD eMTC, the PC1 TX interferer after duplexer rejection (e.g., 51dB rejection) has still quite high power (e.g., -20dBm) and could cause obvious RX desensitization.

Proposal 4: Because the PC1 TX interferer after duplexer rejection (e.g., -20dBm) is still high enough to cause RX desensitization, the RX RSD should not be 0dB for NTN PC1 FD-FDD eMTC for both handheld and non-handheld UEs.

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2420404, “WF on HPUE for IoT NTN”, Vivo