# 3GPP TSG-RAN WG4 Meeting #114		  R4-2501731

# Athens, Greece, 17– 21 February, 2025

Agenda item:			7.10.2.1

Source:	Qualcomm Incorporated

Title:	On coexistence study for NTN HPUE

Document for:	Approval

# Introduction

The new Rel-19 WI on enhanced requirements and test methodology for NR and IoT NTN was approved in [1] in RAN #103. In RAN4#113, coexistence study for NTN HPUE was discussed and the agreements were captured in WF [2]. The simulation results for NR NTN HPUE have been submitted in our paper [3] [4]. In this paper, we provide our further views on coexistence study for NTN HPUE.

# 2. Discussion

## 2.0 Isolation and Wrap-around network layout

The issues for Wrap-around and isolation distance have been discussed for several RAN4 meetings. Back to Rel-17 when the coexistence study for NR NTN was conducted, isolation distance was introduced in a late stage. The original assumption is to distribute all the NTN UEs at the boundary of TN coverage for all the 6 scenarios defined in TR 38.863 (see in Table 1). This assumption is based on that UE has both TN and NTN functionalities which means UE can handover between TN and NTN network with proper NTN-TN selection algorithm. Then it was identified that due to higher received power from TN network, boundary between TN and NTN should be far from the TN cell layout (bule circle in Figure 1) in the simulation. Therefore, we used criterion of equal UE RSRP received from TN and NTN to calculate a new boundary between TN and NTN networks, which is 1.5km isolation distance shown in Figure 2.

Table 1: Scenarios for NR NTN coexistence study in Rel-17

| Scenario No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. NTN UE ACS. |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. NTN SAN ACS. |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. NTN SAN ACLR. |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. NTN UE ACLR. |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. NTN UE ACLR. |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. NTN SAN ACS |

Figure 1. 0km distance between TN and NTN (applying for Scenario 2-6)

Figure 2: 1.5km distance between TN and NTN (only applying for Scenario 1)

In theory, 1.5km distance should be applied for all the 6 scenarios. While since 1.5km isolation distance discussion happened in a relatively late stage, it was agreed to only apply 1.5km isolation distance for Scenario 1.

Note that if NTN UEs drop into the coverage of TN, referring to [5], the required ACIR for Scenario 1 would be 68dB which is not possible to be implemented in both SAN and UE sides.

In addition, with the assumption that NTN UE can handover between TN and NTN network, wrap-around function should be disabled since there would not be any interference from TN cells outside the TN coverage. Otherwise, NTN UE should camp on TN cells rather than NTN cell.

Observation 1:  Following the same principle of UE distribution, 1.5km distance should be applied for all the 6 scenarios in Rel-17. While since 1.5km discussion happened in a relatively late stage, it was agreed to only apply 1.5km isolation distance for Scenario 1 in Rel-17.

Observation 2: With the assumption that NTN UE can handover between TN and NTN network, wrap-around function should be disabled since there would not be any interference from TN cells outside the TN coverage. Otherwise, NTN UE should camp on TN cells rather than NTN cell.

## 2.1 Simulation results for NR NTN HPUE

For the coexistence between TN and NTN HPUE, it was agreed to focus on scenario 4 as the worst case [2].

Issue 1-2: Worst case scenario Agreement: - RAN4 to focus on scenario 4, and to skip scenario 5

For NR-NTN HPUE, we have submitted the simulation results in [3] [4] and copied the simulation results in Table 1.

Note that in the below results, we did not consider 1.5km isolation distance. The wraparound functionality is disabled If the isolation distance is further considered, the Throughput loss will be further reduced.

Table 1 Required ACIR for Scenario 4 without wrap-around, no isolation distance for NR-NTN HPUE

| ACIR | 0 | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S4 avg –THP - no iso, WA disabled | 9.2 | 5.9 | 3.5 | 2 | 1.1 | 0.5 | 0.3 |
| S4 edge –THP - no iso WA disabled | 52.9 | 34.2 | 19.8 | 10.1 | 4.5 | 2.2 | 1.1 |

As shown in Table 1, when wrap-around is disabled, the required ACIR for Scenario 4 is 20dB even without 1.5km isolation distance.

Proposal 1: The ACLR of NR-NTN UE with PC3 can be reused for NR-NTN HPUE PC2, PC1.5 and PC1 from coexistence perspective.

Our companion paper [6] provided analysis from RF perspective, and have the following proposal:

Proposal 2: 31dB is applied for NR-NTN PC2, PC1.5 and PC1.

## 2.1 Simulation results for IoT-NTN HPUE

The simulation results for Scenario 4, IoT-NTN UL to TN UL with PC1 assumptions are provided in the Table 2. There are 2 cases for IoT-NTN simulations, one is with 12 UEs,15 kHz each and other one is with 36 UEs with 3.75 kHz each.

Table 2 Required ACIR for Scenario 4 without wrap-around, no isolation distance for IoT-NTN HPUE

| ACIR | 0 | 5 | 10 | 15 | 20 | 25 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S4 avg –THP - no iso WA disabled 12 UEs, 15kHz | 17.5 | 12.3 | 8 | 5.1 | 3 | 1.7 | 0.9 | - |
| S4 edge –THP - no iso WA disabled 12 UEs, 15kHz | 84 | 67.9 | 47.7 | 29.5 | 16.1 | 7.7 | 3.7 | - |
| S4 avg –THP - no iso WA disabled 36 UEs, 3.75 kHz | 22.9 | 16.8 | 11.7 | 7.7 | 4.8 | 2.8 | 1.5 | 1.2 |
| S4 edge –THP - no iso WA disabled 36 UEs, 3.75 kHz | 100 | 83.4 | 66.5 | 45 | 26 | 13.5 | 6.7 | 4.8 |

Note that, the flat ACLR model is used with FACLR = 0 dB, i.e. no BW scaling, which is same as ACLR modelling used in Rel-18 IoT-NTN coexistence study.

As shown in Table 2, when wrap-around is disabled, the required ACIR for Scenario 4 is about 30dB even without 1.5km isolation distance.

Proposal 3: The ACLR of IoT-NTN UE with PC3 can be reused for IoT-NTN HPUE PC2 and PC1 from coexistence perspective.

Our companion paper [7] provided analysis from RF perspective, and have the following proposal:

Proposal 4: As the starting point, re-use 37 dB UTRA ACLR for NB-IoT PC2 and PC1.

# 3.	Conclusion

In this paper, we provide our views on the coexistence study for NTN HPUE. The following proposals and observation are made:

Observation 1:  Following the same principle of UE distribution, 1.5km distance should be applied for all the 6 scenarios in Rel-17. While since 1.5km discussion happened in a relatively late stage, it was agreed to only apply 1.5km isolation distance for Scenario 1 in Rel-17.

Observation 2: With the assumption that NTN UE can handover between TN and NTN network, wrap-around function should be disabled since there would not be any interference from TN cells outside the TN coverage. Otherwise, NTN UE should camp on TN cells rather than NTN cell.

Proposal 1: The ACLR of NR-NTN UE with PC3 can be reused for NR-NTN HPUE PC2, PC1.5 and PC1 from coexistence perspective.

Proposal 2: 31dB is applied for NR-NTN PC2, PC1.5 and PC1.

Proposal 3: The ACLR of IoT-NTN UE with PC3 can be reused for IoT-NTN HPUE PC2 and PC1 from coexistence perspective.

Proposal 4: As the starting point, re-use 37 dB UTRA ACLR for NB-IoT PC2 and PC1.

# 4.	References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN

- R4-2420403, WF on NR NTN HPUE, Samsung

- R4-2418849, Discussion on coexistence study for NTN HPUE, Qualcomm

- R4-2415307, On coexistence study for NTN HPUE, Qualcomm

- R4-2112247, Coexistence simulation results for TN-NTN, Qualcomm

- R4-2502010, NR NTN high power UE Tx requirements, Qualcomm

- R4-2502008, IoT NTN high power UE Tx requirements, Qualcomm