3GPP TSG-RAN WG4 Meeting #114                                                                R4-2501736

Athens, Greece, February 17 – 21, 2025

Source: 	NTT DOCOMO, INC.

Title:	Clarification of modifiedMPR-behaviour for n1

Agenda item:	4.2.1

Document for:	Approval

- Introduction

Due to changes in the requirements for protecting PHS from n1, reevaluating A-MPR values for n1 is currently discussed. RAN4 agreed that it can use modifiedMPR-behaviour and reuse NS_05 and NS_05U instead of introducing new NS to introduce new smaller A-MPR for protecting DECT. Companies may have different understandings of how to describe modifiedMPR-behaviour in the specifications of each release. Therefore, this paper shares our views to clarify these.

- Background

Based on regulatory changes in Japan, it is necessary to protect DECT instead of PHS for n1 newly. The requirements for protection are relaxed, so RAN4 is currently discussing reevaluating A-MPR values for n1. Regarding NS value to be used to indicate the new A-MPR requirements, the reached agreement at the last meeting is captured in WF [1] as follows.

< Issue 1-1-3: NS value to be used to indicate the new A-MPR requirements >

Way Forward:

- Introduce new A-MPR in Rel-19 spec and reuse NS_05/NS_05U instead of introducing new NS.

  - UE can support it as optional before Rel-19.

NS_05 and NS_05U have been introduced since Rel-15 for protecting PHS from n1. Based on the above agreement, RAN4 can use modifiedMPR-behaviour and reuse NS_05 and NS_05U instead of introducing new NS to introduce new smaller A-MPR for protecting DECT. However, at the last meeting, there was a discussion about how to describe modifiedMPR-behaviour in the specifications of each release. Companies may have different understandings. Therefore, clarifying these points will help reuse existing NS and introduce the new smaller A-MPR for n1.

- Discussion

By checking the latest version of TS38.101-1 specifications for each release, the trend in how definitions of the bits in the field modifiedMPR-behaviour are described can be seen. As an example, the following is excerpt from the modifiedMPR table in TS38.101-1 v18.8.0.

Table 3-1: Excerpt from modifiedMPR table in TS38101-1 v18.8.0 (Table L.1-1: Definitions of the bits in the field modifiedMPR-Behaviour)

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n71 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
| n34 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |

First, column of NOTEs in modifiedMPR table basically uses boilerplate text. For example, it is “This bit shall be set to 1 by a UE supporting ...” for n71. On the other hand, it is “This bit may be set to 1 by a UE of any release supporting ...” for n34. In this case, it looks like "shall" is used if it is mandatory for the corresponding UE to meet the new MPR requirements, and it looks like "may" is used if it is optional.

Observation 1: In the table for the definitions of the bits in the field modifiedMPR-behaviour, "shall" is used if it is mandatory for UE to meet the new MPR requirements, and "may" is used if it is optional.

In addition, in this modifiedMPR table, it is possible to specify the behaviour of UEs of other releases and/or to refer to the requirements of other releases. For example, in the case of n71, the modifiedMPR table in Rel-18 TS38.101-1 refers to the requirements in Rel-15 TS38101-3. Also, in the case of n34, the modifiedMPR table in Rel-18 are applicable to UEs of any release.

Observation 2: In the table for the definitions of the bits in the field modifiedMPR-behaviour, it is possible to specify the behaviour of UEs of other releases and/or to refer to the requirements of other releases.

On the other hand, the confusion is whether the modifiedMPR table of the early release needs to be updated when the new MPR requirements of the later release are applied to the early release UEs. Based on the existing requirements, there seem to be two main cases. The first case is when the specifications in all releases related to the new MPR requirement are updated. For example, in the case of modifiedMPR-behaviour for n71, it is assumed that it was specified as mandatory in the Rel-16 specification, which was the latest release at the time, and was also specified as optional in the Rel-15 specification. The second case is when only the latest release of the specification at the time was updated. For example, in the case of modifiedMPR-behaviour for n34, only the modifiedMPR table in the Rel-18 specification was updated, but as stated in the specification, it is for the UEs of any release supporting power class 1.5, so it also applies to UEs of releases earlier than Rel-18.

In this contribution, the former will be called Case X, and the latter will be called Case Y. The following shows which case modifiedMPR-behaviour for each band in Rel-18 specification is likely to fall into. The requirements introduced in early releases seem to follow case X, and the requirements introduced in late releases seem to follow case Y.

Table 3-2: Classification of how the modifiedMPR table is written for each release

| NR Band | Index of field (bit number) | Rel-15 (Latest version) | Rel-16 (Latest version) | Rel-17 (Latest version) | Rel-18 (Latest version) | Case |
| --- | --- | --- | --- | --- | --- | --- |
| n30 | 0 (leftmost bit) | N/A2 | N/A2 | shall | shall | - |
| n34 | 0 (leftmost bit) | (may) 3 | (may) 3 | (may) 3 | may | Y |
| n39 | 0 (leftmost bit) | (may) 3 | (may) 3 | (may) 3 | may | Y |
| n40 | 0 (leftmost bit) | (may) 3 | (may) 3 | (may) 3 | may | Y |
| n41 | 0 (leftmost bit) | may | shall | shall | shall | X |
|  | 1 | may | shall | shall | shall | X |
|  | 2 | (may) 3 | may | may | may | X |
|  | 3 | (may) 3 | (may) 3 | may | may | Y |
| n71 | 0 (leftmost bit) | may | shall | shall | shall | X |
| n77 | 0 (leftmost bit) | (may) 3 | (may) 3 | may | may | Y |
| n78 | 0 (leftmost bit) | (may) 3 | (may) 3 | may | may | Y |
| n79 | 0 (leftmost bit) | (may) 3 | (may) 3 | may | may | Y |
| n96 | 0 (leftmost bit) | N/A1 | (may) 3 | N/A2 | may 4 | Y |
|  | 1 | N/A1 | (may) 3 | (may) 3 | may 4 | Y |
| n102 | 0 (leftmost bit) | N/A1 | (may) 3 | N/A2 | may 4 | Y |
|  | 1 | N/A1 | (may) 3 | (may) 3 | may 4 | Y |
| NOTE 1: If the modifiedMPR table has been updated in that release, it is indicated with an shall or may in this table. NOTE 2: ‘N/A’ means that a UE supporting this release of the specification cannot set this bit to 1. NOTE 3: ‘(may)’ means that a UE supporting this release of the specification may set this bit to 1, but there is no mention of this in the specification. NOTE 4: This means that a UE supporting this release of the specification cannot set this bit to 1, but modifiedMPR table is updated in the specification because it is latest release. |  |  |  |  |  |  |

Observation 3: There seem to be two main cases for existing requirements regarding modifiedMPR-behavior: either the specifications of all releases related to the new MPR requirements are updated, or only the latest (or specific) release at the time is updated and the behaviour applies to earlier release UEs as well.

Our view is Case X is preferable to Case Y for readability of the specification. If Case Y is followed, it is possible that neither the modified MPR table nor the new MPR requirements will be specified in the early release specification even if the early release UE can use modifiedMPR to meet the new MPR requirements. It should be avoided from the perspective of readability of the specification. As discussed in parallel in RAN4, the use of modifiedMPR is being considered for EESS protection from n257 and n258 [2]. In this case, it was agreed that supporting the new requirements and indication of the corresponding modifedMPR bit is mandatory even for early release UEs [3]. There is a concern about the risk of these being overlooked. Therefore, even if release-independent mechanisms such as band combos can be considered, Case X, which has clearer applicable spurious emission requirements, is preferable to Case Y.

Also, at the last meeting, there was a proposal to change the definitions of the bits in the field modifiedMPR-behaviour between releases. However, this is likely to make network control and specifications more complicated. For example, assuming that modifiedMPR bit 0 means old MPR for Rel-X UE but it does new MPR for Rel-Y UE, NW needs to check not only the bit but also release that the UE supports so that NW correctly understand the applicable MPR. Therefore, changing these definitions will not be preferred.

Observation 4: In some cases, such as EESS protection from n257 and n258, it is mandatory for early release UE to support new requirements and indicate the corresponding modifiedMPR bit, so it should be clearly mentioned which spurious emission requirements apply in early release specification.

Observation 5: If the definitions of the bits in the field modifiedMPR-behaviour is changed between releases, NW needs to check not only the bit but also release that the UE supports.

Proposal 1: For readability of the specification, the table for definitions of the bits in the field modifiedMPR-behaviour will be updated from Rel-15 specification.

Proposal 2: To avoid complicating NW control and specifications, the definitions of the bits in the field modifiedMPR-behaviour are not changed between releases.

- Conclusion

Here we summarize our proposals:

Observation 1: In the table for the definitions of the bits in the field modifiedMPR-behaviour, "shall" is used if it is mandatory for UE to meet the new MPR requirements, and "may" is used if it is optional.

Observation 2: In the table for the definitions of the bits in the field modifiedMPR-behaviour, it is possible to specify the behaviour of UEs of other releases and/or to refer to the requirements of other releases.

Observation 3: There seem to be two main cases for existing requirements regarding modifiedMPR-behavior: either the specifications of all releases related to the new MPR requirements are updated, or only the latest (or specific) release at the time is updated and the behaviour applies to earlier release UEs as well.

Observation 4: In some cases, such as EESS protection from n257 and n258, it is mandatory for early release UE to support new requirements, so it should be clear which spurious emission requirements apply.

Observation 5: If the definitions of the bits in the field modifiedMPR-behaviour is changed between releases, NW needs to check not only the bit but also release that the UE supports.

Proposal 1: For readability of the specification, the table for definitions of the bits in the field modifiedMPR-behaviour will be updated from Rel-15 specification.

Proposal 2: To avoid complicating NW control and specifications, the definitions of the bits in the field modifiedMPR-behaviour are not changed between releases.

References

- R4-2420481, “WF on PHS protection update”, KDDI, OPPO, RAN4#113

- RP-242611, “Revised WID: mmWave in NR: UE spurious emissions and EESS (Earth Exploration Satellite Service) protection”, China Unicom, RAN#106

- R4-2420356, “WF on UE RF requirements for EESS protection”, Huawei, China Unicom, RAN4#113