3GPP TSG-RAN4 Meeting #114	R4-2501813

# Athens, GR, 17th – 21st February, 2025

Title:	Discussion on system parameters of NTN less than 5MHz

Source:	Samsung

Agenda item:	7.10.3.1

Document for:	Approval

1	Introduction

In the RAN4#113 meeting, the less than 5MHz work split was agreed, and we have prepared a draft CR for section 5 system parameters to this meeting, in a separate document.

The WF[1] of last meeting, there are still several open issues remaining in the system parameters section. And in this paper, we will further express our views over those open issues.

2	Discussions

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

Issue 1-2: UE capability Signalling

Issue 1-3: If 3MHz is a mandatory feature, the UE behaviour to support 3MHz Channel bandwidth

For above mentioned issues, we are with the following views:

3MHz CHBW was introduced with punctured PBCH (12 RB). The sync raster location for 3MHz CHBW is separate compared to normal sync raster with other CHBWs.

Supporting 3MHz CHBW is required additional UE processing complexity in terms of initial cell search on new sync raster, baseband processing on punctured PBCH/control channel, RF capability of supporting 3MHz CHBW.

For TN specification, UE feature list (51-1/-2) were introduced for supporting 3MHz CHBW and 12 RB coreset #0 under NR_FR1_lessthan_5MHz_BW WI. And in TN specification, supporting 3MHz CHBW was agreed as optional feature as per band basis.

Supporting this feature can be release independent to Rel-18 given 3MHz PBCH channel was introduced in Rel-18. For mandatory or optional in Rel-19, we prefer to be optional meanwhile we can consider to mandate per band basis in RAN4 specification if strong demands justified.

| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2] for TN FR1 bands and clause 5.3.6 of TS 38.101-5 [x] for FR1 NTN bands. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2] for FR1 TN bands and clause 5.3.6 of TS 38.101-5 [x] for FR1 NTN bands, this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2] for FR1 TN bands and 5.4.3.3-X in TS 38.101-5 [x] for FR1 NTN bands. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |

Regarding UE capability, either extend existing UE feature list/capability signalling (51-1, 51-2) to cover NTN bands as per band basis or new separate UE feature list can be introduced for NTN 3MHz CHBW similar as TN. We believe extending existing UE feature list shall be more proper way given this feature was introduced by RAN1 with basic physical layer update which for NTN operation we reuse the design from TN without any changes on RAN1.

Proposal 1: Support 3MHz CHBW for NR-NTN in FR1-NTN bands is optional feature with capability signalling as per FR1 NTN band basis in general.

- Further discuss whether can be mandatory on certain bands with Rel-19 onwards as per band basis if strong needs justified.

Proposal 2: Extend existing UE capability signaling “support3MHz-ChannelBW-Symmetric-r18” and “support3MHz-ChannelBW-Asymmetric-r18” to cover NTN FR1 bands.

3	Conclusion

In this contribution, we provide views on remaining open issues for supporting 3MHz.

Proposal 1: Support 3MHz CHBW for NR-NTN in FR1-NTN bands is optional feature with capability signalling as per FR1 NTN band basis in general.

- Further discuss whether this feature can be mandatory on certain bands with Rel-19 onwards as per band basis if strong needs justified.

Proposal 2: Extend existing UE capability signaling “support3MHz-ChannelBW-Symmetric-r18” and “support3MHz-ChannelBW-Asymmetric-r18” to cover NTN FR1 bands.

4	Reference

[1] R4-2419789, Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi