3GPP TSG-RAN4 Meeting #114	R4-2501814

# Athens, GR, 17th – 21st February, 2025

Title:	Discussion on Ku-band coex study

Source:	Samsung

Agenda item:	7.11.2

Document for:	Approval

1	Introduction

In the RAN#106 meeting, the WID of introduction of Ku bands for NR NTN was revised and approved in [2], where it further incorporate the RF requirements for mobile VSAT served by NGSO and GSO into the scope. The changes are as follows:

2	Discussion

In last meeting WF[1], most co-ex study assumptions are agreed and with single option. And in this section, we will share our observations over some of the items.

Sub-topic 2-4: VSAT Max Tx antenna gain Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type. The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

For the VSAT polarization converting, it was captured in [1], that the NTN VSAT uses circular polarization while GEO satellite uses linear polarization. And the converting from circular to linear polarization is proposed in WF. We would like to seek clarifications on how this converting to be reflected in the co-ex study simulation, whether we apply a polarization loss or gain over the wanted signal link between VSAT and GEO?

Proposal 1: To clarify whether and how to reflect the polarization converting from circular to linear for the link between VSAT and GEO.

Sub-topic 2-9: TN BS/UE antenna patterns for 11 GHz Agree to use TN BS and UE antenna and beam forming pattern modelling in TS 38.921 for 11GHz carrier frequency for Ku band coexistence study Sub-topic 2-10: TN BS antenna pattern for 14 GHz Agree to use TN BS antenna and beam forming pattern modelling in TS 38.922 for 14GHz carrier frequency for Ku band coexistence study. For TN UE, the isotropic antenna model will be used as mentioned in TS 38.922.

There’s a potential difference in TN UE between 11GHz and 14GHz, we wonder whether we could simply align the assumptions between these two. In current WF, the TN UE in 11GHz assumes beamforming pattern, while the TN UE in higher 14GHz frequency assumes isotropic antenna model.

For simplification purpose, and not to impact other WI or TR discussion, we are wondering if it is possible for us to simply assume same beamforming or same omni-directional for both 11GHz and 14GHz band?

Proposal 2: To clarify if 11GHz and 14GHz TN UE antenna model can be simplified and unified to isotropic antenna model in TS 38.922.

And, the TN parameters for 11GHz and 14GHz were newly adopted in 3GPP RAN4 TR, and it was not really used in previous NTN studies. Some of the critical deployment parameters, including antenna size, cell size are new to the NTN coex study scenarios. It could be better to establish a offline activity for calibration.

Observation 1: Due to the newly studied technical characteristics from TN 11GHz and 14GHz, it would be beneficial to have calibration phase among the co-ex contributors, and it can be an offline activity between the meetings.

3	Conclusion

In this contribution, we provide views on co-ex study for Ku bands NTN:

Proposal 1: To clarify whether and how to reflect the polarization converting from circular to linear for the link between VSAT and GEO.

Proposal 2: To clarify if 11GHz and 14GHz TN UE antenna model can be simplified and unified to isotropic antenna model in TS 38.922.

Observation 1: Due to the newly studied technical characteristics from TN 11GHz and 14GHz, it would be beneficial to have calibration phase among the co-ex contributors, and it can be an offline activity between the meetings.

4	Reference

[1] R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[2] RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT