3GPP TSG-RAN WG4 Meeting #114	R4-2501828
Athens, Greece, 17th - 21st February, 2025

Agenda item:	7.14.2.3

Source:	CMCC

Title:	On dynamic MIMO OTA channel models PDP target values

Document for:	Approval

# 1	Introduction

In RAN4#113 meeting, we have given the contribution of dynamic MIMO OTA channel models target values [1], and it is can be observed that regarding PDP target values, in the case of the segment that the same CDL channel model throughout the segment, i.e., segment 1,2,6,7, and 9 in UMa channel model and segment 2,6, and 7 in UMi channel model, the difference between CMCC and CE vendors is relatively large, some values even exceed 2 dB. In this contribution, we give the comparison results of different processing methods regarding PDP target values.

# 2	Discussion

We adopt four processing methods of the original PDP. The first method converts the PDP to CFR for a 200 MHz bandwidth without any window filtering. The second method converts the PDP to CFR for a 200 MHz bandwidth with a Hanning window filter applied. The last two methods directly add the clusters within 3 ns and 5 ns resolutions in the time domain, respectively. It is important to note that the last approach is consistent with the method agreed upon in the static MIMO OTA channel models in Rel-17. The comparison results between our approach and those from CE vendors are as follows. For this contribution, we use the UMa channel model segment-6 as an example.

- Convert PDP to CFR for 200MHz BW without any window filtering

Figure 2.1 PDP for UMa channel model at segment 6 without Window filtering

- Convert PDP to CFR for 200MHz BW with Hanning window filtering

Figure 2.2 PDP for UMa channel model at segment 6 with Hanning Window filtering

Figure 2.3.1 PDP for UMa channel model at segment 6 with Hanning Window filtering (with Original Curve)

Figure 2.3.2 PDP for UMa channel model at segment 6 with Hanning Window filtering (2295ns and 2415ns parts leak phenomenon with Original Curve)

- Directly sum up clusters within 3ns resolution in the time domain

Figure 2.4 PDP for directly summing up clusters within 3ns resolution in the time domain

- Directly sum up clusters within 5ns resolution in the time domain

Figure 2.5 PDP for directly summing up clusters within 5ns resolution in the time domain

Observation 1: When converting the PDP to CFR and directly summing up in the time domain, it is noted that the trends of the PDP target values for CMCC and CE vendors are significantly different, which is particularly evident when using Method 2 and Method 4.

Observation 2：Due to the limited delay resolution, the power of the converted PDP curve might leak to the adjacent 5-ns grid, i.e., 2295ns and 2415ns, as represented in Figure 2.3.2, contributing to the random functions of the obtained PDP. Moreover, the peaks at 80 ns and 235 ns of PDP curve contains 4 clusters. It can be observed that the PDP curve at these two clusters is much wider than that of others. How to obtain the powers of these two clusters? directly by the peak or integration of the PDP curve?

Proposal 1: The handling methods of the initial PDP value significantly affects the final target value results. Hence, it is recommended that CE vendors provide the original PDP without any processing related to the delay resolution during this target value alignment stage. This approach will help ensure the accuracy and consistency of the original PDP target values.

Proposal 2：For the handling methods of PDP target values with the 5ns resolution problem, the method should be aligned with the static MIMO OTA channel models as agreed upon in Release 17, considering different CEs will have different filter impacts in the actual validation process.

# 3	Conclusion

Observation 1: When converting the PDP to CFR and directly summing up in the time domain, it is noted that the trends of the PDP target values for CMCC and CE vendors are significantly different, which is particularly evident when using Method 2 and Method 4.

Observation 2：Due to the limited delay resolution, the power of the converted PDP curve might leak to the adjacent 5-ns grid, i.e., 2295ns and 2415ns, as represented in Figure 2.3.2, contributing to the random functions of the obtained PDP. Moreover, the peaks at 80 ns and 235 ns of PDP curve contains 4 clusters. It can be observed that the PDP curve at these two clusters is much wider than that of others. How to obtain the powers of these two clusters? directly by the peak or integration of the PDP curve?

Proposal 1: The handling methods of the initial PDP value significantly affects the final target value results. Hence, it is recommended that CE vendors provide the original PDP without any processing related to the delay resolution during this target value alignment stage. This approach will help ensure the accuracy and consistency of the original PDP target values.

Proposal 2：For the handling methods of PDP target values with the 5ns resolution problem, the method should be aligned with the static MIMO OTA channel models as agreed upon in Release 17, considering different CEs will have different filter impacts in the actual validation process.

# References

- R4- 2418745, Reference target values for FR1 UMa and UMi dynamic channel models, CMCC, RAN4 #113, Nov. 2024.