3GPP TSG-RAN WG4 Meeting #114	R4-2501836

# Athens, Greece, 17th – 21st Feb, 2025

Title: 	On MIMO layer evaluation for 6Rx UE

Source: 	Huawei, HiSilicon

Agenda item:	7.1.1.3.2

Document for:	Discussion

Background

In last meeting, WF [1] was approved, one objective is MIMO layers for 6Rx. This contribution provides our views.

- Simulation results

Simulation assumptions are captured in Table 2.1-1:

Table 2.1-1: Simulation assumptions for MIMO layers evaluation for 6Rx

| Assumptions |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30kHz |
| Antenna configuration |  | 8T6R |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6} |
| MCS |  | Adaptive MCS |
| TxEVM |  | 3% |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8T6R SVD for 32T6R |
| PDSCH DMRS configuration | DMRS Type | Rel-15 Type 1 |
|  | DMRS Configurations | Option 1: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 2: Rel-15 1+1(4L), Rel-15 2+2(6L) |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8T6R SVD for 32T6R |
| UE Correlation matrix |  | Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 |
| BS correlation matrix |  | Option 1: 3GPP MIMO correlation matrix with Alpha = 0.0 |
| RF impairment |  | With or without RF impairment. For cases with RF impairment, include phase noise, IQ imbalance and LNA non-linearity impact. |

Figure 2-1 shows results for correlation matrixes option 2(From Huawei), the first sub-diagram compares the 4 layers, DMRS1+1 with 6 layers, DMRS 2+2, which is the most common case in the real network. It can be observed that 4 layers always outperforms 6 layers even when SNR is extremely high.

Second sub-diagram compares 4 layers, DMRS1+1 with 6 layers, DMRS 2+2 from the baseband perspective only, it can be observed that when SNR is higher than 30dB, 6 layers starts to outperform 4 layers. The maximum gain of 6 layers over 4 layers is 7.1%.

Last sub-diagram compares 4 layers, DMRS 1+1 with 6layers, DMRS 2+2 with RF impairment included. Three kinds of RF impairment are considered: phase noise, IQ imbalance and LNA non-linearity impact, it can be observed that when SNR is higher than 33dB, 6 layers starts to outperform 4 layers. The maximum gain of 6 layers over 4 layers is 6.7%. The switch SNR point increase by 3dB with RF impairment included.\

Observation 1: With RF impairment included (phase noise, IQ imbalance and LNA non-linearity impact), compared to case without RF impairment, SNR point increase by 3dB and maximum gain of 6 layers over 4layers is reduced.

Figure 2-1: Simulation results for 8T6R for UE Correlation matrix Option 2

Figure 2-2 shows results for correlation matrixes option 3(from Samsung), RF impairment is included. Similar observation as UE correlation matrixes option 2, 6layers have a chance to outperform 4 layers only if DMRS 2+0 is configured and the corresponding switch SNR point is 34dB.

Figure 2-2: Simulation results for 8T6R for UE Correlation matrix Option 3

Next we check the antenna efficiency impact, the simulation results are shown in Figure 2-3 and antenna efficiency is explicitly modeled in the simulation as per [2]. It can be observed that even with same DMRS overhead, 4 layers always outperforms 6layers.

Figure 2-3: Simulation results for 8T6R for UE Correlation matrix Option 3

Observation 2: 4 layers always outperforms 6layers with antenna efficiency considered.

Cover different UE correlation matrix, the following general observations are made:

Observation 3: 6layers outperforms 4layers only when DMRS overhead are aligned and SNR is higher than 34dB

- Discussions

UE type

According to WI[1], the requirements should target at handled UE and the specified requirements can be applicable to both UE and FWA devices.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

Therefore, FWA should be excluded when evaluating the feasibility of 6 MIMO layers. FWA already support 8Rx with 8 layers and are available on market, it’s very meaningless to discuss the feasibility of 6 layers. Hence we propose the following: Whether to introduce the 6 layers capability depends on the outcome of 6 MIMO layers evaluation on handled UE.

Observation 4: WI specifies that the target UE type for 6 MIMO layers evaluation is handled UE.

Proposal 1: RAN4 shall conclude that whether to introduce 6 layers for both FWA and handheld UE depends on the outcome of 6 MIMO layers evaluation on handled UE.

Rank

It is noted that Rank 5 is one candidate option, however, according to the WI, rank5 is out of the scope, we propose to only focus on 6layers and 4layers.

Proposal 2: RAN4 shall only focus on Rank4 and 6, not consider Rank5.

Number of Tx ports

We don’t support 32 ports+ SVD precoder. As we discussed in last meeting, SVD based precoder needs SRS transmission from UE side, which is quite challenge for UE. Furthermore, supporting large number of CSI-RS port and SRS switching are optional feature needing UE capability and how to handle large insertion loss between across antennas is a big issue. Also, SRS based precoder is not supported by all BSs.

Proposal 3: RAN4 shall use 8Tx+Single panel I codebook feedback for 6Rx 6layers evaluation.

DMRS configuration

DMRS overhead is a key factor that affects performance. It’s mentioned that Rel-15 DMRS type 1 is the most common configuration in the real deployment. eDMRS is introduced in Rel-18 and has not been supported by UE and BS vendors currently. The application of this feature is not clear in the future, so it is too early to couple 6layers with eDMRS in current stage. Feasibility evaluation should be based on the simulation assumptions which has been widely used in the real deployment. Rel-18 eDMRS can be used for evaluation in the future release if commonly implemented by most UE vendors. Also, based on our investigation, additional DMRS is always configured to guarantee the channel estimation accuracy, so DMRS 2+0 should be excluded.

Proposal 4: In Rel-19, RAN4 to use Rel-15 DMRS type 1 for 6Rx MIMO layers evaluation, DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6.

TxEVM

TxEVM is one open issue, following options were made in last meeting:

Issue 3-1-3: Tx EVM for feasibility study Way Forward: FFS on Tx EVM values to be used for feasibility evaluation. RAN4 to further discuss the following: - Option 1: Use Tx EVM = 1% - Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation - Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM - Option 4: use the existing BS Tx EVM core requirement - Option 5: The representative BS Tx EVM values provided by the BS vendor.

We support Option3, which is always used for demodulation requirements definition and has been already relaxed compared to core requirement. Using lower value will relax the BS requirements and different BS vendor may have different Tx EVM, which is hard to be unified.

Proposal 5: RAN4 shall use 6% for 64QAM and 3% for 256QAM

RxEVM/RF impairment

According to observation1, RF impairment highly impact performance, which is never considered in RAN4 discussion. Hence, it’s proposed to further discuss what kind of RF impairment should be considered and how to explicitly model them in the simulation. From our side, at least three kind of RF impairment should be considered: phase noise, IQ imbalance and LNA non-linearity impact.

Proposal 6: RAN4 to further discuss what kind of RF impairment should be considered how to explicitly model it into simulation.

Antenna efficiency

Antenna efficiency is also one potential factor degrading 6layer performance. [2] gives a proposal how to model the antenna efficiency when deriving UE correlation matrix, however, RAN4 haven’t reached a consensus on the modelling method. RAN4 shall further discuss the method and exact antenna efficiency values.

Proposal 7: RAN4 to further discuss modelling method of antenna efficiency and exact antenna efficiency values.

Concerns on foldable UE

People may think that the foldable UE could have more space to accommodate 6Rx , but we have different views on this with the following concerns.:

- One should note that the foldable phone is not always unfolded, which could depend on unpredictable factors e.g. user preference and hand blockage. When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- From receiving power perspective, if the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- Actually the hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

In conclusion, we have the following proposals for the foldable UE assumed for the 6 layers feasibility study.

Proposal 8: For the foldable UE assumed for the 6 layers feasibility study, the following aspects should be considered for the 6 MIMO evaluation with additional 1~2dB margin:

- When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- If the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- The hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

Complexity analysis

Besides performance gain, baseband complexity analysis and power consumption should also be considered. To support 2CWs, , which means complexity of decoder is doubled. Besides, dimension of channel matrix is increased from 4x6 to 6x6, the complexity of MIMO detector is also much increased, which is the module with the highest complexity increase. Besides, complexity of channel estimation and DMRS based interference estimation is also increased by 1.5 times when number DMRS ports is increased from 4 to 6. So, we strongly encourage to take baseband processing into account in addition to consider performance gains. It is not worthwhile to implement 6Layers after trading off the complexity and performance gain.

Proposal 9: RAN4 shall analyse the increased baseband complexity and power consumption for 6Layers compared to 4Layers including LDPC decoder, channel estimation, DMRS based interference estimation and MIMO detector.

Performance analysis

Based on our simulation results in clause 2, 6 layers outperforms 4layers only in some corner cases: Besides the DMRS configuration with same DMRS overhead, SNR is higher 34dB, it’s very rare to happen in the real deployment. Furthermore, the peak performance gain is quite limited 10%, it’s not reasonable to introduce 6 layers in R19.

Proposal 10: Don’t introduce 6 layers for 6Rx in Rel-19.

- Conclusion

This contribution provides our views on spatial channel model for demodulation requirements definition, the observations and proposals are:

Observation 1: With RF impairment included (phase noise, IQ imbalance and LNA non-linearity impact), compared to case without RF impairment, SNR point increase by 3dB and maximum gain of 6 layers over 4layers is reduced.

Observation 2: 4 layers always outperforms 6layers with antenna efficiency considered.

Observation 3: 6layers outperforms 4layers only when DMRS overhead are aligned and SNR is higher than 34dB

Observation 4: WI specifies that the target UE type for 6 MIMO layers evaluation is handled UE.

Proposal 1: RAN4 shall conclude that whether to introduce 6 layers for both FWA and handheld UE depends on the outcome of 6 MIMO layers evaluation on handled UE.

Proposal 2: RAN4 shall only focus on Rank4 and 6, not consider Rank5.

Proposal 3: RAN4 shall use 8Tx+Single panel I codebook feedback for 6Rx 6layers evaluation.

Proposal 4: In Rel-19, RAN4 to use Rel-15 DMRS type 1 for 6Rx MIMO layers evaluation, DMRS 1+1 for Rank4 and DMRS 2+2 for Rank6.

Proposal 5: RAN4 shall use 6% for 64QAM and 3% for 256QAM

Proposal 6: RAN4 to further discuss what kind of RF impairment should be considered how to explicitly model it into simulation.

Proposal 7: RAN4 to further discuss modelling method of antenna efficiency and exact antenna efficiency values.

Proposal 8: For the foldable UE assumed for the 6 layers feasibility study, the following aspects should be considered for the 6 MIMO evaluation with additional 1~2dB margin:

- When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- If the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- The hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

Proposal 9: RAN4 shall analyse the increased baseband complexity and power consumption for 6Layers compared to 4Layers including LDPC decoder, channel estimation, DMRS based interference estimation and MIMO detector.

Proposal 10: Don’t introduce 6 layers for 6Rx in Rel-19.

- Reference

[1]  R4-2420376 WF on 6Rx UE requirements.

[2]  R4-2417561 Feasibility evaluation of MIMO layer for 6Rx UE. MediaTek Inc.