3GPP TSG-RAN WG4 Meeting #114                                         R4-2501841

Athens, Greece, February 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On potentially new requirements for SBFD

Agenda Item:	7.23.2.1

Document for:	Approval

Introduction

During RAN4#113, potentially new requirements for SBFD have been discussed. Following conclusions and FFS points are reproduced from [1].

##### Transient period requirement applicability on frequency/antenna Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases. ##### Location of transient period between SBFD and non-SBFD Way Forward: - FFS whether the transmitter transient period shall be located within the SBFD symbol or not: - FFS it is an implementation issue and no RAN4 standard impact ##### Necessity of in-channel adjacent subband leakage ratio requirements Agreement: - New requirement for in-channel adjacent subband leakage ratio should be defined - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline. ##### Necessity of in-channel adjacent subband blocking requirement Agreement: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration. ##### Necessity of in-channel adjacent subband selectivity requirements Agreement: - No necessity to introduce the new requirement for in-channel adjacent subband selectivity. ##### Necessity of new requirement for inter-sector and inter-site interference Agreement: - No necessity to introduce the new requirement for inter-sector interference.

In this contribution, we would like to share our views regarding this topic.

Discussion

The requirement for transient period for SBFD

In previous meeting, a conclusion on this issue was achieved. While there will be transient period between SBFD and non-SBFD slot, the remaining issue is to determine where the transient period will be. Our preference is to select SBFD slot to accommodate the transient period for the necessary RF resource rearrangement, so the TDD slot can be intact.

Proposal 1: For the transient period between SBFD and non-SBFD slot, suggest that it should be located within the SBFD slot/symbol.

In-channel adjacent subband requirement for SBFD

For in-channel adjacent subband leakage ratio it was agreed that new requirement for it should be defined. During Rel-18 SI phase, no specific conclusion has been achieved but the value of legacy ACLR requirement was used as simulation assumption. So from our understanding, one can start with figuring out whether the value of existing ACLR requirement can be used as baseline. One thing that might need further consideration is whether we need to distinguish between this requirement of DU and DUD case.

Proposal 2: Suggest to use the value of existing ACLR requirement as starting point for in-channel adjacent subband leakage ratio.

- Whether to use the power sum of two DL subbands for DUD case should be further discussed.

Regarding in-channel adjacent subband blocking, it has been agreed that new requirement is needed. But the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes should also be considered. To our understanding, such DL sub-band interference might cause some nonlinearity to LNA or other RF components but that interference is anyway coming from the same operator so some methods e.g. beam nulling/coordination can be applied in order to mitigate the interference.

Observation 1: Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

Following simulation based on our co-ex simulation platform is conducted to provide some inputs for further discussion, note that CLI handling is NOT included.

Figure 1. CDF of in-channel adjacent subband blocking w/o CLI handling

According to the result, it is -28.03 dBm @99%-tile. Further discussion on how to combine the CLI handling with this interference level should be further discussed.

Observation 2: How to accommodate the CLI handling capability with the in-channel adjacent subband interference should be further discussed.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observation and proposals:

Observation 1: Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

Observation 2: How to accommodate the CLI handling capability with the in-channel adjacent subband interference should be further discussed.

Proposal 1: For the transient period between SBFD and non-SBFD slot, suggest that it should be located within the SBFD slot/symbol.

Proposal 2: Suggest to use the value of existing ACLR requirement as starting point for in-channel adjacent subband leakage ratio.

- Whether to use the power sum of two DL subbands for DUD case should be further discussed.

- References

- R4-2419908, “Way Forward for [113][306] NR_duplex_evo_BSRF”, Samsung, 3GPP TSG-RAN4 Meeting #113, Orlando, USA, November 18-22, 2024.