3GPP TSG-RAN WG4 Meeting #114 	R4-2501854

Athens, Greece, Feb 17 – 21, 2025

Source: 	Huawei

Title: 	OTA test case reduction

Agenda Item:	7.13.4.2

Document for:	Approval

1 Introduction

New WID on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at RAN#103 meeting. The work item concentrates on 2 work areas where the current BS specification may be optimised, i.e. Expected EIRP mask for upper 6GHz and OTA test enhancement. One of major identified RF requirements with the aim to shorten the test time is TX IM.

The considerations on test case reduction for TX IM test reduction was discussed in RAN4#113 [1] and the WF is captured in [2]. In this contribution

2 Test scope reduction for Tx requirement

At RAN4#113 meeting, for TX test case reduction it is agreed to focus on Tx IMD requirement and further discuss the applicable interferer offsets and test configurations.

Issue 1-1: test scope reduction for Tx requirement

- Recommended WF:

  - Focus on test scope reduction for Tx IMD requirement

  - Further discuss the applicable interference freq offset for ACLR, OBUE and spurious emission requirements;

  - Further discuss the test scope reduction of test configuration.

2.1	Interferer offsets

6 interferer offsets are tested for each and 12 if AAS supports NC operation, these offsets date back to the UTRA requirements and have been carried through for each subsequent RAT however not they are generating such excessive test time so it is appropriate to see if they are all still necessary or can be reduced.

Table 6.8.5.1-1 in 38.141-2: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

For each side test 3 offsets (1xCBW offset, 2xCBW offset, 3xCBW offset) started in UMTS specification and continued through LTE, MSR and NR. In the following text we look at each case respectively.

ACLR

UMTS has ACLR1 and ACLR2 requirement, and ACLR2 is 5 dB harder. LTE and NR effectivity have an ACLR1 and ACLR2 requirement but they are both the same. hence the 1xCBW offset interferer and the 1st adjacent channel can be assumed to be the worst case.

Observation 1: For LTE and NR ACLR, the 1xCBW offset is the worst case so other offsets are not needed.

OBUE

LTE/NR OBUE is generally considered that in the 1st adjacent channel the ALCR requirement dominates and as you move away from the carrier the OBUE is tougher. They have a tougher requirement beyond 10 MHz. As we are testing at M only we can pretty safely say 3xCBW case is probably the toughest.

Observation 2: For OBUE, the 3xCBW offset is the worst case so other offsets are not needed.

Spurious emissions

Spurious emissions are applied 10MHz outside the operating band. The only test case which can generate 3rd order products in SE domain is the 3xCBW case, and this is only if the wanted signal is places on the band edge. As we only test TX IM at middle there are very few bands where we will ever see 3rd order products in the SE domain. If so only the 3xCBW offset seems worth testing.

Observation 3: For spurious emissions, only the 3xCBW offset is needed.

Proposal 1: During TX IMD test, it is proposed to use 1xCBW offset for ACLR test and 3xCBW offset to check OBUE and spurious emissions limits. The following update is made for the test requirements.

Table 6.8.5.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | For OTA ACLR requirements in clause 6.7.3, n=1. For OTA OBUE in clause 6.7.4 and OTA spurious emission in clause 6.7.5, n=3. |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

2.2	Test configurations

For NR TX IMD test, NRTC1is applicable to contiguous spectrum capable BS and NRTC3 is applicable to C and NC capable BS with identical parameters, and both NRTC1 and NRTC3 are applicable to C and NC capable BS with different parameters. Meanwhile for AAS TS 37.145-2, even for C and NC capable BS with identical parameters, both ATCR7 and ANTCR7 are applied. If follow the same logic, only ANTCR7 is needed for the case.

Proposal 2: for AAS TS 37.145-2, only ANTCR7 is needed for C and NC capable BS with identical parameters, so ATCR7 can be removed from the tests.

3 Out of band blocking

At RAN4#113 meeting, for RX test case reduction it is agreed to focus on out of band blocking and focus on the measurement step size for Rx OOBB requirement.

Issue 1-2: Out of band blocking

- Recommended WF:

  - Focus on test scope reduction for Rx OOBB requirement

  - Focus on the measurement step size for Rx OOBB requirement;

RAN4 had similar discussion for FR2 in Rel-15 and finally agreed to reduce the amount of time by increasing the interferer step size. The less than 1/3 BW approach is adopted when define the measurement step.

Table 7.6.4.2.3-1: Interferer signal step size for FR2-1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000 | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |

For FR1 similar approach can be considered. And the frequency range can be divided into 1 to 1000 MHz, 1000 MHz to 3000 MHz, 3000 MHz to 6000 MHz and above 6000 MHz.

Observation 4: 1/3 BW approach can also apply to FR1 and the frequency range can be divided into 1 to 1000 MHz, 1000 MHz to 3000 MHz, 3000 MHz to 6000 MHz and above 6000 MHz.

4	Summary

Proposal 1: During TX IMD test, it is proposed to use 1xCBW offset for ACLR test and 3xCBW offset to check OBUE and spurious emissions limits. The following update is made for the test requirements.

Table 6.8.5.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | For OTA ACLR requirements in clause 6.7.3, n=1. For OTA OBUE in clause 6.7.4 and OTA spurious emission in clause 6.7.5, n=3. |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

Proposal 2: for AAS TS 37.145-2, only ANTCR7 is needed for C and NC capable BS with identical parameters, so ATCR7 can be removed from the tests.

3	Reference

[1] R4-2419243, Test case reduction for TX IM, Huawei

[2] R4-2419828, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation