3GPP TSG-RAN WG4 Meeting#114                                R4-2501867

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.24.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussions on General aspect for A-IoT

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

Discussion

- 2.1 General aspects for A-IoT BS in D1T1

- First of all, only D1T1-B was agreed in the WID which means CW would be outside of topology deployment. During the RAN#106 meeting, how to capture the RF requirement of CW node in the specification is still not clear. Currently, there are the following options to specify the RF requirement for CW node:

- Option 1: to be captured in BS specification;

- Option 2: to be captured in the Device specification

- Option 3: separate specification for CW node

- Proposal 1: propose RAN4 to discuss how to capture the RF requirement of CW node in RAN4 specification.

- In addition, as indicated in the current WID, only in-band operation mode and standalone operation mode is supported, from our understanding, the definition of operation mode is defined for R2D transmission/reception and D2R reflection. For frequency position to place the CW node is not clearly clarified. Since the device 1 could also support some frequency shift compared with operating frequency of CW node, in other words, even though CW signal is placed within the guard band, then it is still possible to have D2R reflection signal to be within in-band spectrum of the coexisting NR carrier.

- Proposal 2: RAN4 need to clarify the applicable frequency position within NR carrier if R2D/D2R is in-band operation mode.

- For A-IoT BS in the D1T1-B mode, BS is expected to operate in the half duplex mode. For the legacy TDD BS, there would be some guard period between DL/UL transmission to avoid the cross link interference, however for FDD BS with half duplex operation, this kind of interference mitigation consideration is not needed. For transition period between DL and UL is mainly used to allow for transition time for ON-OFF and OFF-ON of RF transmitter and receiver hardware. Another issue different from the legacy TDD system, the decoding time after the R2D transmission should be considered to design the guard period between R2D transmission and D2R reception.

- Proposal 3: propose RAN4 to discuss both ON-OFF/OFF-ON transition time (e.g. 10us transition time) and also the decoding time (e.g. 400us or 500us decoding time) for R2D reception at device side for the appropriate guard period configuration.

Figure 1. the illustration of D1T1-B

Figure 2. consideration for the the timeline for R2D and D2R

- Regarding the operation frequency, it is indicated in the WID that band n8 would be used as example band, from our understanding, other NR bands close to band n8 is not precluded, however this is also dependent on the operator market demand. From A-IoT BS perspective, NR BS is capable to operate at both band n8 and band n5. From A-IoT device perspective, as far as we know, device 1 is capable to operate from 800MHz to 1000MHz, the applicable frequency range is very wide to accommodate lots of operating bands, however the exact operating frequency could be limited into 20MHz considering antenna radiation performance. In other words, device need to have some antenna retunning capability to support the large applicable frequency range and the exact operating bandwidth is limited within 20MHz.

Figure 3. the candidate example bands for A-IoT operation

Proposal 4: propose RAN4 to discuss the supported frequency range at device side to further determine the supported NR bands for A-IoT system.

- Regarding the transmission power for A-IoT BS and CW node, during the Rel-19 coexistence study, for transmission power for R2D transmission power with single PRB is assumed as 33dBm and CW signal transmission is assumed as 23dBm. However considering the potential minimum PRB configurations for R2D transmission as listed in the following table, the transmission power of A-IoT BS as 33dBm assumed in the coexistence study is limited. In addition, the applicable BS class for A-IoT BS is also assumed as Micro BS/medium range in SI phase which is specified with maximum transmission power as 38dBm in the current specification.

- Proposal 5: to consider the applicable BS type for A-IoT BS as Medium range BS with maximum transmission power as 38dBm.

- Table 1. potential PRB configuration for R2D transmission

| - M | - R2D - (1/2 Manchester code, kbps) | - Minimum PRB configuration for R2D |
| --- | --- | --- |
| - 1 | - 7 | - 1 |
| - 2 | - 14 | - 1 |
| - 4 | - 28 | - 1 |
| - 6 | - 42 | - 1 |
| - 8 | - 56 | - 2 |
| - 12 | - 84 | - 2 |
| - 16 | - 112 | - 2 |
| - 24 | - 168 | - 3 |
| - 32 | - 224 | - 4 |

- Regarding the transmission power for CW node, in the coexistence study during the SI phase, it was agreed to assume the transmission power as 23dBm with D2R transmission bandwidth as 15KHz if CW signal in the uplink direction. However considering the R2D data rate and its corresponding D2R bandwidth, CW signal with 23dBm is too limited. Considering the typical power class-4 of GSM UE transmission in 900MHz in the past, there should be no regulation limitation for 33dBm CW transmission power. In short, we propose to consider the maximum transmission power for CW signal as 33dBm.

- Proposal 5: consider the CW signal with maximum transmission power as 33dBm.

- Table 2. potential bandwidth for D2R reflection

| - D2R bandwidth (kHz) | - Chip length (us) | - D2R - (1/2 Manchester code, kbps) |
| --- | --- | --- |
| - 15 | - 133.3 | - 3.75 |
| - 30 | - 66.67 | - 7.5 |
| - 60 | - 33.33 | - 15 |
| - 90 | - 22.22 | - 22.5 |
| - ... | - ... | - ... |
| - 1440 | - 1.389 | - 360 |
| - 1920 | - 1.042 | - 480 |
| - 2160 | - 0.926 | - 540 |
| - 2700 | - 0.741 | - 675 |

- For the system parameters for R2D and D2R transmission, it’s expected to have more discussions on it. For R2D transmission, from our initial understanding, the minimum channel bandwidth is supposed to be 4PRB with 15KHz based on the potential minimum PRB configuration for R2D and additional guard band on each side (e.g. 1/18 of R2D transmission bandwidth which is the same as LTE spectrum utilization) to define the ACLR and OBUE requirement.

- Proposal 6: propose to define the minimum carrier bandwidth (e.g. 0.8MHz) with 4PRB with 15KHz and additional guard band on each side (e.g. 1/18 of R2D transmission bandwidth which is the same as LTE spectrum utilization).

Figure 4a. the illustration of minimum carrier bandwidth for R2D transmission

- In addition, considering other potential R2D coding rate to be specified by RAN1 or requested from SA1 traffic demand, other candidate channel bandwidth could be also considered.

Figure 4b. the illustration of other carrier bandwidth for R2D transmission

- Proposal 7: propose to discuss other carrier bandwidth for R2D transmission until RAN1 has draw the conclusion for the maximum data rate.

Figure 5. the illustration of OOK signal generation for R2D [Other REs is assumed as 0 for Rel-19 A-IoT BS transmission]

- Regarding the carrier bandwidth for D2R transmission, as clarified in table 2, different D2R bandwidth might need be specified according to the corresponding D2R data rate, however considering the potential FDM operation between different devices, then carrier bandwidth could be larger than D2R bandwidth per device. In addition, D2R signal generation (e.g. OOK based or BPSK based approach) is different compared with R2D signal generation, therefore the PRB configuration or sub-carrier spacing related concept cannot be reused for D2R transmission from our understanding.

- Proposal 8: propose not to reuse the PRB configuration or sub-carrier spacing related concept for D2R transmission.

Figure 6. the illustration of OOK signal generation for D2R signal [freq error of D2R transmission should be considered within GB]

- Proposal 9: propose to discuss the carrier bandwidth for D2R until RAN1 has draw the conclusion for the maximum data rate.

- Regarding the channel raster for A-IoT service, if the standalone spectrum is allocated for A-IoT R2D transmission, then channel raster for R2D transmission should be at least 100KHz. If the in-band spectrum is allocated for A-IoT R2D transmission, then channel raster could be defined as following:

- N*100kHz channel raster+delta_offset

- where offset could be -47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5KHz

Figure 7. the illustration of OOK signal generation for D2R signal [freq error of D2R transmission should be considered within GB]

- Proposal 10: for R2D transmission in Standalone operation mode, to define the channel raster as 100KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [-47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5]KHz.

- Regarding the channel raster for D2R transmission, due to the poor frequency stability performance of the device 1, this might be very difficult to maintain the 100KHz freq channel raster as R2D transmission from BS side.

- Proposal 11: for D2R transmission, propose to further discuss the channel raster especially considering the poor frequency stability performance.

Conclusions

In this contribution, we shared some initial views on the RF requirement for A-IoT BS in D1T1 and proposals are made as following:

- Proposal 1: propose RAN4 to discuss how to capture the RF requirement of CW node in RAN4 specification.

- Proposal 2: RAN4 need to clarify the applicable frequency position within NR carrier if R2D/D2R is in-band operation mode.

- Proposal 3: propose RAN4 to discuss both ON-OFF/OFF-ON transition time (e.g. 10us transition time) and also the decoding time (e.g. 400us or 500us decoding time) for R2D reception at device side for the appropriate guard period configuration.

Proposal 4: propose RAN4 to discuss the supported frequency range at device side to further determine the supported NR bands for A-IoT system.

- Proposal 5: consider the CW signal with maximum transmission power as 33dBm.

- Proposal 6: propose to define the minimum carrier bandwidth (e.g. 0.8MHz) with 4PRB with 15KHz and additional guard band on each side (e.g. 1/18 of R2D transmission bandwidth which is the same as LTE spectrum utilization).

- Proposal 7: propose to discuss other carrier bandwidth for R2D transmission until RAN1 has draw the conclusion for the maximum data rate.

- Proposal 8: propose not to reuse the PRB configuration or sub-carrier spacing related concept for D2R transmission.

- Proposal 9: propose to discuss the carrier bandwidth for D2R until RAN1 has draw the conclusion for the maximum data rate.

- Proposal 10: for R2D transmission in Standalone operation mode, to define the channel raster as 100KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [-47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5]KHz.

- Proposal 11: for D2R transmission, propose to further discuss the channel raster especially considering the poor frequency stability performance.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282, RF requirements for Ambient IoT BS, Huawei,

- [4] R4-2417088, WF on RF requirement impact of A-IoT, Huawei, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Huawei, approved.

- R4-2410597,WF on impacts of A-IoT on RF requirements, Approved.

- RP-243326, New WID on Rel-19 Ambient IoT, Approved.

- TR 38.769,Study on solutions for Ambient IoT (Internet of Things) in NR