3GPP TSG-RAN WG4 Meeting#114                                R4-2501868

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.24.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussions on RF requirements for A-IoT BS

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

Discussion

- 2.1 RF requirement impacts for A-IoT BS

- Table 2.1-1. The initial analysis for RF requirement of A-IoT BS in D1T1.

| Tx part |  |
| --- | --- |
| Base station output power | Agreement in TR38.769: Base station output power requirement is potentially needed, RAN4 agrees to use current requirements of Micro NR BS type 1-C as the starting point. Status: Complete |
| Output power dynamics | Agreement in TR38.769: Output power dynamic requirement usually includes RE power control dynamic range requirement and total power dynamic range requirement. Power boosting requirement is not needed in R19 with the assumption that ambient IoT BS only transmits and receives AIOT signal. Status: RE power control and total dynamic range power is still open. For RE power control requirement, this is mainly used for FR1 to mitigate the inter-cell co-channel interference from scheduling granularity perspective, however for R2D transmission in OOK modulation with PRB scheduling perspective, we don’t see the necessity to define the requirement for it. Proposal 1: propose not to define the RE power control requirement for A-IoT BS; For the total dynamic range requirement, the PRB configuration for R2D transmission is still under the discussion. According to the following potential minimum PRB configuration for R2D transmission as agreed in RAN1, the total dynamic range requirement is still needed. Proposal 2: propose to define the total dynamic range requirement for A-IoT BS according to the agreed PRB configuration for R2D transmission in RAN1. |
| Transmit ON/OFF power | Agreement in TR38.769: Regarding the transmit on/off requirement, it usually includes of transmitter off power requirement and transmitter transient period requirement, RAN4 agrees to further discuss in the WI phase. Status: transmit ON/OFF power and transition period requirement is still open. - For A-IoT BS in the D1T1-B mode, BS is expected to operate in the half duplex mode. For the legacy TDD BS, there would be some guard period between DL/UL transmission to avoid the cross link interference, however for FDD BS with half duplex operation, this kind of interference mitigation consideration is not needed. For transition period between DL and UL is mainly used to allow for transition time for ON-OFF and OFF-ON of RF transmitter and receiver hardware. Another issue different from the legacy TDD system, the decoding time after the R2D transmission should be considered to design the guard period between R2D transmission and D2R reception. Proposal 3: propose RAN4 to discuss both ON-OFF/OFF-ON transition time (e.g. 10us transition time) and also the decoding time (e.g. 400us or 500us decoding time) for R2D reception at device side for the appropriate guard period configuration. |
| Transmitted signal quality | Agreement in TR38.769: Frequency error and modulation quality (EVM) are potentially needed. TAE is only applicable for BS transmitting from multiple antennas via MIMO, CA, or combination of them. A-IoT BS doesn’t support these features, so TAE requirement is not needed. Status: freq error and EVM requirement are still open. The legacy transmit frequency error requirement could be reused for A-IoT BS. Proposal 4: propose to reuse the frequency error requirement of FR1 MR BS for A-IoT BS; The legacy EVM for OOK signal is not needed since OOK signal is not mapped by the legacy constellation, instead it’s reflected in the envelope level as following figure. Tag also don’t need to have channel estimation and equalization to decode the OOK signal for Device 1, instead of the power stability or power accuracy for OOK ON signal and OOK OFF signal is important and also power difference between OOK ON and OOK OFF is essential to ensure the tag OOK detection performance. Figure 2.4-1. time domain waveform for OOK-4 In addition, as pointed out in [3], RFID uses ASK modulation with PIE data encoding. The ASK modulation mask was defined as shown in Figure 2.4-2 below. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements. Figure 2-4-2: Interrogator-to-Tag RF envelope For A-IoT BS EVM requirement, we propose to define the EVM requirement similar as RFID with OOK signal considered. Proposal 5: propose to define the EVM requirement for A-IoT BS using similar method as for RFID. |
| OBW | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. Since OBW requirement is the regulatory requirement, so the legacy OBW requirement could be reused for A-IoT BS; Proposal 6: propose to reuse the legacy OBW requirement for A-IoT BS R2D transmission. |
| Tx intermodulation | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. Based on the some discussion in other WIDs, it seems that Tx intermodulation requirement is kind of regulatory requirement from the co-existence perspective, therefore we propose still to define this requirement. Proposal 7: propose to reuse the FR1 MR Tx intermodulation requirement for A-IoT BS. |
| ACLR | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. Based on the initial simulation results, 45dBc ACLR requirement should be sufficient. Proposal 8: propose to define ACLR requirement up to the coexistence study (e.g. 38dBm A-IoT BS transmission power). |
| Operating band unwanted emissions | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. This depends on the conclusion of ACLR and carrier bandwidth requirement etc. Proposal 9: propose to define OBUE requirement up to the ACLR requirement and carrier bandwidth requirement. |
| Transmitter spurious emissions | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. The Tx transmitter spurious emission requirement is the regulatory requirement, therefore we propose still to reuse the current FR1 requirement for it. The legacy transmitter spurious emission requirement could be reused since this is coming from the ITU regulatory definition. Proposal 10: propose to reuse the FR1 BS transmitter spurious emission requirement for A-IoT BS |
| Rx part |  |
| REFSENS | Agreement in TR38.769: Regarding the reference sensitivity level for A-IoT BS, RAN4 reached the following consensus: - For BS type 1-C, PREFSENSdBm=−174dBm+10×log10(BW)+NF+IM+SNR+[ desens target] - BW is the transmission bandwidth for D2R. - NF is the noise figure of A-IoT BS, equal to 10 dB for Medium Range BS. - IM is the implementation margin, equal to 2dB. - SNR is the SNR value for D2R decoding, the detailed metrics would be discussed in the WI phase. - Desens target is considered for CW signal interference cancellation in D1T1 scenario. The desens level for CW signal cancellation might be different for D1T1-A1/D1T1-A2 and D1T1-B. For D1T1-A2, CW signal interference is coming from self-interference and also potential CW interference from other nodes transmitting CW signal. For D1T1-A1, the CW signal interference is coming from other nodes transmitting CW inside the topology. For D1T1-B, the CW signal interference is coming from CW nodes transmitting outside topology. Status: open [simulation results will be provided later on.] For D2R transmission bandwidth, this is still under the discussion in RAN1. For modulation for D2R transmission, whether this is OOK signal or BPSK modulation, this is not decided yet. Meanwhile whether CRC other coding scheme is to be specified for D2R transmission is also not decided yet. In short, it is premature to discuss the test metric for REFSENS requirement from BW, SNR; Proposal 11: propose to postpone the REFSENS requirement discussion until RAN1 reach the consensus. In addition, regarding the desens due to the CW transmission, this also need more discussion since CW signal is outside of topology in D1T1-B, after the A-IoT receive the CW signal, then CFO estimation and following other CW cancellation technique (e..g high pass filtering or digital IC etc ) to remove the received CW signal. Proposal 12: propose to further discuss the maximum input power for CW signal in D1T1-B scenario and potential maximum desens due to the CW reception. |
| Dynamic range requirement | Agreement in TR38.769: For NR and LTE, receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. The interference level is derived from simulation of co-channel interference from the same operator. It measures the effects of base station receiver impairments. For Ambient IoT indoor deployment, the co-channel interference comes from co-channel device transmission and CW interference. The necessity and the value can be studied at WI phase. Status: open [simulation results will be provided later on.] Proposal 13: propose to further discuss the co-channel interference coming from co-channel device transmission and CW interference for dynamic range requirement in D1T1-B scenario . |
| ACS, | Agreement in TR38.769: Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase. Status: open [simulation results will be provided later on.] Based on the initial simulation results, 46dBc ACS requirement might be problematic for SINR degradation of A-IoT BS receiver performance. Maybe some guard band could be provided to mitigate the in-band emissions from NR UE UL transmission towards the A-IoT BS receiver. Proposal 14: propose to further discuss ACS requirement up to the coexistence study. NR UE UL interference in the indoor case is not concerning one if NR UE could cell re-select to other freq or Redirecton to other frequency. |
| Blocking requirement | Agreement in TR38.769: Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase. Status: open [simulation results will be provided later on.] Proposal 15: for IBB requirement, propose to specify the requirement up to the co-existence study. |
| OOBB | Agreement in TR38.769: Regarding Out-of-band blocking requirement, the existing out-of-band blocking requirement may be still applicable for A-IoT BS except for the wanted signal level. Don’t see the reason not to reuse the -15dBm CW signal as interference signal of OOBB requirement. For f_OOBB requirement, this could be further discussed once we have more clear assumption on A-IoT BS. Status: open Proposal 16: propose the reuse FR1 MR BS requirement in the licensed band for A-IoT BS OOBB requirement. |
| RX IMD | Agreement in TR38.769: Regarding the general intermodulation requirement, it is agreed to postpone to WI phase due to the potential relationship between D2R operation assumption e.g. single tone or multiple tone D2R assumption. Regarding the narrowband intermodulation requirement, it is potentially necessary due to the potential Narrow band blocking at the operating frequency range, however its detailed requirement could be postponed to WI phase. This is somehow similar as Tx intermodulation requirement. Proposal 17: propose the reuse FR1 MR BS Rx IMD requirement in the licensed band for A-IoT BS Rx IMD requirement. |
| Rx spurious emission | Agreement in TR38.769: Regarding the receiver spurious emission requirement, this requirement might differ as following: - For D1T1-A1 and D1T1-B, the receiver spurious emission requirement is still needed due to is half duplex operation mode. - For D1TA-A2, due to the simultaneous transmission of CW signal and D2R reception, if the spurious emission requirement of CW signal is exceeding the receiver spurious emission requirement, then the receiver spurious requirement is not needed similar as FDD BS otherwise this requirement is still needed to regulate the receiver RF performance. Proposal 18: propose to reuse FR1 legacy receiver spurious emission requirement for A-IoT BS. |
| ICS | Agreement in TR38.769: Regarding the receiver in-channel selectivity requirement, this is agreed to be discussed in WI phase. We don’t see the necessity to have this requirement for standalone A-IoT BS or in-band/guard band operation with the individual RF hardware similar as standalone NB-IoT ICS requirement. Proposal 19: propose not to define the ICS requirement for A-IoT BS. |

Conclusions

In this contribution, we shared some further views on the RF requirement for A-IoT BS in D1T1 and proposals are made as following:

Proposal 1: propose not to define the RE power control requirement for A-IoT BS;

Proposal 2: propose to define the total dynamic range requirement for A-IoT BS according to the agreed PRB configuration for R2D transmission in RAN1.

Proposal 3: propose RAN4 to discuss both ON-OFF/OFF-ON transition time (e.g. 10us transition time) and also the decoding time (e.g. 400us or 500us decoding time) for R2D reception at device side for the appropriate guard period configuration.

Proposal 4: propose to reuse the frequency error requirement of FR1 MR BS for A-IoT BS;

Proposal 5: propose to define the EVM requirement for A-IoT BS using similar method as for RFID.

Proposal 6: propose to reuse the legacy OBW requirement for A-IoT BS R2D transmission.

Proposal 7: propose to reuse the FR1 MR Tx intermodulation requirement for A-IoT BS.

Proposal 8: propose to define ACLR requirement up to the coexistence study (e.g. 38dBm A-IoT BS transmission power).

Proposal 9: propose to define OBUE requirement up to the ACLR requirement and carrier bandwidth requirement.

Proposal 10: propose to reuse the FR1 BS transmitter spurious emission requirement for A-IoT BS.

Proposal 11: propose to postpone the REFSENS requirement discussion until RAN1 reach the consensus.

Proposal 12: propose to further discuss the maximum input power for CW signal in D1T1-B scenario and potential maximum desens due to the CW reception.

Proposal 13: propose to further discuss the co-channel interference coming from co-channel device transmission and CW interference for dynamic range requirement in D1T1-B scenario

Proposal 14: propose to further discuss ACS requirement up to the coexistence study. NR UE UL interference in the indoor case is not concerning one if NR UE could cell re-select to other freq or Redirecton to other frequency.

Proposal 15: for IBB requirement, propose to specify the requirement up to the co-existence study.

Proposal 16: propose the reuse FR1 MR BS requirement in the licensed band for A-IoT BS OOBB requirement.

Proposal 17: propose the reuse FR1 MR BS Rx IMD requirement in the licensed band for A-IoT BS Rx IMD requirement.

Proposal 18: propose to reuse FR1 legacy receiver spurious emission requirement for A-IoT BS.

Proposal 19: propose not to define the ICS requirement for A-IoT BS.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282, RF requirements for Ambient IoT BS, Huawei,

- [4] R4-2417088, WF on RF requirement impact of A-IoT, Huawei, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Huawei, approved.

- R4-2410597,WF on impacts of A-IoT on RF requirements, Approved.

- RP-243326, New WID on Rel-19 Ambient IoT, Approved.

- TR 38.769,Study on solutions for Ambient IoT (Internet of Things) in NR