3GPP TSG-RAN WG4 Meeting#114                                R4-2501871

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.12.1.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for Inter-band CA for ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we would like to share some views on the MSD issue for n39 UL transmission in CA_n3A-n39A.

Issue 2-1: whether to allow n39 UL for CA_n3A-n39A Agreement: - The conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE. - Further check if the performance of these parameters in the following table can be achieved in ATG UE with 5MHz frequency gap in n39 filter. - Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; 50dB. n39 Tx filter rejection at 1805~1880MHz; 50dB. - Other RF architectures or parameters are not precluded. Issue 2-2: BCS for DL CA_n3_n39 WF: Further consider the following table, if the feasibility is confirmed for uplink of n39. Table 1. Supported bandwidths per CA band combination of band n3+n39 NR CA configuration; Uplink CA configuration or single uplink carrier; NR Band; Channel bandwidth (MHz); Bandwidth combination set. CA_n3A-n39A; n3, [n39]; n3; 5, 10, 15, 20, 25, 30; 0. n39; 5, 10, 15, 20, 25, 30, 35, 40. Issue 2-3: MSD for CA_n3A-n39A with n39 UL WF: - Further check the MSD value with 5MHz frequency gap in 1880~1885MHz, if the feasibility is confirmed for uplink of n39. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1827.5; 5; [TBD]; ACLR2. - Other test points are not precluded.

Discussion

- 2.1. RF requirements for Inter-band CA

In this section, we provide some further analysis for DL CA_n3_n39 with UL band 39 transmission. Based on the reported UE capability for ATG UE ranging from 23dBm to 40dBm, the detailed MSD analysis are provided as following in Table 1 and Table 2.

Table 1. MSD analysis for ATG UE supporting inter-band DL CA with band 39 UL transmission as 23dBm

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n39, dBm |  | 23 | 23 |
| Tx band n39 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 received signal at ANT port, dBm |  | 23 | 3 |
| n3 filter rejection at 1885~1915MHz, dB | 50 |  |  |
| signal After n3 filter, dBm |  | -31 | -51 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -152 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -148 |
| noise figure dB | 12 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162 |  |  |
| Rx band n3 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.47 | -95.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n39 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -34.77 |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -28.24 |  |
| n39 Tx filter rejection at 1805~1880MHz, dB | 50 |  |  |
| n39 PA noise power at Rx n3 ant port at 1825~1830MHz, dBm |  | -78.24 | -98.24 |
| Total noise level at ANT port |  | -78.15 | -93.63 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -79.15 | -94.63 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -92.25 |  |  |
| MSD | 4.75 |  |  |

Table 2. MSD analysis for ATG UE supporting inter-band DL CA with band 39 UL transmission as 40dBm

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n39, dBm |  | 40 | 40 |
| Tx band n39 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 received signal at ANT port, dBm |  | 40 | 20 |
| n3 filter rejection at 1885~1915MHz, dB | 50 |  |  |
| signal After n3 filter, dBm |  | -14 | -51 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -78 | -152 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -74 | -148 |
| noise figure dB | 12 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162 |  |  |
| Rx band n3 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.47 | -95.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n39 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -17.77 |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -11.24 |  |
| n39 Tx filter rejection at 1805~1880MHz, dB | 50 |  |  |
| n39 PA noise power at Rx n3 ant port at 1825~1830MHz, dBm |  | -61.24 | -81.24 |
| Total noise level at ANT port |  | -61.01 | -81.08 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -62.01 | -82.08 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -79.62 |  |  |
| MSD | 17.38 |  |  |

Based on the above analysis, MSD for 23dBm to 40dBm transmission power would be ranging from 4.75dB to 17.38dB which will bring the significant performance degradation of band n3 DL reception.

Conclusions

In this contribution, we shared some initial views on the RF requirements for ATG UE supporting inter-band DL CA in Rel-19 and proposals are made as following:

Observation 1:MSD for 23dBm to 40dBm transmission power would be ranging from 4.75dB to 17.38dB which will bring the significant performance degradation of band n3 DL reception.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.

- R4-2410687,WF on Rel-19 ATG UE RF requirements, Approved.

- TR 38.718-02-01

- R4-2417121, WF on UE RF for ATG enhancement, CMCC, approved.