3GPP TSG-RAN WG4 Meeting#114                               R4-2501875

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.13.3

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on OTA test enhancement

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we have some initial discussions on the necessity of improvement of co-location issues. In this contribution, we would like to share further views on the OTA co-location requirements.

Sub-topic 1-2: BS-to-BS isolation analysis Way forward: - Provide inputs on the proposed BS-to-BS coupling derivation methodology, considering valid co-location deployment scenarios: - Impact of beam steering for coupling analyses, - Investigation on the set of impacted specifications (core, test, etc.), - Investigation on impact for specific requirements (spurious emission, receiver blocking, [TDD OFF power] and TX IMD) - Investigation on the set of impacted OTA co-location test requirements, - Considered deployments to be modelled (horizontal antennas stacking, vertical stacking, etc.), - Other inputs not precluded. - Aim at band-agnostic solution for BS-to-BS isolation analysis, prioritizing FR1 bands (higher bands beyond FR1 as second priority). Sub-topic 1-3: TX IMD co-location Way forward: - Decision on potential limitation of TX IMD co-location requirement applicability is postponed till conclusion on BS-to-BS isolation analysis is reached (Sub-topic 1-2). - Applicable frequency limit for the potential removal of TX IMD co-location requirement to be derived based on the BS-to-BS isolation analysis outcomes (Sub-topic 1-2).

Discussion

As we discussed in the last RAN4 meeting, the following issues could be further discussed to identify the necessity for co-location reference antenna improvement.

  - .Coupling loss between two AAS BS in FR1 high bands

First of all, NR BS coexisting with non-AAS BS in FR1 high bands are not very common. Instead, two AAS BS coexisting with each other are more ubiquitous which are one candidate issue captured in the agreed WF [1]. Based on the above consideration from the practical deployment perspective, we conducted some coupling loss measurement shown as following:

- (left figure) Two AAS BS are separated in the vertical domain (10cm separation as worst assumption).

- (Right figure) Two AAS BS are separated in the horizontal domain (10cm separation as worst assumption).

Figure 2.1-1. coupling loss measurement for FR1 high bands with separation distance as 10cm from edge to edge

NOTE: coupling loss measurement with CW source signal is done in the OTA chamber without considering the different beam steering steering. Beam of two AAS BSs are steering towards the boresight direction.

Figure 2.1-2a. antenna ports for coupling loss measurement with vertical separation [expected worst coupling loss]

Figure 2.1-2b. antenna ports for coupling loss measurement with horizontal separation [expected worst coupling loss]

According to the above testing conditions/assumptions, some initial measurement results for FR1 high bands are provided in the following Table 2.1-1. As shown in the following table, if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss between sub-array would be larger than 50dB under the vertical isolation (closet antenna elements as shown in Figure 2.1-2a). However if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss between sub-array is still larger than 30dBc (closet antenna elements as shown in Figure 2.1-2b). It also depends on the BS’s implementation, for different BS RF architecture, coupling loss among different antenna ports are varying from 31dB to 46dBc.

Table 2.1-1. summary of measured coupling loss different frequency ranges for FR1 WA BS

| Antenna type | 2.6G-BS1 | 2.6G-BS2 | 3.5G-BS1 | 3.5G-BS2 | 4.9G-BS1 |
| --- | --- | --- | --- | --- | --- |
| Isolation in vertical domain(dB) | <-54 | <-57 | <-52 | <-53 | <-56 |
| Isolation in horizontal domain | <-31 | <-40 | <-34 | <-35 | <-46 |

Observation 1:if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss between sub-array would be larger than 50dB with10cm separation distance.

Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc.

In the last RAN4 meeting, there were some discussions on the impacts of beam steering on coupling loss, from our understanding, it’s true that the beam steering from antenna array of aggressive BS towards sub-array of victim BS and from antenna array of aggressive BS towards antenna array of victim BS would have the impacts. More measurement results or simulation results by EM software platform could be provided later on to figure out the exact coupling loss in different deployment scenario.

  - . The set of impacted specifications

In the previous discussions, it is agreed to have some further discussion on the impacted specification. From our understanding, at least the following specification would be impacted since all these specification include the TDD OFF power, co-location spurious emission requirement, Tx intermodulation requirement and also Rx co-location blocking requirement.

- TS 38.104

- TS 38.141-1/2

- TS 38.174

- TS 38.176-1/2

- TS 38.106

- TS 38.115-1/2

- Proposal 1: propose to consider the following specification for BS-BS isolation analysis.

- TS 38.104

- TS 38.141-1/2

- TS 38.174

- TS 38.176-1/2

- TS 38.106

- TS 38.115-1/2

  - . The set of impacted specific requirement

In the previous discussions, there were some discussions on the impacted specific requirement, we would like to review the background for the following requirement firstly.

### 2.3.1.Transmitter OFF power requirement

For NR BS transmit OFF power is defined in terms of PSD in 1MHz, calculated as following:

[Transmit OFF power] < [Thermal noise] + [Noise Figure] – 6dB + [Minimum coupling loss (MCL)] + 10log[1MHz]

< -174 + 5 – 6 + MCL+60

< -115dBm/MHz + MCL

Considering the worst case is that eNBs are co-located, the minimum coupling loss(MCL) is assumed 30dB. The transmit OFF power is -85dBm/MHz.

### 2.3.2.Co-location spurious emission requirement/Protection of the BS receiver of own or different BS

- The Spurious Emission Level of WA BS should be:

- Spurious Emission LevelWA ＜ kT0 + NFWA  + 10*log10(100kHz) －7dB + MCL

- = -174dBm/Hz + 5dB + 50dB－7dB + MCL

- Where :

- MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP.

- The BS of different classes will have different Noise figures. Generally, in most 3GPP spec, we assume that   the WA NF = 5dB, MR NF = 10dB, and LA NF = 13dB.

### 2.3.3.Tx IMD requirement

- Interfering signal level is equal to Rated total output power (Prated,t,AC) in the operating band – 30 dB where MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP.

Table 6.7.2.1-1: Interfering and wanted signals for the co-location transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported. |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5. |
| Interfering signal level | Rated total output power (Prated,t,AC) in the operating band – 30 dB |
| Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. |  |

### 2.3.4.Co-located OOBB requirement

- Interfering signal level for WA, MR and LA is based on the following calculation assumption:

- 46dBm-MCL;

- 38dBm-MCL;

- 24dBm-MCL;

- Where MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP.

Table 7.5.3-1: Blocking performance requirement for NR BS when co-located with BS in other frequency bands.

| Frequency range of interfering signal | Wanted signal mean power for WA BS (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | +8 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the BS channel bandwidth as specified in Table 7.2.2-1, 7.2.2-2, and 7.2.2-3. NOTE 2: x = -7 dBm for NR BS co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for NR BS co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for NR BS co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOB immediately outside any of the supported uplink operating band(s). |  |  |  |  |  |

The concept of co-location reference antenna is defined due to the measurement capability issue in the OTA chamber. E..g. -85dBm/MHz OFF power,  -96dBm/100KHz co-location spurious emission requirement is too low to be measured in the OTA chamber, using the alternative co-location reference antenna could help replace OTA measurement as the conducted interface measurement with much better measurement capability.

For receiver OOBB requirement, to generate the CW signal with 46dBm/38dBm is also very challenging somehow, to use the co-location reference antenna could also alleviate the measurement risk.

- Proposal 2: propose to consider the following specific requirement for BS-BS isolation analysis.

- Transmitter OFF power requirement

- Co-location spurious emission requirement/Protection of the BS receiver of own or different BS

- Tx IMD requirement

- Co-located OOBB requirement

- Proposal 3: propose to further consider the co-location reference antenna or removal of certain requirement after the worst coupling loss is confirmed.

  - .Difference between AAS BS vs AAS BS and AAS vs co-location reference antenna

In the previous discussions, one of the critical issue is to identify the difference between AAS vs AAS BS and AAS vs co-location reference antenna. In the following section,we would like to provide the further analysis from both transmitter and receiver requirements perspective.

| Clause number | Requirement | Co-location reference antenna operation | Type |
| --- | --- | --- | --- |
| 6.5 | OTA transmit ON/OFF power for FR1 | Measure emission | Mandatory |
| 6.7.5.3 6.7.5.5 | OTA spurious emission: Protection of the BS receiver of own or different BS | Measure emission | Optional based on declaration |
|  | OTA spurious emission: Co-location with other base stations |  |  |
| 6.8 | OTA transmitter intermodulation | Inject the interferer signal | Mandatory |
| 7.6.3 | OTA out-of-band blocking: Co-location with other base stations | Inject the interferer signal | Optional based on declaration |

Figure 2.3-1. the legacy co-location reference antenna for OFF power and spurious emission requirement

For the legacy co-location reference antenna, the received power at co-location reference antenna output should be Ptx+G_Tx-Coupling_loss+G_Rx_CLTA.

Figure 2.3-2. the proposed new co-location reference antenna for OFF power and spurious emission requirement

For the proposed new co-location reference antenna for FR1 high bands, received power at co-location reference antenna output should be Ptx+G_Tx-Coupling_loss+G_Rx_sub-array.

The main difference for OFF power and spurious emission measurement would be G_Rx_CLTA and G_Rx_sub-array pattern. These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be representative for AAS BS co-locating with AAS BS at sub-array level.

Observation 3: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Figure 2.3-3. the legacy co-location reference antenna for Tx intermodulation and co-location OOBB requirement

For the legacy co-location reference antenna, the received power at sub-array of AAS BS should be Ptx+G_Tx_CLTA -Coupling_loss+G_Rx_sub_array

Figure 2.3-4 the proposed new co-location reference antenna for Tx intermodulation and co-location OOBB requirement

For the proposed new co-location reference antenna for FR1 high bands, received power at sub-array of AAS BS should be Ptx+G_Tx_AAS -Coupling_loss+G_Rx_sub_array.

The main difference for Tx intermodulation and co-location reference antenna would be G_Tx_CLTA and G_Tx_AAS antenna pattern. These two passive antenna and AAS BS pattern would be quite different. G_Tx_CLTA should provide much more stringent requirement for it since it provides the much wider beam in azimuth plane towards the victim AAS BS with narrow beam. In other words, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

Observation 4: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

  - .Initial simulation results for coupling loss in FR1 high bands

During the last RAN4 meeting, there were some discussions on the technical analysis for coupling loss in FR1 high bands. According to the simulation assumption agreed for Rel-18 SBFD coexistence study, we provided some initial evaluation results for coupling loss at 4GHz. Based on the simulation results got far, the coupling loss is definitely larger than 50dBc instead of 30dBc in FR1 WA, MR and LA scenario.

FR1 WA BS:

FR1 MR BS:

FR1 LA BS:

Observation 5: Based on the initial simulation results for FR1 high bands, coupling loss is expected to be larger than 50dBc regardless from array to array or array to antenna port.

Conclusions

In this contribution, we shared some further views on co-location reference antenna concept in RAN4 and proposals are made as following:

Observation 1:if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB with10cm separation distance.

Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc.

- Proposal 1: propose to consider the following specification for BS-BS isolation analysis.

- TS 38.104

- TS 38.141-1/2

- TS 38.174

- TS 38.176-1/2

- TS 38.106

- TS 38.115-1/2

- Proposal 2: propose to consider the following specific requirement for BS-BS isolation analysis.

- Transmitter OFF power requirement

- Co-location spurious emission requirement/Protection of the BS receiver of own or different BS

- Tx IMD requirement

- Co-located OOBB requirement

- Proposal 3: propose to further consider the co-location reference antenna or removal of certain requirement after the worst coupling loss is confirmed.

Observation 3: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Observation 4: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

Observation 5: Based on the initial simulation results for FR1 high bands, coupling loss is expected to be larger than 50dBc regardless from array to array or array to antenna port.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2319398, Background and technical challenges with BS RF co-location requirements in relation to development of NCR RF co-location requirements, Ericsson, Noted

- R4-1703389, “On BS to BS co-location/co-siting background”, Ericsson

- R4-1706766,Co-location isolation between different frequency bands, Ericsson, Noted

- R4-2406088,Way Forward for [110bis][311] NR_BS_RF, ZTE, approved.

- R4-2404259,OTA test enhancements - CLTA, Nokia

- R4-2409955, Way forward for [111][311] NR_BS_RF, Approved.

- R4-2416574,Way Forward for [112bis][303] NR_BS_RF_Part2_CLTA, Huawei, approved.

- R4-2419797,Way Forward for [113][303] NR_BS_RF_Part2_CLTA, Approved

- RP-243270,Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, approved.