3GPP TSG-RAN WG4 Meeting#114                                R4-2501879

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.13.4.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on the applicability improvement of TRP measurement

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. For the applicability improvement of TRP in Annex, it was agreed to have some clarifications for it to avoid the confusion outside of 3GPP especially.

- TRP simplification: Simplify BS TRP estimation methods (e.g. Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods. Issue 2-1: Simplification of BS TRP test methods - Recommended for further discussion: - Focus on the improvement of TRP estimation method in Annex I in TS 38.141-2 which depends on WID revision.

Discussion

Based on the latest updated objective for TRP simplification, RAN4 need to work on how to improve the applicability of different TRP measurement method for the corresponding requirement, meanwhile to make the measurement with some procedures instead of listing of TRP measurement methods for different RF requirements.

Here, we would like to have some initial proposals for further discussions, e.g.

- BS output power

- Two cuts or three cuts method or beam peak based method was proposed at the late stage for TRP measurement method in Rel-15 with its main purpose to reduce the testing time due to the relative long and expensive OTA testing cost. From RAN4 perspective, for BS output power, we could start with two cuts+pattern multiplication or [beam peak based method] to check whether NR BS could fulfill it TRP accuracy requirement. If TRP measurement accuracy requirement failed to be met in the step 1, then reference measurement method as indicated in I.2/3/4 could be used as accurate method method

- Step 1: Two cuts + Pattern multiplication (accurate) [or beam peak based method]

- Step 2: reference measurement if it fails to meet the requirement in step 2;

- In addition, RAN4 also need to further discuss whether measurement step 1 could be representative for the measurement in step 2 or if NR BS in step 1 fulfilling the accuracy requirement can also guarantee that NR BS in step 2 also fulfilling the accuracy requirement.

Proposal 1: for BS output power TRP measurement, propose to consider the following testing procedure and its applicable TRP testing method as starting point;

- Step 1: Two cuts + Pattern multiplication (accurate) [or beam peak based method]

- Step 2: reference measurement if it fails to meet the requirement in step 2;

Proposal 2: RAN4 need to further discuss whether measurement step 1 could be representative for the measurement in step 2 or if NR BS in step 1 fulfilling the accuracy requirement can also guarantee that NR BS in step 2 also fulfilling the accuracy requirement.

- BS ACLR requirement/unwanted emission requirement

- For BS ACLR requirement, pre-scanning approach could be used as 1st step to identify whether NR BS fails the absolute ACLR limits. If NR BS fails the absolute limit in step 1, then more accurate TRP measurement method in step 2 could be considered. For some TRP testing method, e.g. beam peak based method might be not applicable for ACLR and unwanted emission requirement since the antenna pattern for wanted signal and unwanted signal might be different. If NR BS fail to fulfill the limits in step 1/2, then reference measurement as indicated in I.2/3/4 could be used as accurate method method.

- Step 1: Pre-scanning for absolute ACLR limit

- Step 2: Two cuts + Pattern multiplication (accurate) or

- Step 2a : three cuts (if two cuts fails the limit) or [peak method or equal sector with peak average]  if measurement results of Pre-scanning fails the limits [CATR]

- Step 3: reference measurement  if it fails to meet the requirement in step 2;

Proposal 3: for BS ACLR and unwanted emission requirement, propose to consider the following testing procedure and its applicable TRP testing method as starting point;

- Step 1: Pre-scanning for absolute ACLR limit

- Step 2: Two cuts + Pattern multiplication (accurate) or

- Step 2a : three cuts (if two cuts fails the limit) or [peak method or equal sector with peak average]  if measurement results of Pre-scanning fails the limits [CATR]

- Step 3: reference measurement  if it fails to meet the requirement in step 2;

- BS spurious emission requirement [how to consider RC testing method]

- For BS spurious emission requirement, we propose to consider the following TRP measurement procedures and the applicable TRP test method as starting point..

- Step 1: Pre-scanning method

- [Step 2: peak method or equal sector with peak average if measurement results of Pre-scanning fails the limits]

- Step 3: reference measurement if it fails to meet the requirement in step 2;

Proposal 4: for BS spurious emission requirement, propose to consider the following testing procedure and its applicable TRP testing method as starting point;

- Step 1: Pre-scanning method

- [Step 2: peak method or equal sector with peak average if measurement results of Pre-scanning fails the limits]

- Step 3: reference measurement if it fails to meet the requirement in step 2;

Conclusions

In this contribution, we shared some initial views on the applicability improvement of TRP in Annex in RAN4 and proposals are made as following:

Proposal 1: for BS output power TRP measurement, propose to consider the following testing procedure and its applicable TRP testing method;

- Step 1: Two cuts + Pattern multiplication (accurate) [or beam peak based method]

- Step 2: reference measurement if it fails to meet the requirement in step 2;

Proposal 2: RAN4 need to further discuss whether measurement step 1 could be representative for the measurement in step 2 or if NR BS in step 1 fulfilling the accuracy requirement can also guarantee that NR BS in step 2 also fulfilling the accuracy requirement.

Proposal 3: for BS ACLR and unwanted emission requirement, propose to consider the following testing procedure and its applicable TRP testing method;

- Step 1: Pre-scanning for absolute ACLR limit

- Step 2: Two cuts + Pattern multiplication (accurate) or

- Step 2a : three cuts (if two cuts fails the limit) or [peak method or equal sector with peak average]  if measurement results of Pre-scanning fails the limits [CATR]

- Step 3: reference measurement  if it fails to meet the requirement in step 2;

Proposal 4: for BS spurious emission requirement, propose to consider the following testing procedure and its applicable TRP testing method as starting point;

- Step 1: Pre-scanning method

- [Step 2: peak method or equal sector with peak average if measurement results of Pre-scanning fails the limits]

- Step 3: reference measurement if it fails to meet the requirement in step 2;

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2419797	Way Forward for [113][303] NR_BS_RF_Part2_CLTA

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP