3GPP TSG-RAN WG4 Meeting #114                                                              R4-2501965

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:	Discussion on testing framework and procedure for AI-BM

Agenda item:		7.20.2

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two confirmed use cases (i.e., beam management and positioning accuracy enhancements) [2].

Particularly for the use case of beam management, it is required to provide specification support for DL TX beam prediction for both UE-sided and NW-sided model, and correspondingly necessary RAN4 core requirements for this use case and LCM procedure including performance monitoring [2]. Since AI-BM involves more complicated test procedure compared with Rel-15 FR2 beam management related test, the testability issue are highlighted in previous RAN4 discussion.

Based on the outcome of the study objectives captured in TR 38.843 [1] and other RAN1 progress, we would like to discuss the remaining testability and interoperability issues for beam management in this contribution.

# 2. Discussion

## 2.1 Test procedure for AI-BM (UE-side model)

We have seen the importance of clarifying test procedure for AI-BM with UE-sided model. Among two AI-BM use case (Case1 & Case2), we would like to take Case 1 as example to elaborate the test procedure for AI-BM conformance testing.

Proposal 1: The conformance testing procedure for AI-BM UE-side model (using AI-BM Case1 as an example), is provided as:

- Preparation Phase:

- Step-1: TE configures resource sets for both Set A and Set B beams in CSI-ReportConfig, and associated ID as side condition

- Model Inference Phase:

- Step-2: TE configures the test environment to transmit Set B beams

- Step-3: DUT (a.k.a, UE) predicts Set A beams based on the measurement from Set B beams, and reports the prediction results back to TE

- Ideal Measurement Phase:

- Step-4: TE adjusts the test environment to transmit Set A beams, ensuring a certain level of SNR for the ideal measurement of RSRP

- Step-5: DUT measures the Set A beams, and report measurement results to TE

- TE Evaluation Phase:

- Step-6: TE evaluates the success of the prediction by comparing the predicted results from Step 3 with the actual measurement results from Step 5.

It should be noted that the procedure to build up the dataset used for UE sided model training is out of the test procedure for AI-BM, and the corresponding data collection procedure should be determined by UE vendors.

Proposal 2: The procedure to build up the dataset used for UE sided model training is out of the test procedure for AI-BM.

## 2.2 Consistence between training and inference for AI-BM

The consistency between training and inference is a critical aspect of ensuring reliable performance for AI/ML-based BM.

For static condition used in AI-BM (which should be considered as the baseline case), the consistency between Set A and Set B must be enforced during training and inference. For instance, if the UE is trained on a specific beam configuration (e.g., Set B), inference under a modified configuration (e.g., Set B') may lead to performance degradation due to misalignment. On the other hand, the non-static conditions achieved by dynamically changed scenarios (e.g., varying beam directions, UE mobility) in the test could challenge the UE’s ability to adapt via diverse AI/ML models. However, if the inconsistencies between training and inference environments exist in such cases, we see the risk that the AI/ML model fails to generalize to new conditions.

The additional conditions have been discussed in RAN1 but not yet been clarified in detailed definition for AI-BM, like network-side parameters (e.g., beam shapes, antenna spacing, and carrier frequencies could be often excluded from UE capability reporting). These "additional conditions" could influence model training but may not align with inference scenarios, leading to performance gaps. For example, a model trained on narrow beams may fail if deployed with wider beams.

Proposal 3: The consistence between training and inference for AI-BM shall be guaranteed by using static condition as baseline, in which additional condition shall also be remains to be static.

In some of the previous discussion, the problem of overfitting to test environments is highlighted, i.e., over-reliance on synthetic test data (e.g., limited AoAs in OTA chambers) can lead to model overfitting, where models excel in controlled tests but fail in real-world deployments. Accordingly, some company proposed testing across diverse scenarios (e.g., varying UE speeds, SINR, propagation models) to mitigate this. However, we think this is similar to define multiple test cases with different channel conditions, while not a new concept to AI only.

## 2.3 Test setup for UE-sided AI-BM testing

As discussed over previous RAN 4 meetings, the group agrees to further discuss and down-select the following options considering TE feasibility:

- Option 2: TDL

- Option 5: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Based on RAN4#112bis, the following agreement is provided for test setup requirements:

Agreement from RAN4#112bis - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

Here we would like to firstly identify the key difference by using TDL and simplified CDL models from the perspective of testing target (based on the analysis in section 2.1):

- DUT RX beam performance and relevant procedure can only be verified with CDL models, in which the spatial differentiated AoAs can be emulated:

  - For AI/ML model with TX-RX beam pair measurement as model input, CDL model with the spatial differentiated AoAs is required.

  - For AI/ML models by TX beam measurement from the best RX beam as model input, if an ideal spherical coverage (uniform in all directions) is assumed, to test this AI/ML models, the spatial AoA characteristics could be not that important.

Figure 1: AI/ML models by using the measurements from the best Rx beam (left) and Tx-Rx beam pair (right)

From technical aspects, for DFF chamber, RAN4 has studied the method to use one probe to simulate multi-path fading condition, as provided in Section 8.2 in TR 38.810, in which two options are given. However, it should be noted that in the corresponding single probe channel modelling, the UE RX beamforming can be included in the channel model, while since there is only one AoA, it is impossible to test actual RX beam management procedure.

#### 8.2.1.1 Channel model Option 1 Channel model option 1 is based on the TDL methodology in the TR 38.901 [10]: - The multi-path propagation conditions model consists of several parts: - A power delay profile in the form of a "tapped delay-line" (TDL), characterized by a number of taps with certain power at fixed positions on a sampling grid. - The channel model parameters include the Delay spread scaling factor and the maximum Doppler frequency. The test system shall allow flexible control of the respective parameters. - Each tap is modeled based on the Jakes fading model. - Generation of TDL channel models and power delay profiles from CDL channel models by including spatial filters to capture Tx and Rx antenna patterns is not precluded and based on the procedure described in the TR 38.901 [10].

Observation 1: The necessity of introducing spatial differentiated AoAs in AI-BM testing could depends on different AI/ML model inputs:

- For AI/ML model with TX-RX beam pair measurement as model input, the spatial differentiated AoAs is required.

- For AI/ML models by TX beam measurement from the best RX beam as model input, to test this AI/ML models, the spatial AoA characteristics could be not that important.

It should be noted that the spatial differentiated AoAs could not only be CDL channel but also TDL with multi-AoA/probes test setup.

Proposal 4: Taking into account the conclusion from beam or beam pair as model input, RAN4 can determine the test setup with spatial differentiated AoAs required or not.

Another aspect is how many TRPs to be simulated for AI-BM. Based on our understanding, the relevant AI-BM prediction are primarily used for single TRP case. At least for Rel-19 AI-BM, RAN4 discussion shall focus only on single TRP case.

Proposal 5: At least for Rel-19 AI-BM, RAN4 discussion shall focus on single TRP case, which is considered as baseline assumption for testability study.

Based on the above test procedure, it has been recognized that relying solely on Set B of beams for the test setup might not suffice. Based on previous RAN4 agreement, the ground truth shall be “the ideal measurement of RSRP on the predicted Tx beam”, which in the conformance test is “the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam”. Our interpretation is that the ideal RSRP measurement on the predicted transmission beam should be executed by producing Set A of beams.

Agreement from RAN4#111 Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded Agreement from RAN4#112bis Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Proposal 6: AI-BM OTA test setup shall generate the not only Set B of beams, but also Set A of beams for the ideal measurement of RSRP on the predicted Tx beam.

For the selection of OTA chamber, we see the difference for the required probes for MPAC OTA chamber. For instance, the analysis provided in [R4-2411254] requires 8 probes (2 additional one compared with existing 3D MPAC chamber) to simulate 4 strongest TX beams, with acceptable PSP (PAS similarity percentage) for InO and UMi scenarios. On the other hand, in [R4-2411587], upscaling the 3D MPAC chamber to emulate CDL-C UMi requires a test system with >100 probes. The difference may come from how many TX beams and how many clusters to be emulated with accepted accuracy.

From our understanding, the feasibility of enhanced 3D MPAC chamber shall consider the following factors, which are provided in the following proposal.

Proposal 7: The feasibility of enhanced 3D MPAC chamber shall consider the following factors:

- How many beams out of Set A should be emulated?

  - At least X strongest beams out of the whole beambook (e.g., 128 beams containing Set B beams) are relevant for AI-BM inference testing.

- How many beams out of Set B should be emulated?

  - At least Y strongest beams out of the whole beambook (e.g., 16 beams) are relevant for AI-BM inference testing.

- All Tx beams from a single TRP;

- Acceptable PAS similarity percentage based on a certain configuration of probes

To determine the value of X and Y in the above proposal, RAN4 need to perform the system-level simulation by considering the performance degradation from and reasonable dynamic range from the beam signal strength level.

Proposal 8: RAN4 perform system-level simulation to determine the value of X and Y (in Proposal 7):

- To determine the value of X, by allowing a reasonable AI/ML prediction performance degradation (e.g., nearly 0dB absolute error degradation for RSRP accuracy) by only measuring  X strongest beams from the whole Set A beambook in the ideal Measurement Phase.

- To determine the value of Y, by allowing a reasonable AI/ML prediction performance degradation (e.g., 1dB absolute error degradation for RSRP accuracy) by only measuring Y strongest beams from the whole Set B beambook.

- Reasonable dynamic range of the beam signal strength level shall also be assumed.

# 3. Conclusion

In this contribution, we provided our viewpoints on testability and interoperability issues for beam management in this contribution, accordingly the following observations and proposals are obtained:

Test procedure for AI-BM (UE-side model)

Proposal 1: The conformance testing procedure for AI-BM UE-side model (using AI-BM Case1 as an example), is provided as:

- Preparation Phase:

- Step-1: TE configures resource sets for both Set A and Set B beams in CSI-ReportConfig, and associated ID as side condition

- Model Inference Phase:

- Step-2: TE configures the test environment to transmit Set B beams

- Step-3: DUT (a.k.a, UE) predicts Set A beams based on the measurement from Set B beams, and reports the prediction results back to TE

- Ideal Measurement Phase:

- Step-4: TE adjusts the test environment to transmit Set A beams, ensuring a certain level of SNR for the ideal measurement of RSRP

- Step-5: DUT measures the Set A beams, and report measurement results to TE

- TE Evaluation Phase:

- Step-6: TE evaluates the success of the prediction by comparing the predicted results from Step 3 with the actual measurement results from Step 5.

Proposal 2: The procedure to build up the dataset used for UE sided model training is out of the test procedure for AI-BM.

Consistence between training and inference for AI-BM

Proposal 3: The consistence between training and inference for AI-BM shall be guaranteed by using static condition as baseline, in which additional condition shall also be remains to be static.

Test setup for UE-sided AI-BM testing

Observation 1: The necessity of introducing spatial differentiated AoAs in AI-BM testing could depends on different AI/ML model inputs:

- For AI/ML model with TX-RX beam pair measurement as model input, the spatial differentiated AoAs is required.

- For AI/ML models by TX beam measurement from the best RX beam as model input, to test this AI/ML models, the spatial AoA characteristics could be not that important.

Proposal 4: Taking into account the conclusion from beam or beam pair as model input, RAN4 can determine the test setup with spatial differentiated AoAs required or not.

Proposal 5: At least for Rel-19 AI-BM, RAN4 discussion shall focus on single TRP case, which is considered as baseline assumption for testability study.

Proposal 6: AI-BM OTA test setup shall generate the not only Set B of beams, but also Set A of beams for the ideal measurement of RSRP on the predicted Tx beam.

Proposal 7: The feasibility of enhanced 3D MPAC chamber shall consider the following factors:

- How many beams out of Set A should be emulated?

  - At least X strongest beams out of the whole beambook (e.g., 128 beams containing Set B beams) are relevant for AI-BM inference testing.

- How many beams out of Set B should be emulated?

  - At least Y strongest beams out of the whole beambook (e.g., 16 beams) are relevant for AI-BM inference testing.

- All Tx beams from a single TRP;

- Acceptable PAS similarity percentage based on a certain configuration of probes

Proposal 8: RAN4 perform system-level simulation to determine the value of X and Y (in Proposal 7):

- To determine the value of X, by allowing a reasonable AI/ML prediction performance degradation (e.g., nearly 0dB absolute error degradation for RSRP accuracy) by only measuring  X strongest beams from the whole Set A beambook in the ideal Measurement Phase.

- To determine the value of Y, by allowing a reasonable AI/ML prediction performance degradation (e.g., 1dB absolute error degradation for RSRP accuracy) by only measuring Y strongest beams from the whole Set B beambook.

- Reasonable dynamic range of the beam signal strength level shall also be assumed.

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402389, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] 3GPP TR 38.810, v16.7.0, “NR; Study on test methods”.

[6] 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[7] R4-2405652, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110bis, April 2024.

[8] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[9] R4-2409761, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#111, May 2024.

[10] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[11] R4-2411635, “Further discussion on testability and interoperability issues for beam management”, Samsung, RAN#112, Aug. 2024.

[12] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[13] R4-2416493, “Further discussion on testability and interoperability issues for beam management”, RAN#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[15] R4-2418094, “Further discussion on testability and interoperability issues for beam management”, RAN#113, Nov. 2024.

[16] R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, RAN#113, Nov. 2024.

# 5. Appendix

From RAN4#111 to RAN4#113, for reported measurements and ground truth, test setup (including channel model) and datasets for training/testing, the following agreements are achieved:

Agreement from RAN4#111 Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded Issue 2-5: Test setup needs Agreement: - maximum number of set B Tx beams that test system should be able to emulate: [8-16] - maximum number of set A Tx beams that test system should be able to emulate: [64-128] - FFS on AoAs - UE rotation during the test: FFS - UE rotation/repositioning between different tests: Yes Issue 2-6: Datasets for training/testing Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions Agreement from RAN4#112 Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy. Issue 2-7: UE reporting for network side models Agreement: postpone discussion until other WGs define any reporting mechanisms Agreement from RAN4#112bis Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point. Agreement from RAN4#113 Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions Measurement error impact evaluation for beam prediction Agreement: Updated simulation assumptions for measurement error impact evaluation for beam prediction is approved in R4-2420480 Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS